3GPP TSG-CT Meeting #98-e	CP-223260

# E-Meeting, 12th – 14th December 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.124 | CR | 0659 | rev | 3 | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | x | ME | x | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test case on PROVIDE LOCAL INFORMATION to get Slice(s) information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | THALES DIS, Keysight Technologies UK Ltd., Qualcomm Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | THALES DIS, Keysight Technologies UK Ltd., Qualcomm Technologies |  |  |  |  |  |  |  |  |  |
| Work item code: | UEConTest_R16 |  |  |  |  |  | Date: |  |  | 2022-12-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During CT6#98-e meeting was approved the CR0730 on on 3GPP TS 31.111 (C6-200132) introducing the new ‘slice(s) information’ option from PROVIDE LOCAL INFORMATION command. Test Case shall be added to cover this feature. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR adds : - the new test case in B.1 Applicability of tests The CR updates : - Clause 27.22.4.15.3 Test purpose adding new PLI option, - Clause 27.22.4.15.4.1 Initial conditions adding NG-RAN slices configuration, - Clause 27.22.4.15.4.2 adding corresponding Expected Sequence 1.xx in test procedure, |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No test case covering the slice(s) information option in the PROVIDE LOCAL INFORMATION |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 3.4, 27.22.4.15.3, 27.22.4.15.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Was CP-223153 was C6-220702 was C6-220640 |  |  |  |  |  |  |  |  |

***** first change *****

## 3.3	Table of optional features

Support of USIM Application Toolkit is optional for Mobile Equipment. However, if an ME states conformance with a specific 3GPP release, it is mandatory for the ME to support all functions of that release, as stated in table B.1, with the exception of the functions:

-	"Alpha identifier in REFRESH command supported by terminal";

-	"Event Language Selection";

-	"Proactive UICC: PROVIDE LOCAL INFORMATION (language)"; and

-	"Proactive UICC: LANGUAGE NOTIFICATION".

In accordance to 3GPP TS 36.300 [40], clause 4.10, additional exceptions apply for NB-IoT only MEs.
As a number of E-UTRA protocol functions supported by Rel-8 MEs are not required for NB-IoT, the related USIM Application Toolkit functions may not be supported by NB-IoT only MEs.

Note:	NB-IoT only MEs are MEs that only support NB-IoT and no other radio access technology.

The support of letter classes, which specify mainly ME hardware dependent features, is optional for the ME and may supplement the USIM Application Toolkit functionality described in the present document. If an ME states conformance to a letter class, it is mandatory to support all functions within the respective letter class.

The supplier of the implementation shall state the support of possible options in table A.1.

Table A.1: Options

| Item | Option | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- |
| 1 | Capability Configuration parameter | M |  | O_Cap_Conf |
| 2 | Sustained text | C002 |  | O_sust_text |
| 3 | UCS2 coding scheme for Entry | O |  | O_Ucs2_Entry |
| 4 | Extended Text String | C002 |  | O_Ext_Str |
| 5 | Help information | O |  | O_Help |
| … |  |  |  |  |
| 185 | Terminal supports Date-Time and Time zone | O |  | O_DateTime_Time_zone |
| 186 | Terminal supports NB-IoT only | O |  | O_NB-IoT_only |
| 187 | Terminal supports NG-RAN | O |  | pc_NG_RAN |
| 188 | Class E: Terminal does support NG-RAN | O |  | pc_BIP_NG_RAN |
| 189 | Class V: support of PROVIDE LOCATION INFORMATION, H(e)NB IP address | O |  | O_PLI_HeNB_IP_Address |
| 190 | Class W: support of PROVIDE LOCATION INFORMATION, H(e)NB surrounding macrocells | O |  | O_PLI_HeNB_Sur_Macrocells |
| 191 | Terminal supports SUPI as Network Access Identifier | O |  | O_SUPI_NAI |
| C001 If terminal is implemented according to Rel-6 or later then M, else O |  |  |  |  |
| C002 If feature is implemented according to Rel-8 or later then O, else M. It is possible to implement the related features according to Rel-8 or later even if the generic toolkit implementation is according to a release earlier then Rel-8. |  |  |  |  |
| C003 If terminal is implemented according to Rel-8 or later AND ((A.1/139 OR A.1/140) AND (NOT A.1/64) AND (NOT A.1/134)) THEN M ELSE N/A |  |  |  |  |
| C004 If feature is implemented according to Rel-11 or later then M, else N/A |  |  |  |  |
| C005 If feature is implemented according to Rel-12 or later then O, else N/A |  |  |  |  |
| C006 If feature is implemented according to Rel-13 or later then M, else O |  |  |  |  |
| NOTE: Items 161, 162, 163 and 164 were made optional as a consequence of the approval of CR 0429 against TS 31.111 and CR 0419 against TS 31.124 |  |  |  |  |

***** next change *****

## 3.4	Applicability table

NOTE:	It is possible that the applicability of some tests indicated in table B.1 does not match with the value in the Release column, due to late definition of the test sequences. Tests should be performed without considering the Release column, but only based on the conditions indicated for each release.

Table B.1: Applicability of tests

| 18 | PROVIDE LOCAL INFORMATION 27.22.4.15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | location information | R99 | 1.1 | M | M | M | M | M | M | M | M | M | M | M | M | M | M | E.1/31 | Yes |  | AER003 |
|  | IMEI | R99 | 1.2 | M | M | M | M | M | M | M | M | M | M | M | M | M | M | E.1/31 | No |  |  |
|  | network measurement results and BCCH channel list | R99 | 1.3 | C167 | C167 | C167 | C167 | C167 | C167 | C167 | C167 | C167 | C167 | C167 | C167 | C167 | C167 | E.1/32 AND E.1/67 | System Simulator only |  |  |
|  | Date, time and time zone | R99 | 1.4 | M | M | M | M | M | M | M | M | M | M | M | M | M | M | E.1/59 | No |  |  |
|  | language setting | R99 | 1.5 | C217 | C217 | C217 | C217 | C217 | C217 | C217 | C217 | C217 | C217 | C217 | C217 | C217 | C217 | E.1/68 | No |  |  |
|  | Timing advance | R99 | 1.6 | C167 | C167 | C167 | C167 | C167 | C167 | C167 | C167 | C167 | C167 | C167 | C167 | C167 | C167 | E.1/69 | System Simulator only |  |  |
|  | Access Technology | Rel-4 | 1.7 |  |  |  | M | M | C184 | C184 | C184 | C184 | C184 | C184 | C184 | C184 | C184 | E.1/72 | UMTS System Simulator only |  | AER004 |
|  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IMEISV | Rel-6 | 1.9 |  |  |  | M | M | M | M | M | M | M | M | M | M | M | E.1/143 | No |  |  |
|  | Network Search Mode | Rel-6 | 1.10 |  |  |  |  |  |  | M | M | M | M | M | M | M | M | E.1/144 | E-USS, UMTS System Simulator or System Simulator |  |  |
|  | Charge State of the Battery | Rel-6 | 1.11 |  |  |  | C139 | C139 | C139 | C139 | C139 | C139 | C139 | C139 | C139 | C139 | C139 | E.1/170 | No |  |  |
|  | Intra-frequency UTRAN measurements | Rel-6 | 1.12 |  |  |  | M | M | C184 | C184 | C184 | C184 | C184 | C184 | C184 | C184 | C184 | E.1/183 | UMTS System Simulator only |  |  |
|  | Inter-frequency UTRAN measurements | Rel-6 | 1.13 |  |  |  | M | M | C184 | C184 | C184 | C184 | C184 | C184 | C184 | C184 | C184 | E.1/183 | UMTS System Simulator only |  |  |
|  | Access Technology, E-UTRAN | Rel-8 | 1.14 |  |  |  |  |  | C190 | C190 | C190 | C190 | C190 | C222 | C222 | C222 | C222 | E.1/72 | E-USS or NB-SS (See NOTE) |  |  |
|  | E-UTRAN Intra-Frequency Measurements | Rel-8 | 1.15 |  |  |  |  |  | C190 | C190 | C190 | C190 | C190 | C190 | C190 | C190 | C190 | E.1/183 | E-USS only |  |  |

| Item | Description | Re-lease | Test sequence (s) | Rel 99 ME | Rel-4 ME | Rel-5 ME | Rel-6 ME | Rel-7 ME | Rel-8 ME | Rel-9 ME | Rel-10 ME | Rel-11 ME | Rel-12 ME | Rel-13 ME | Rel-14 ME | Rel-15 ME | Rel-16 ME | Terminal Profile | Network Dependency | Sup-port | Additional test case execution parameter |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | E-UTRAN Intrer-Frequency Measurements | Rel-8 | 1.16 |  |  |  |  |  | C190 | C190 | C190 | C190 | C190 | C190 | C190 | C190 | C190 | E.1/183 | E-USS only |  |  |
|  | E-UTRAN Local Info (MCC, MNC, TAC & E-UTRAN Cell ID) | Rel-8 | 1.17 |  |  |  |  |  | C190 | C190 | C190 | C190 | C190 | C222 | C222 | C222 | C222 | E.1/31 AND E.1/135 | E-USS or NB-SS (See NOTE) |  |  |
|  | Discovery of surrounding CSG cells | Rel-9 | 1.18 |  |  |  |  |  |  | C195 | C195 | C195 | C195 | C195 | C195 | C195 | C195 | E.1/242 | E-USS only |  |  |
|  | Location Information for multiple Access Technologies | Rel-8 | 1.19 |  |  |  |  |  | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |  |  |
|  | NMR for multiple Access Technologies | Rel-8 | 1.20 |  |  |  |  |  | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |  |  |
|  | Current access technologies, multiple Access Technologies | Rel-8 | 1.21 |  |  |  |  |  | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |  |  |
|  | NG-RAN Local Info (MCC, MNC, TAC & NG-RAN Cell ID) | Rel-15 | 1.22 |  |  |  |  |  |  |  |  |  |  |  |  | C231 | C231 | E.1/31 | NG-SS only |  |  |
|  | Access Technology, NG-RAN | Rel-15 | 1.23 |  |  |  |  |  |  |  |  |  |  |  |  | C231 | C231 | E.1/72 | NG-SS only |  |  |
|  | Slices Information | Rel-16 | 1.xx |  |  |  |  |  |  |  |  |  |  |  |  |  | C231 | E.1/284 | NG-SS only |  |  |
| 19 | SET UP EVENT LIST 27.22.4.16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

…

| C206 | IF (A.1/139 OR A.1/140) AND A.1/153 THEN M ELSE N/A | -- (pc_eFDD OR pc_eTDD) AND O_SMS_SGs_MO |
| --- | --- | --- |
| C207 | IF A.1/147 AND A.1/148 AND A.1/150 THEN M ELSE O | -- O_Event_IMS_Registration AND O_UICC_ACCESS_IMS AND O_IMS |
| C208 | IF A.1/146 AND A.1/147 AND A.1/148 AND A.1/150 THEN M ELSE N/A | -- O_Event_Incoming_IMS_Data AND O_Event_IMS_Registration AND O_UICC_ACCESS_IMS AND O_IMS |
| C209 | IF (A.1/157 OR A.1/159) THEN M ELSE N/A | -- (pc_SMS_CS_MO OR pc_SMS_PS_MO) |
| C210 | IF (NOT A.1/135) AND (A.1/64 OR A.1/134) AND (A.1/157 OR A.1/159) THEN M ELSE N/A | -- (NOT (O_EUTRAN_NO_UTRAN_NO_GERAN) AND (O_GERAN OR O_UTRAN)) AND (pc_SMS_CS_MO OR pc_SMS_PS_MO) |
| C211 | IF (A.1/156 OR A.1/158) THEN M ELSE N/A | -- (pc_SMS_CS_MT OR pc_SMS_PS_MT) |
| C212 | IF (NOT A.1/135) AND (A.1/64 OR A.1/134) AND (A.1/156 OR A.1/158) THEN M ELSE N/A | -- (NOT (O_EUTRAN_NO_UTRAN_NO_GERAN) AND (O_GERAN OR O_UTRAN)) AND (pc_SMS_CS_MT OR pc_SMS_PS_MT) |
| C213 | IF (NOT A.1/160) THEN M ELSE N/A | -- NOT O_Rej_Launch_Browser_withDefURL |
| C214 | IF A.1/160 THEN M ELSE N/A | -- O_Rej_Launch_Browser_withDefURL |
| C215 | IF A.1/16 THEN M ELSE N/A | -- O_GPRS |
| C216 | IF A.1/161 THEN M ELSE N/A | -- O_Lang_Select |
| C217 | IF A.1/162 THEN M ELSE N/A | -- O_Provide_Local_LS |
| C218 | IF A.1/163 THEN M ELSE N/A | -- O_Lang_Notif |
| C219 | IF A.1/164 THEN M ELSE N/A | -- O_Refresh_AlphaIdentifier |
| C220 | IF (A.1/139 OR A.1/140 OR A.1/173) AND A.1/153 THEN M ELSE N/A | -- (pc_eFDD OR pc_eTDD OR pc_NB) AND O_SMS_SGs_MO |
| C221 | IF (A.1/139 OR A.1/140 OR A.1/173) AND A.1/152 THEN M ELSE N/A | -- (pc_eFDD OR pc_eTDD OR pc_NB) AND O_SMS_SGs_MT |
| C222 | IF (A.1/139 OR A.1/140 OR A.1/173) THEN M ELSE N/A | -- pc_eTDD OR pc_eFDD OR pc_NB |
| C223 | IF A.1/18 AND (A.1/132 OR A.1/133 OR A.1/177) THEN M ELSE N/A | -- O_TCP AND (pc_BIP_eFDD OR pc_BIP_eTDD OR pc_BIP_NB) |
| C224 | IF A.1/18 AND A.1/178 AND (A.1/132 OR A.1/133 OR A.1/177) THEN M ELSE N/A | -- O_TCP AND pc_Multiple_PDN AND (pc_BIP_eFDD OR pc_BIP_eTDD OR pc_BIP_NB) |
| C225 | IF A.1/18 AND (A.1/132 OR A.1/133 OR A.1/177) AND A.1/182 THEN M ELSE N/A | -- O_TCP AND (pc_BIP_eFDD OR pc_BIP_eTDD OR pc_BIP_NB) AND O_PSM_SUSPEND_UICC |
| C226 | IF A.1/18 AND (A.1/132 OR A.1/133 OR A.1/177) AND A.1/181 THEN M ELSE N/A | -- O_TCP AND (pc_BIP_eFDD OR pc_BIP_eTDD OR pc_BIP_NB) AND O_PSM_DEAC_UICC |
| C227 | IF A.1/18 AND (A.1/132 OR A.1/133 OR A.1/177) AND A.1/183 THEN M ELSE N/A | -- O_TCP AND (pc_BIP_eFDD OR pc_BIP_eTDD OR pc_BIP_NB) AND O_eDRX_SUSPEND_UICC |
| C228 | IF (A.1/132 OR A.1/133) AND A.1/152 AND A.1/184 THEN M ELSE N/A | -- (pc_BIP_eFDD OR pc_BIP_eTDD) AND O_SMS_SGs_MT AND O_PS_Data_Off |
| C229 | IF (A.1/132 OR A.1/133 OR A.1/177) THEN M ELSE N/A | -- pc_BIP_eFDD OR pc_BIP_eTDD OR pc_BIP_NB |
| C230 | A.1/17 AND A.1/178 AND (A.1/132 OR A.1/133 OR A.1/177) THEN M ELSE N/A | -- O_UDP AND pc_Multiple_PDN AND (pc_BIP_eFDD OR pc_BIP_eTDD OR pc_BIP_NB) |
| C231 | IF A.1/187 THEN M ELSE N/A | -- pc_NG_RAN |
| C232 | IF (A.1/187 AND A.1/188) THEN M ELSE N/A | -- pc_NG_RAN AND pc_BIP_NG_RAN |
| C233 | IF (A.1/191) THEN M ELSE N/A | --O_SUPI_NAI |

***** next change *****

##### 27.22.4.15.3	Test purpose

To verify that the ME returns the following requested local information within a TERMINAL RESPONSE:

-	location information:

-	Mobile Country Code (MCC);

-	Mobile Network Code (MNC);

-	Location Area Code (LAC); and

-	cell ID of the current serving cell;

-	the IMEI of the ME;

-	the Network Measurement Results and the BCCH channel list;

-	the current date, time and time zone;

-	the current ME language setting;

-	the Timing Advance;

-	the Access Technology;

-	the IMEISV;

-	the Search Mode change;

-	the Battery charge State;

-	the UTRAN intra- and inter-frequency measurements;

-	the E-UTRAN intra- and inter-frequency measurements

-	the CSG ID list and corresponding HNB names of surrounding CSG cells (if class "q" is supported);

-	the list of slice(s) information.

If the local information is stored in the ME; otherwise, sends the correct error code to the UICC in the TERMINAL RESPONSE.

To verify that the ME returns required error information in the TERMINAL RESPONSE in case requested information cannot be provided due to missing network coverage.

To verify that the E-UTRAN cell identifier is correctly transmitted when requesting the location information while accessing an E-UTRAN.

To verify that the NG-RAN cell identifier is correctly transmitted when requesting the location information while accessing an NG-RAN SA mode Cell.

To verify that the NG-RAN cell identifier is correctly transmitted when requesting the location information while accessing an NG-RAN Cell.

To verify that the Served S-NSSAIs are correctly transmitted in the TERMINAL RESPONSE when requesting the list of slice(s) information.

***** next change *****

##### 27.22.4.15.4	Method of tests

27.22.4.15.4.1	Initial conditions

The ME is connected to the USIM Simulator.

Except for sequences 1.2, 1.4, 1.5, 1.9, 1.10 and 1.11 the ME is connected to the USS and except for sequence 1.10 has performed the location update procedure or routing area update or combined update procedure.

For sequence 1.22, 1.23 and 1.xx the ME is connected to NG-SS and it has performed the Registration procedure.

The NG-RAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Tracking Area Code (TAC) = 000001;

-	NG-RAN Cell Identity value = 0001 (36 bits);

-	Allowed  S-NSSAIs =   01010103 (SST: MBB, SD: 010103);

The E-UTRAN/NB-IoT parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Tracking Area Code (TAC) = 0001;

-	E-UTRAN Cell Identity value = 0001 (28 bits);

The UTRAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Location Area Code (LAC) = 0001;

-	Cell Identity value = 0001;

The GERAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Location Area Code (LAC) = 0001;

-	Cell Identity value = 0001;

-	Timing advance = 0;

-	Neighbour allocations = 561, 565, 568, 569, 573, 575, 577, 581, 582 and 585.

The PCS 1900 parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 011;

-	Location Area Code (LAC) = 0001;

-	Cell Identity value = 0001;

-	Timing advance = 0;

-	Neighbour allocations = 561, 565, 568, 569, 573, 575, 577, 581, 582 and 585.

The elementary files are coded as the USIM Application Toolkit default with the exception that for sequences 1.14 to 1.18, the default E-UTRAN/EPC UICC is used and for sequence 1.22, 1.23 and 1.xx the default NG-RAN UICC is used.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

Expected sequence 1.3 and 1.6 shall be used on a USS setting up only a GERAN or PCS 1900 cell and expected sequences 1.7 and 1.12 shall be used on a USS setting up only a UTRAN cell.

Expected sequence 1.12 requires 2 UTRA cells on the same frequency and 1.13 requires 2 UTRA cells on different frequencies.

Expected sequences 1.14 and 1.17 shall be used on a E-USS/NB-SS setting up only a E-UTRAN/NB-IoT cell.

Expected sequence 1.22, 1.23 and 1.xx shall be used on a NG-SS setting up only a NG-RAN cell.

Expected sequence 1.15 requires 2 E-UTRA cells on the same frequency and 1.16 requires 2 E-UTRA cells on different frequencies, with second cell having EARFCN less than maxEARFCN. For both sequences if the USIM request is triggered in the RRC_CONNECTED state, the system simulator shall configure the corresponding frequency measurement for a sufficient period before sending the USIM request.

Expected sequence 1.18 requires 2 E-UTRAN cells configured in CSG mode.

For sequence 1.18 the default E-UTRAN/EPC UICC is used and the E-USS transmits on 2 cells with the following parameters:

Network parameters for cell 1:

-	TAI (MCC/MNC/TAC):	001/01/0001.

-	Access control:	unrestricted.

-	csg-Indication:	TRUE

-	csg-Identity:	01 (27 bits)

- Home (e)NB Name	Home ONE

Network parameters for cell 2:

-	TAI (MCC/MNC/TAC):	001/01/0002.

-	Access control:	unrestricted.

-	csg-Indication:	TRUE

-	csg-Identity:	02 (27 bits)

- Home (e)NB Name	Home TWO

***** next change *****

27.22.4.15.4.2	Procedure

Expected Sequence 1.1 (PROVIDE LOCAL INFORMATION, Local Info (MCC, MNC, LAC & Cell ID))

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING PROVIDE LOCAL INFORMATION 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.1.1 |  |
| 4 | ME UICC | TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.1.1A or TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.1.1B | [Command performed successfully, MCC MNC LAC and Cell Identity as USS, option A shall apply for 3GPP parameters] [Command performed successfully, MCC MNC LAC and Cell Identity as USS, option B shall apply for PCS1900 parameters] |

PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.1.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"00" Location information (MCC MNC LAC and Cell Identity)

Device identities

Source device:	UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 26 | 00 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.1.1A

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"00" Location information (MCC MNC LAC and Cell Identity)

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Location Information

MCC & MNC:	MCC = 001, MNC = 01

Location Area Code:	0001

Cell Identity Value:	0001

Extended Cell Identity Value:	RNC-id value (for Rel-4 onwards), see also Note 2

Coding:

| BER-TLV: | 81 | 03 | 01 | 26 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 93 | Note 1 | 00 | F1 | 10 | 00 | 01 | 00 | 01 | Note 2 |  |  |

Note 1: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'

Note 2: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.1.1B

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"00" Location information (MCC MNC LAC and Cell Identity)

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Location Information

MCC & MNC:	MCC = 001, MNC = 011

Location Area Code:	0001

Cell Identity Value:	0001

Coding:

| BER-TLV: | 81 | 03 | 01 | 26 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 93 | 07 | 00 | 11 | 10 | 00 | 01 | 00 | 01 |  |  |  |

…

Expected Sequence 1.23 (PROVIDE LOCAL INFORMATION, Access Technology, NG-RAN)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING PROVIDE LOCAL INFORMATION 1.14.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.14.1 |  |
| 4 | ME UICC | TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.23.1 | [Command performed successfully] |

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.23.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"06" Access Technology

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Access Technology

Technology:	3GPP NR

Coding:

| BER-TLV: | 81 | 03 | 01 | 26 | 06 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3F | 01 | 0A |  |  |  |  |  |  |  |  |  |

Expected Sequence 1.xx (PROVIDE LOCAL INFORMATION, slice(s) information)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | ME NG-SS | ME successfully REGISTER with NG-RAN cell. |  |
| 2 | ME NG-SS | An Internet PDU Session is established successfully. | Served S-NSSAIs = 01010103 |
| 3 | UICC ME | PROACTIVE COMMAND PENDING PROVIDE LOCAL INFORMATION 1.xx.1 |  |
| 4 | ME UICC | FETCH |  |
| 5 | UICC ME | PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.xx.1 |  |
| 6 | ME UICC | TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.xx.1 | [Command performed successfully] |

PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.xx.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"15" slices information

Device identities

Source device:	UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 26 | 15 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.xx.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"15" slices information

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Slice Information

Number of Served S-NSSAIs: 1

Served S-NSSAI:  '01 01 01 03' (SST: MBB, SD: 010103)

Coding:

| BER-TLV: | 81 | 03 | 01 | 26 | 15 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D6 | 05 | 01 | 01 | 01 | 01 | 03 |  |  |  |  |  |

***** next change *****