3GPP TSG-CT WG4 Meeting #114	C4-230725

# Athens, Greece, 27th Feb – 03rd Mar 2023						Revision of C4-230621, C4-230413

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.502 | CR | 0630 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PDU Session for supporting HR-SBO in VPLMN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-03-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Procedure for PDU Session supporting HR-SBO in VPLMN is defined in clause 6.7.2.2 of 3GPP TS 23.548, and more updates agreed in CR0084 (S2-2301396), with the following requirement from AMF to V-SMF: the AMF selects a V-SMF supporting HR-SBO and sends an HR-SBO allowed indication to the V-SMF… From V-SMF to H-SMF: send to the H-SMF the request for the establishment of the PDU Session supporting HR-SBO in VPLMN and the V-EASDF IP address to in the Nsmf_PDUSession_Create Request… From H-SMF to V-SMF: If HR-SBO is allowed for the PDU session, the H-SMF provides in the Nsmf_PDUSession_Create Response in the step 13 of the procedure in the clause 4.3.2.2.2-1 of TS 23.502 [3] with the following information: - optional VPLMN specific offloading policy - the V-EASDF IP address (corresponding to clause 6.7.2.3) or DNS server IP address of HPLMN (corresponding to clause 6.7.2.X) as DNS server address to be sent to the UE via PCO and - the DNS server address of HPLMN (to be used for DNS requests related with traffic not to be subject to HR-SBO) - the HR-SBO authorization result (i.e. whether HR-SBO request is authorized or not). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Include HR-SBO allowed indication in SmContextCreate Data； - Include HR-SBO authorization request indication and V-EASDF address in PduSessionCreateData; - Include HR-SBO authorized Indication and DNS server address in HPLMN in PduSessionCreatedData. - Define new feature for HR-SBO. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with stage2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 6.1.6.1, 6.1.6.2.2, 6.1.6.2.9, 6.1.6.2.10, 6.1.6.2.x(new), 6.1.6.2.y(new), 6.1.8, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.548 CR 0084 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This contribution introduces backward compatible new features to the OpenAPI file of Nsmf_PDUSession API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1:Define new feature for HR-SBO. Rev2: authentication is changed to authorization. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BP	Branching Point

DNAI	Data Network Access Identifier

DNN	Data Network Name

HR	Home Routed

HR-SBO	Home Routed Session BreakOut

H-SMF	Home SMF

I-SMF	Intermediate SMF

JSON	Javascript Object NotationNAS	Non-Access Stratum

LADN	Local Area Data Network

MA	Multi-Access

MO	Mobile Originated

MT	Mobile TerminatedPSA	PDU Session Anchor

RSN	Redundancy Sequence Number

SM	Session Management

SMF	Session Management Function

SNPN	Stand-alone Non-Public Network

TNGF	Trusted Non-3GPP Gateway Function

TWIF	Trusted WLAN Interworking Function

UAS	Uncrewed Aerial System

UAV	Uncrewed Aerial Vehicle

UL CL	Uplink Classifier

UPF	User Plane Function

USS	UAS Service Supplier

UUAA	USS UAV Authorization/Authentication

V-SMF	Visited SMF

W-AGF	Wireline Access Gateway Function

* * * Next Change * * * *

6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nsmf service based interface protocol.

Table 6.1.6.1-1: Nsmf specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| SmContextCreateData | 6.1.6.2.2 | Data within Create SM Context Request |
| SmContextCreatedData | 6.1.6.2.3 | Data within Create SM Context Response |
| SmContextUpdateData | 6.1.6.2.4 | Data within Update SM Context Request |
| SmContextUpdatedData | 6.1.6.2.5 | Data within Update SM Context Response |
| SmContextReleaseData | 6.1.6.2.6 | Data within Release SM Context Request |
| SmContextRetrieveData | 6.1.6.2.7 | Data within Retrieve SM Context Request |
| SmContextStatusNotification | 6.1.6.2.8 | Data within Notify SM Context Status Request |
| PduSessionCreateData | 6.1.6.2.9 | Data within Create Request |
| PduSessionCreatedData | 6.1.6.2.10 | Data within Create Response |
| HsmfUpdateData | 6.1.6.2.11 | Data within Update Request towards H-SMF, or from I-SMF to SMF |
| HsmfUpdatedData | 6.1.6.2.12 | Data within Update Response from H-SMF, or from SMF to I-SMF |
| ReleaseData | 6.1.6.2.13 | Data within Release Request |
| HsmfUpdateError | 6.1.6.2.14 | Error within Update Response from H-SMF, or from SMF to I-SMF |
| VsmfUpdateData | 6.1.6.2.15 | Data within Update Request towards V-SMF, or from SMF to I-SMF |
| VsmfUpdatedData | 6.1.6.2.16 | Data within Update Response from V-SMF, or from I-SMF to SMF |
| StatusNotification | 6.1.6.2.17 | Data within Notify Status Request |
| QosFlowItem | 6.1.6.2.18 | Individual QoS flow |
| QosFlowSetupItem | 6.1.6.2.19 | Individual QoS flow to setup |
| QosFlowAddModifyRequestItem | 6.1.6.2.20 | Individual QoS flow requested to be created or modified |
| QosFlowReleaseRequestItem | 6.1.6.2.21 | Individual QoS flow requested to be released |
| QosFlowProfile | 6.1.6.2.22 | QoS flow profile |
| GbrQosFlowInformation | 6.1.6.2.23 | GBR QoS flow information |
| QosFlowNotifyItem | 6.1.6.2.24 | Notification related to a QoS flow |
| SmContextRetrievedData | 6.1.6.2.27 | Data within Retrieve SM Context Response |
| TunnelInfo | 6.1.6.2.28 | Tunnel Information |
| StatusInfo | 6.1.6.2.29 | Status of SM context or of PDU session |
| VsmfUpdateError | 6.1.6.2.30 | Error within Update Response from V-SMF, or from I-SMF to SMF |
| EpsPdnCnxInfo | 6.1.6.2.31 | EPS PDN Connection Information from H-SMF to V-SMF, or from SMF to I-SMF |
| EpsBearerInfo | 6.1.6.2.32 | EPS Bearer Information from H-SMF to V-SMF, or from SMF to I-SMF |
| PduSessionNotifyItem | 6.1.6.2.33 | Notification related to a PDU session |
| EbiArpMapping | 6.1.6.2.34 | EBI to ARP mapping |
| SmContextCreateError | 6.1.6.2.35 | Error within Create SM Context Response |
| SmContextUpdateError | 6.1.6.2.36 | Error within Update SM Context Response |
| PduSessionCreateError | 6.1.6.2.37 | Error within Create Response |
| MmeCapabilities | 6.1.6.2.38 | MME capabilities |
| SmContext | 6.1.6.2.39 | Complete SM Context |
| ExemptionInd | 6.1.6.2.40 | Exemption Indication |
| PsaInformation | 6.1.6.2.41 | PSA Information |
| DnaiInformation | 6.1.6.2.42 | DNAI Information |
| N4Information | 6.1.6.2.43 | N4 Information |
| IndirectDataForwardingTunnelInfo | 6.1.6.2.44 | Indirect Data Forwarding Tunnel Information |
| SmContextReleasedData | 6.1.6.2.45 | Data within Release SM Context Response |
| ReleasedData | 6.1.6.2.46 | Data within Release Response |
| SendMoDataReqData | 6.1.6.2.47 | Data within Send MO Data Request |
| CnAssistedRanPara | 6.1.6.2.48 | SMF derived CN assisted RAN parameters tuning |
| UlclBpInformation | 6.1.6.2.49 | UL CL or BP Information |
| TransferMoDataReqData | 6.1.6.2.50 | Data within Transfer MO Data Request |
| TransferMtDataReqData | 6.1.6.2.51 | Data within Transfer MT Data Request |
| TransferMtDataError | 6.1.6.2.52 | Transfer MT Data Error Response |
| TransferMtDataAddInfo | 6.1.6.2.53 | Transfer MT Data Error Response Additional Information |
| VplmnQos | 6.1.6.2.54 | VPLMN QoS |
| DdnFailureSubs | 6.1.6.2.55 | DDN Failure Subscription |
| RetrieveData | 6.1.6.2.56 | Data within Retrieve Request |
| RetrievedData | 6.1.6.2.57 | Data within Retrieve Response |
| SecurityResult | 6.1.6.2.58 | Security Result |
| UpSecurityInfo | 6.1.6.2.59 | User Plane Security Information |
| DdnFailureSubInfo | 6.1.6.2.60 | DDN Failure Subscription Information |
| AlternativeQosProfile | 6.1.6.2.61 | Alternative QoS Profile |
| ProblemDetailsAddInfo | 6.1.6.2.62 | Problem Details Additional Information |
| ExtProblemDetails | 6.1.6.2.63 | Extended Problem Details |
| QosMonitoringInfo | 6.1.6.2.64 | QoS Monitoring Information |
| IpAddress | 6.1.6.2.65 | IP Address |
| RedundantPduSessionInformation | 6.1.6.2.66 | Redundant PDU Session Information |
| QosFlowTunnel | 6.1.6.2.67 | Tunnel Information per QoS Flow |
| TargetDnaiInfo | 6.1.6.2.68 | Target DNAI Information |
| AfCoordinationInfo | 6.1.6.2.69 | AF Coordination Information |
| NotificationInfo | 6.1.6.2.70 | Notification Correlation ID and Notification URI provided by the NF service consumer |
| AnchorSmfFeatures | 6.1.6.2.71 | Anchor SMF supported features |
| HrsboReqInfo | 6.1.6.2.x | HR-SBO Information in request message |
| HrsboRspInfo | 6.1.6.2.y | HR-SBO Information in response message |
| Teid | 6.1.6.3.2 | GTP Tunnel Endpoint Identifier |
| ProcedureTransactionId | 6.1.6.3.2 | Procedure Transaction Identifier |
| EpsPdnCnxContainer | 6.1.6.3.2 | UE EPS PDN Connection container from SMF to AMF |
| EpsBearerId | 6.1.6.3.2 | EPS Bearer Id |
| EpsBearerContainer | 6.1.6.3.2 | EPS Bearer container from SMF to AMF |
| EpsBearerContextStatus | 6.1.6.3.2 | EPS Bearer context status |
| DrbId | 6.1.6.3.2 | Data Radio Bearer Identifier |
| UpCnxState | 6.1.6.3.3 | User Plane Connection State |
| HoState | 6.1.6.3.4 | Handover State |
| RequestType | 6.1.6.3.5 | Request Type in Create (SM context) service operation. |
| RequestIndication | 6.1.6.3.6 | Request Indication in Update (SM context) service operation. |
| NotificationCause | 6.1.6.3.7 | Cause for generating a notification |
| Cause | 6.1.6.3.8 | Cause information |
| ResourceStatus | 6.1.6.3.9 | Status of SM context or PDU session resource |
| DnnSelectionMode | 6.1.6.3.10 | DNN Selection Mode |
| EpsInterworkingIndication | 6.1.6.3.11 | EPS Interworking Indication |
| N2SmInfoType | 6.1.6.3.12 | N2 SM Information Type |
| MaxIntegrityProtectedDataRate | 6.1.6.3.13 | Maximum Integrity Protected Data Rate |
| MaReleaseIndication | 6.1.6.3.14 | Multi-Access PDU session release Indication |
| SmContextType | 6.1.6.3.15 | Type of SM Context information |
| PsaIndication | 6.1.6.3.16 | Indication of whether a PSA is inserted or removed |
| N4MessageType | 6.1.6.3.17 | N4 Message Type |
| QosFlowAccessType | 6.1.6.3.18 | Access type associated with the QoS Flow |
| UnavailableAccessIndication | 6.1.6.3.19 | Indicates the access type of a MA PDU session that is unavailable |
| ProtectionResult | 6.1.6.3.20 | Protection Result of the security policy indicated as "preferred" |
| QosMonitoringReq | 6.1.6.3.21 | Indicates to measure UL, or DL, or both UL/DL delays, or to stop on-going measurements. |
| Rsn | 6.1.6.3.22 | Redundancy Sequence Number |
| SmfSelectionType | 6.1.6.3.23 | SMF Selection Type |
| PduSessionContextType | 6.1.6.3.24 | PDU Session Context Type |

Table 6.1.6.1-2 specifies data types re-used by the Nsmf service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nsmf service based interface.

Table 6.1.6.1-2: Nsmf re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Uint32 | 3GPP TS 29.571 [13] | Unsigned 32-bit integers |
| Ipv4Addr | 3GPP TS 29.571 [13] | IPv4 Address |
| Ipv6Prefix | 3GPP TS 29.571 [13] | IPv6 Prefix |
| Uri | 3GPP TS 29.571 [13] | Uniform Resource Identifier |
| Supi | 3GPP TS 29.571 [13] | Subscription Permanent Identifier |
| Pei | 3GPP TS 29.571 [13] | Permanent Equipment Identifier |
| Gpsi | 3GPP TS 29.571 [13] | General Public Subscription Identifier |
| AccessType | 3GPP TS 29.571 [13] | Access Type (3GPP or non-3GPP access) |
| SupportedFeatures | 3GPP TS 29.571 [13] | Supported features |
| Qfi | 3GPP TS 29.571 [13] | QoS Flow Identifier |
| PduSessionId | 3GPP TS 29.571 [13] | PDU Session Identifier |
| PduSessionType | 3GPP TS 29.571 [13] | PDU Session Type |
| Ambr | 3GPP TS 29.571 [13] | PDU Session Aggregate Maximum Bit Rate |
| 5Qi | 3GPP TS 29.571 [13] | 5G QoS Identifier |
| Arp | 3GPP TS 29.571 [13] | Allocation and Retention Priority |
| ReflectiveQoSAttribute | 3GPP TS 29.571 [13] | Reflective QoS Attribute |
| Dynamic5Qi | 3GPP TS 29.571 [13] | QoS characteristics for a 5QI that is neither standardized nor pre-configured. |
| NonDynamic5Qi | 3GPP TS 29.571 [13] | QoS characteristics that replace the default QoS characteristics for a standardized or pre-configured 5QI. |
| PacketLossRate | 3GPP TS 29.571 [13] | Packet Loss Rate |
| NotificationControl | 3GPP TS 29.571 [13] | Notification Control |
| Dnn | 3GPP TS 29.571 [13] | Data Network Name |
| Snssai | 3GPP TS 29.571 [13] | Single Network Slice Selection Assistance Information |
| NfInstanceId | 3GPP TS 29.571 [13] | NF Instance Identifier |
| UserLocation | 3GPP TS 29.571 [13] | User Location |
| TimeZone | 3GPP TS 29.571 [13] | Time Zone |
| ProblemDetails | 3GPP TS 29.571 [13] | Error description |
| UpSecurity | 3GPP TS 29.571 [13] | User Plane Security Policy Enforcement information |
| RefToBinaryData | 3GPP TS 29.571 [13] | Cross-Reference to binary data encoded within a binary body part in an HTTP multipart message. |
| Guami | 3GPP TS 29.571 [13] | Globally Unique AMF ID |
| BackupAmfInfo | 3GPP TS 29.571 [13] | Backup AMF Information |
| PresenceState | 3GPP TS 29.571 [13] | Indicates the UE presence in or out of a LADN service area |
| TraceData | 3GPP TS 29.571 [13] | Trace control and configuration parameters |
| PlmnId | 3GPP TS 29.571 [13] | PLMN Identity |
| RatType | 3GPP TS 29.571 [13] | RAT Type |
| NgApCause | 3GPP TS 29.571 [13] | NGAP Cause |
| 5GMmCause | 3GPP TS 29.571 [13] | 5G MM Cause |
| DurationSec | 3GPP TS 29.571 [13] | Duration in units of seconds |
| AdditionalQosFlowInfo | 3GPP TS 29.571 [13] | Additional QoS Flow Information |
| NfGroupId | 3GPP TS 29.571 [13] | Network Function Group Id |
| SecondaryRatUsageReport | 3GPP TS 29.571 [13] | Secondary RAT Usage Report |
| SecondaryRatUsageInfo | 3GPP TS 29.571 [13] | Secondary RAT Usage Information |
| Dnai | 3GPP TS 29.571 [13] | Data Network Access Identifier |
| PlmnIdNid | 3GPP TS 29.571 [13] | PLMN Identity and, for SNPN, Network Identity |
| SmallDataRateStatus | 3GPP TS 29.571 [13] | Small Data Rate Control Status |
| ApnRateStatus | 3GPP TS 29.571 [13] | APN Rate Control Status |
| StationaryIndication | 3GPP TS 29.571 [13] | Stationary Indication |
| ScheduledCommunicationTime | 3GPP TS 29.571 [13] | Scheduled Communication Time |
| ScheduledCommunicationType | 3GPP TS 29.571 [13] | Scheduled Communication Type |
| TrafficProfile | 3GPP TS 29.571 [13] | Traffic Profile |
| BatteryIndication | 3GPP TS 29.571 [13] | Battery Indication |
| NfSetId | 3GPP TS 29.571 [13] | NF Set Identifier |
| MoExpDataCounter | 3GPP TS 29.571 [13] | MO Exception Data Counter |
| DddTrafficDescriptor | 3GPP TS 29.571 [13] | Traffic Descriptor |
| NfServiceSetId | 3GPP TS 29.571 [13] | NF Service Set ID |
| RedirectResponse | 3GPP TS 29.571 [13] | Response body of the redirect response message |
| ServerAddressingInfo | 3GPP TS 29.571 [13] | Information of a Provisioning Server |
| PcfUeCallbackInfo | 3GPP TS 29.571 [13] | The callback information of the PCF for the UE to allow the PCF for the PDU session to send SM Policy Association Establishment and Termination events notification |
| SatelliteBackhaulCategory | 3GPP TS 29.571 [13] | Satellite backhaul category |
| Uint16 | 3GPP TS 29.571 [13] | Unsigned 16-bit integer |
| ServiceName | 3GPP TS 29.510 [19] | Service Name |
| WAgfInfo | 3GPP TS 29.510 [19] | Information about N3 terminations at the W-AGF |
| TngfInfo | 3GPP TS 29.510 [19] | Information about N3 terminations at the TNGF |
| TwifInfo | 3GPP TS 29.510 [19] | Information about N3 terminations at the TWIF |
| ChargingInformation | 3GPP TS 29.512 [30] | CHF addresses |
| NgRanTargetId | 3GPP TS 29.518 [20] | NG-RAN Target Id |
| SbiBindingLevel | 3GPP TS 29.518 [20] | SBI Binding Level |
| IpIndex | 3GPP TS 29.519 [38] | Information that identifies which IP pool or external server is used to allocate the IP address. |
| RoamingChargingProfile | 3GPP TS 32.291 [26] | Roaming Charging Profile |

* * * Next Change * * * *

6.1.6.2.2	Type: SmContextCreateData

Table 6.1.6.2.2-1: Definition of type SmContextCreateData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supi | Supi | C | 0..1 | This IE shall be present, except if the UE is emergency registered and UICCless. When present, it shall contain the subscriber permanent identify. |  |
| unauthenticatedSupi | boolean | C | 0..1 | This IE shall be present if the SUPI is present in the message but is not authenticated and is for an emergency registered UE. When present, it shall be set as follows: - true: unauthenticated SUPI; - false (default): authenticated SUPI. |  |
| pei | Pei | C | 0..1 | This IE shall be present if the UE is emergency registered and it is either UIClless or the SUPI is not authenticated. For all other cases, this IE shall be present if it is available. When present, it shall contain the permanent equipment identifier. |  |
| gpsi | Gpsi | C | 0..1 | This IE shall be present if it is available. When present, it shall contain the user's GPSI. |  |
| pduSessionId | PduSessionId | C | 0..1 | This IE shall be present, except during an EPS to 5GS Idle mode mobility or handover using the N26 interface. When present, it shall contain the PDU Session ID. |  |
| dnn | Dnn | C | 0..1 | This IE shall be present, except during an EPS to 5GS Idle mode mobility or handover using the N26 interface. When present, it shall contain the requested DNN; the DNN shall be the full DNN (i.e. with both the Network Identifier and Operator Identifier) for a HR PDU session, and it should be the full DNN in LBO and non-roaming scenarios. If the Operator Identifier is absent, the serving core network operator shall be assumed. |  |
| selectedDnn | Dnn | C | 0..1 | This IE shall be present, if another DNN other than the UE requested DNN is selected for this PDU session. When present, it shall contain the selected DNN. The DNN shall be the full DNN (i.e. with both the Network Identifier and Operator Identifier) for a HR PDU session, and it should be the full DNN in LBO and non-roaming scenarios. If the Operator Identifier is absent, the serving core network operator shall be assumed. |  |
| sNssai | Snssai | C | 0..1 | This IE shall be present during the PDU session establishment procedure. In this case, it shall contain the requested S-NSSAI for the serving PLMN. This corresponds to an S-NSSAI from the allowed NSSAI. This IE shall also be present during an EPS to 5GS idle mode mobility or handover with I-SMF/V-SMF involved using the N26 interface. In this case, it shall contain the S-NSSAI configured in the AMF for EPS interworking. |  |
| hplmnSnssai | Snssai | C | 0..1 | This IE shall be present for a roaming PDU session, except during an EPS to 5GS idle mode mobility or handover using the N26 interface. When present, it shall contain the requested S-NSSAI for the HPLMN. This corresponds to an S-NSSAI from the Mapping Of Allowed NSSAI corresponding to the SNSSAI value included in the sNssai IE. |  |
| servingNfId | NfInstanceId | M | 1 | This IE shall contain the identifier of the serving NF (e.g. serving AMF). |  |
| guami | Guami | C | 0..1 | This IE shall contain the serving AMF's GUAMI. It shall be included if the NF service consumer is an AMF. |  |
| serviceName | ServiceName | O | 0..1 | When present, this IE shall contain the name of the AMF service to which SM context status notifications are to be sent (see clause 6.5.2.2 of 3GPP TS 29.500 [4]). This IE may be included if the NF service consumer is an AMF. |  |
| servingNetwork | PlmnIdNid | M | 1 | This IE shall contain the serving core network operator PLMN ID and, for an SNPN, the NID that together with the PLMN ID identifies the SNPN. |  |
| requestType | RequestType | C | 0..1 | This IE shall be present if the Request type IE is received from the UE for a single access PDU session and if the request refers to an existing PDU session or an existing emergency PDU session. The requestType IE shall not be included for a MA-PDU session establishment request. It may be present otherwise. When present, it shall indicate whether the request refers to a new PDU session or emergency PDU session, or to an existing PDU session or emergency PDU session. For request sent from UE, this IE shall be set based on the Request type IE received (see clause 9.11.3.47 of 3GPP TS 24.501 [7]). |  |
| n1SmMsg | RefToBinaryData | C | 0..1 | This IE shall be present and reference the N1 SM Message binary data (see clause 6.1.6.4.2), except during an EPS to 5GS Idle mode mobility or handover using N26. |  |
| anType | AccessType | M | 1 | This IE shall indicate the Access Network Type to which the PDU session is to be associated. |  |
| additionalAnType | AccessType | C | 0..1 | This IE shall indicate the additional Access Network Type to which the PDU session is to be associated. This IE shall be present if a MA-PDU session is requested and the UE is registered over both 3GPP access and Non-3GPP access. | MAPDU |
| ratType | RatType | C | 0..1 | This IE shall be present and indicate the RAT Type used by the UE, if available. |  |
| presenceInLadn | PresenceState | C | 0..1 | This IE shall be present if the DNN corresponds to a LADN. When present, it shall be set to "IN" or "OUT" to indicate that the UE is in or out of the LADN service area. |  |
| ueLocation | UserLocation | C | 0..1 | This IE shall contain the UE location information (see clause 5.2.3.4), if it is available. (NOTE 1). |  |
| ueTimeZone | TimeZone | C | 0..1 | This IE shall contain the UE Time Zone, if it is available. |  |
| addUeLocation | UserLocation | O | 0..1 | Additional UE location. This IE may be present, if anType indicates a non-3GPP access and valid 3GPP access user location information is available. When present, it shall contain: - the last known 3GPP access user location (see clause 5.2.3.4); and - the timestamp, if available, indicating the UTC time when the addUeLocation information was acquired. (NOTE 1) |  |
| smContextStatusUri | Uri | M | 1 | This IE shall include the callback URI to receive notification of SM context status. |  |
| hSmfUri | Uri | C | 0..1 | This IE shall be present in HR roaming scenarios, including Indirect Communication with Delegated Discovery, if the AMF and V-SMF do not support the ACSCR feature. This IE shall be present in HR roaming scenarios during a PDU session establishment procedure and EPS to 5GS mobility procedures, if the AMF and V-SMF support the ACSCR feature. When present, it shall contain the API URI of the Nsmf_PDUSession service of the selected H-SMF. The API URI shall be formatted as specified in clause 6.1.1. (NOTE 8) |  |
| hSmfId | NfInstanceId | O | 0..1 | This IE may be present when hSmfUri is present. If present, this IE shall carry the NF instance ID of the selected H-SMF. (NOTE 2) |  |
| smfUri | Uri | C | 0..1 | This IE shall be present for a PDU session with an I-SMF, including Indirect Communication with Delegated Discovery, if the AMF and I-SMF do not support the ACSCR feature. This IE shall be present for a PDU session with an I-SMF during a PDU session establishment procedure and EPS to 5GS mobility procedures, if the AMF and I-SMF support the ACSCR feature. When present, it shall contain the API URI of the Nsmf_PDUSession service of the selected SMF. The API URI shall be formatted as specified in clause 6.1.1. (NOTE 8) | DTSSA |
| smfId | NfInstanceId | O | 0..1 | This IE may be present when smfUri is present. If present, this IE shall carry the NF instance ID of the selected SMF. (NOTE 2) | DTSSA |
| oldPduSessionId | PduSessionId | C | 0..1 | This IE shall be present if this information is received from the UE. When present, it shall contain the old PDU Session ID received from the UE. See clauses 4.3.2.2.1 and 4.3.5.2 of 3GPP TS 23.502 [3]. |  |
| pduSessionsActivateList | array(PduSessionId) | C | 1..N | This IE shall be present, during an EPS to 5GS Idle mode mobility using the N26 interface, if the UE indicated PDU session(s) to be activated in the Registration Request. When present, it shall indicate all the PDU session(s) requested to be re-activated by the UE. |  |
| ueEpsPdnConnection | EpsPdnCnxContainer | C | 0..1 | This IE shall be present, during an EPS to 5GS Idle mode mobility or handover using the N26 interface. When present, it shall contain an MME/SGSN UE EPS PDN connection including the EPS bearer context(s). |  |
| hoState | HoState | C | 0..1 | This IE shall be present during an EPS to 5GS handover using N26 interface, to request the preparation of a handover of the PDU session. When present, it shall be set as specified in clause 5.2.2.2.3. |  |
| additionalHsmfUri | array(Uri) | O | 1..N | This IE may be present in HR roaming scenarios. When present, it shall contain an array of API URI of the Nsmf_PDUSession service of the additional H-SMFs discovered by the AMF for the given DNN, hplmnSnssai and for this PDU session. If provided, the V-SMF shall use these additional H-SMF(s) if the V-SMF is not able to receive any response from the H-SMF identified by hSmfUri. The API URI shall be formatted as specified in clause 6.1.1. |  |
| additionalHsmfId | array(NfInstanceId) | O | 1..N | This IE may be present when additionalHsmfUri is present. If present, this IE shall carry the NF instance ID(s) of H-SMF(s) as stated in additionalHsmfUri IE, in exactly the same order. (NOTE 2) |  |
| additionalSmfUri | array(Uri) | O | 1..N | This IE may be present for a PDU session with an I-SMF. When present, it shall contain an array of API URI of the Nsmf_PDUSession service of the additional SMFs discovered by the AMF for the given DNN, Snssai and for this PDU session. If provided, the I-SMF shall use these additional SMF(s) if the I-SMF is not able to receive any response from the SMF identified by smfUri. The API URI shall be formatted as specified in clause 6.1.1. | DTSSA |
| additionalSmfId | array(NfInstanceId) | O | 1..N | This IE may be present when additionalSmfUri is present. If present, this IE shall carry the NF instance ID(s) of SMF(s) as stated in additionalSmfUri IE, in exactly the same order. (NOTE 2) | DTSSA |
| pcfId | NfInstanceId | O | 0..1 | When present, this IE shall contain the identifier of: - the H-PCF selected by the AMF (for UE Policy), for a HR PDU session; or - the V-PCF selected by the AMF (for Access and Mobility Policy), for a PDU session in LBO roaming scenarios; or - the PCF selected by the AMF (for Access and Mobility Policy and/or UE Policy), for a PDU session in non-roaming scenarios. |  |
| pcfGroupId | NfGroupId | O | 0..1 | This IE may be present in non-roaming and HR roaming scenarios. When present, this IE shall contain the identity of the (home) PCF group serving the UE for Access and Mobility Policy and/or UE Policy. |  |
| pcfSetId | NfSetId | O | 0..1 | This IE may be present if pcfId IE is present. When present, this IE shall contain the NF Set ID of the PCF indicated by the pcfId IE. |  |
| nrfUri | Uri | O | 0..1 | This IE may be present to indicate the NRF to use for PCF selection within the same network slice instance. When present, the SMF shall use the NRF URI to select the PCF. |  |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one optional feature defined in clause 6.1.8 is supported. |  |
| selMode | DnnSelectionMode | C | 0..1 | This IE shall be present if it is available. When present, it shall be set to: - "VERIFIED", if the requested DNN provided by UE or the selected DNN provided by the network corresponds to an explicitly subscribed DNN; or - "UE_DNN_NOT_VERIFIED", if the requested DNN provided by UE corresponds to the usage of a wildcard subscription; or - "NW_DNN_NOT_VERIFIED", if the selected DNN provided by the network corresponds to the usage of a wildcard subscription. If both the requested DNN (i.e. dnn IE) and selected DNN (i.e. selected Dnn IE) are present, the selMode shall be related to the selected DNN. |  |
| backupAmfInfo | array(BackupAmfInfo) | C | 1..N | This IE shall be included if the NF service consumer is an AMF and the AMF supports the AMF management without UDSF for the following cases: - First interaction with SMF. - Modification of the BackupAmfInfo. |  |
| traceData | TraceData | C | 0..1 | This IE shall be included if trace is required to be activated (see 3GPP TS 32.422 [22]). |  |
| udmGroupId | NfGroupId | O | 0..1 | When present, it shall indicate the identity of the UDM group serving the UE. |  |
| routingIndicator | string | O | 0..1 | When present, it shall indicate the Routing Indicator of the UE. |  |
| hNwPubKeyId | integer | O | 0..1 | When present, it shall indicate the Home Network Public Key Identifier of the UE. (NOTE 3) |  |
| epsInterworkingInd | EpsInterworkingIndication | O | 0..1 | The AMF may provide the indication when a PGW-C+SMF is selected to serve the PDU Session. When present, this IE shall indicate whether the PDU session may possibly be moved to EPS and whether N26 interface to be used during EPS interworking procedures. The AMF may derive the value of the indication from different sources, like UE 5GMM capabilities (e.g. "S1 mode supported"), UE subscription data (e.g. "Core Network Type Restriction to EPC" and "Interworking with EPS Indication" for the DNN) and configurations. |  |
| indirectForwardingFlag | boolean | C | 0..1 | The AMF shall include this indication during N26 based Handover procedure from EPS to 5GS (see 3GPP TS 23.502 [3], clause 4.11.1.2.2), to inform the SMF of the applicability or non-applicability of indirect data forwarding. When present, it shall be set as follows: - True: indirect data forwarding is applicable - False: indirect data forwarding is not applicable |  |
| directForwardingFlag | boolean | C | 0..1 | The AMF shall include this indication during N26 based Handover procedure from EPS to 5GS (see 3GPP TS 23.502 [3], clause 4.11.1.2.2), to inform the SMF of the applicability or non-applicability of direct data forwarding. When present, it shall be set as follows: - True: direct data forwarding is applicable - False: direct data forwarding is not applicable |  |
| targetId | NgRanTargetId | C | 0..1 | This IE shall be present in the following cases: - during an EPS to 5GS handover preparation using the N26 interface, when the hoState IE is set to the value "PREPARING"; - during N2 based handover procedure with I-SMF or V-SMF insertion/change/removal, when hostate IE is set to the value "PREPARING". When present, it shall contain the Target ID identifying the target RAN Node ID and TAI. In case of EPS to 5GS handover, the TAI is received in the Forward Relocation Request from the Source MME. |  |
| epsBearerCtxStatus | EpsBearerContextStatus | C | 0..1 | This IE shall be present during an EPS to 5GS idle mode mobility using the N26 interface, if received in the Registration Request from the UE. When present, it shall be set to the value received from the UE. |  |
| cpCiotEnabled | boolean | C | 0..1 | This IE shall be present with the value "True", if - the NF service consumer (e.g. the AMF) has verified that the CIOT feature is supported by the SMF (and for a home-routed session, that it is also supported by the H-SMF); and - Control Plane CIoT 5GS Optimisation is enabled for the PDU session (see 3GPP TS 23.502 [3], clauses 4.3.2.2.1 and 4.3.2.2.2). When present, it shall be set as follows: - True: Control Plane CIoT 5GS Optimisation is enabled. - False (default): Control Plane CIoT 5GS Optimisation is not enabled. | CIOT |
| cpOnlyInd | boolean | C | 0..1 | This IE shall be present with the value "True", if the PDU session shall only use Control Plane CIoT 5GS Optimisation (see clause 5.31.4.1 of 3GPP TS 23.501 [2]). When present, it shall be set as follows: - True: the PDU session shall only use Control Plane CIoT 5GS Optimisation - False (default): the PDU session is not constrained to only use Control Plane CIoT 5GS Optimisation. | CIOT |
| invokeNef | boolean | C | 0..1 | This IE shall be present with the value "True", if Control Plane CIoT 5GS Optimisation is enabled and data delivery via NEF is selected for the PDU session (see 3GPP TS 23.502 [3], clause 4.3.2.2.2). When present, it shall be set as follows: - True: Data delivery via NEF is selected. - False (default): Data delivery via NEF is not selected. | CIOT |
| maRequestInd | boolean | C | 0..1 | This IE shall be present if a MA-PDU session is requested to be established. When present, it shall be set as follows: - True: a MA-PDU session is requested - False (default): a MA-PDU session is not requested | MAPDU |
| maNwUpgradeInd | boolean | C | 0..1 | This IE shall only be present if the PDU session is allowed to be upgraded to MA PDU session (see clause 4.22.3 of 3GPP TS 23.502 [3]). When present, it shall be set as follows: - True: the PDU session is allowed to be upgraded to MA PDU session - False (default): the PDU session is not allowed to be upgraded to MA PDU session When maRequestInd is present and set to "true", this IE shall not be present. | MAPDU |
| n2SmInfo | RefToBinaryData | C | 0..1 | This IE shall be present if N2 SM Information needs to be sent to the I-SMF. | DTSSA |
| n2SmInfoType | N2SmInfoType | C | 0..1 | This IE shall be present if "n2SmInfo" attribute is present. When present, this IE shall indicate the NG AP IE type for the NG AP SMF related IE container carried in "n2SmInfo" attribute. | DTSSA |
| n2SmInfoExt1 | RefToBinaryData | C | 0..1 | This IE shall be present if more than one N2 SM Information has been received from the AN. When present, this IE shall reference the N2 SM Information binary data (see clause 6.1.6.4.3). | DTSSA |
| n2SmInfoTypeExt1 | N2SmInfoType | C | 0..1 | This IE shall be present if "n2SmInfoExt1" attribute is present. When present, this IE shall indicate the NG AP IE type for the NG AP SMF related IE container carried in "n2SmInfoExt1" attribute. | DTSSA |
| smContextRef | Uri | C | 0..1 | This IE shall be present during an I-SMF or V-SMF insertion if available and during an I-SMF or V-SMF change or removal. When present, this IE shall contain the URI of the SM Context resource in the SMF or of the SM context resource in the source I-SMF or V-SMF during an I-SMF or V-SMF insertion or during an I-SMF or V-SMF change/removal respectively. The URI shall be an absolute URI, including apiRoot (see clause 6.1.3.3.2). (NOTE 6) | DTSSA |
| smContextSmfPlmnId | PlmnIdNid | C | 0..1 | This IE shall be present during an inter-PLMN mobility procedure if the smContextRef IE is present. It may be present otherwise, if the smContextRef IE is present. When present, this IE shall carry the PLMN ID of the SMF which hosts the SM Context resource identified by smContextRef IE. For an SNPN, the NID together with the PLMN ID shall identify the SNPN. (NOTE 7) | DTSSA |
| smContextSmfId | NfInstanceId | O | 0..1 | This IE may be present if smContextRef is present. When present, this IE shall carry the NF instance ID of the SMF which hosts the SM Context resource identified by smContextRef IE. (NOTE 2) | DTSSA |
| smContextSmfSetId | NfSetId | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the NF Set ID of the old V-SMF or the old I-SMF or the SMF as identified by the smContextSmfId. | DTSSA |
| smContextSmfServiceSetId | NfServiceSetId | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the NF Service Set ID of the PDUSession service instance (for this SmContext) in the old V-SMF or the old I-SMF or the SMF. | DTSSA |
| smContextSmfBinding | SbiBindingLevel | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the SBI binding level of the SM context resource. | DTSSA |
| upCnxState | UpCnxState | C | 0..1 | This IE shall be present to request the activation of the user plane connection of the PDU session, in the following cases: - during Service Request procedure with an I-SMF insertion / change / removal, or with a V-SMF change (see clause 5.2.2.2.6); - during Registration procedure with an I-SMF insertion / change / removal, or with a V-SMF insertion / change / removal (see clause 5.2.2.2.7), if this PDU session is requested to be activated by the UE. | DTSSA |
| smallDataRateStatus | SmallDataRateStatus | C | 0..1 | This IE shall be present if the small data rate control status is available in AMF, see clause 5.31.14.3 of 3GPP TS 23.501 [2] and clause 4.3.2.2.1 of 3GPP TS 23.502 [3]. | CIOT |
| apnRateStatus | ApnRateStatus | C | 0..1 | This IE shall be present if the APN rate control status is available in AMF, see clause 4.7.7.3 in 3GPP TS 23.401 [33] and clause 5.2.8.2.5 in 3GPP TS 23.502 [3]. | CIOT |
| extendedNasSmTimerInd | boolean | C | 0..1 | This IE shall be present with the value "True" if the UE supports CE mode B and use of CE mode B is not restricted according to the Enhanced Coverage Restriction information in the UE context in the AMF. When present, it shall indicate whether extended NAS SM timers shall be used for the UE as specified in 3GPP TS 24.501 [7], as follows: - True: extended NAS SM timers shall be used - False (default): normal NAS SM timers shall be used. | CIOT |
| dlDataWaitingInd | boolean | C | 0..1 | This IE shall be present during an EPS to 5GS Idle mode mobility using N26 interface with data forwarding (see clause 4.11.1.3.3A of 3GPP TS 23.502 [3]), if the same indication is received from the MME in the Context Response message. When present, it shall be set as follows: - true: DL data needs to be sent to the UE; - false (default): no DL data needs to be sent to the UE. | CIOT |
| ddnFailureSubs | DdnFailureSubs | C | 0..1 | This IE shall be present to subscribe the notification of the DDN Failure if the Availability after DDN failure event is subscribed by the UDM, see clause 4.15.3.2.7 of 3GPP TS 23.502 [3]. | CIOT |
| smfTransferInd | boolean | C | 0..1 | This IE shall be present during an SMF Context Transfer procedure, LBO or no Roaming, no I-SMF. When present, it shall be set as follows: - True: SMF Context Transfer - False (default): Not an SMF Context Transfer | CTXTR |
| oldSmfId | NfInstanceId | C | 0..1 | This IE shall be present if smfTransferInd is set to true. When present, it shall indicate old SMF instance identifier. | CTXTR |
| oldSmContextRef | Uri | C | 0..1 | This IE shall be present if smfTransferInd is set to true. When present, this IE shall contain the identifier of the SM Context resource in the old SMF. This IE shall also be present, without smfTransferInd set, if this information was received earlier in Notify SM Context Status, see clause 4.3.5.2 of 3GPP TS 23.502 [3]. | CTXTR EnEDGE |
| wAgfInfo | WAgfInfo | C | 0..1 | This IE shall be present, if received from the W-AGF. When present, it shall contain information about the N3 terminations of the W-AGF. The SMF may use this information when selecting the UPF. |  |
| tngfInfo | tngfInfo | C | 0..1 | This IE shall be present, if received from the TNGF. When present, it shall contain information about the N3 terminations of the TNGF. The SMF may use this information when selecting the UPF. |  |
| twifInfo | twifInfo | C | 0..1 | This IE shall be present, if received from the TWIF. When present, it shall contain information about the N3 terminations of the TWIF. The SMF may use this information when selecting the UPF. |  |
| ranUnchangedInd | boolean | C | 0..1 | This IE shall be present if the NG-RAN is not changed in case of I-SMF/V-SMF change or insertion during CM-CONNECTED registration procedure after EPS to 5GS handover (see clause 5.2.2.2.7) or I-SMF selection per DNAI (see clause 5.2.2.2.12). When present, it shall be set as follows: - true: NG-RAN is not changed; - false: NG-RAN is changed. | DTSSA |
| samePcfSelectionInd | boolean | C | 0..1 | This IE shall be present, if the AMF received the PCF Selection Assistance info from the UDM indicating that the same PCF is required. (NOTE 4) When present, it shall be set as follows: - True: the SMF is indicated to select the same PCF instance for the PDU session. - False (default): the SMF is not indicated to select the same PCF instance for the PDU session. |  |
| targetDnai | Dnai | C | 0..1 | This IE shall be present, if this information was received earlier in Notify SM Context Status. I-SMF or SMF shall use this information for I-UPF / L-PSA / PSA selection, see clauses 4.3.5.1, 4.3.5.2 or 4.23.5.4 of 3GPP TS 23.502 [3]. | EnEDGE |
| nrfManagementUri | Uri | C | 0..1 | If included, this IE shall contain the API URI of the NFManagement Service (see clause 6.1.1 of 3GPP TS 29.510 [19]) of the NRF or hNRF. It shall be present during the PDU session establishment procedure with an I-SMF/V-SMF or mobility procedure with I-SMF/V-SMF insertion/change, if it is returned from the NSSF or hNSSF (see clause 6.1.6.2.7 of 3GPP TS 29.531 [40]). |  |
| nrfDiscoveryUri | Uri | C | 0..1 | If included, this IE shall contain the API URI of the NFDiscovery Service (see clause 6.2.1 of 3GPP TS 29.510 [19]) of the NRF or hNRF. It shall be present during the PDU session establishment procedure with an I-SMF/V-SMF or mobility procedure with I-SMF/V-SMF insertion/change, if it is returned from the NSSF or hNSSF (see clause 6.1.6.2.7 of 3GPP TS 29.531 [40]). |  |
| nrfAccessTokenUri | Uri | C | 0..1 | If included, this IE shall contain the API URI of the Access Token Service (see clause 6.3.2 of 3GPP TS 29.510 [19]) of the NRF or hNRF. It shall be present during the PDU session establishment procedure with an I-SMF/V-SMF or mobility procedure with I-SMF/V-SMF insertion/change, if it is returned from the NSSF or hNSSF (see clause 6.1.6.2.7 of 3GPP TS 29.531 [40]). |  |
| nrfOauth2Required | map(boolean) | C | 1..N | This IE should be present if the nrfUri IE, nrfManagementUri IE or nrfDiscoveryUri IE is present and if the information is available. When present, this IE shall indicate whether the NRF requires Oauth2-based authorization for accessing its services. The key of the map shall be the name of an NRF service, e.g. "nnrf-nfm" or "nnrf-disc". The value of each entry of the map shall be encoded as follows: - true: OAuth2 based authorization is required. - false: OAuth2 based authorization is not required. The absence of this IE means that no indication is available about the usage of Oauth2 for authorization of NRF services. |  |
| smfBindingInfo | string | C | 0..1 | This IE shall be present, if available. When present, this IE shall contain the Binding indications of the SM context resource and shall be set to the value of the 3gpp-Sbi-Binding header defined in clause 5.2.3.2.6 of 3GPP TS 29.500 [4], without the header name. | DTSSA |
| pvsInfo | array(ServerAddressingInfo) | C | 1..N | This IE shall be present, if the AMF received this information from AUSF during User Plane Remote Provisioning of UEs procedure (see clause 5.30.2.10.4 of 3GPP TS 23.501 [40]). When present, this IE shall contain the remote Provisioning Server(s) information. | ENPN |
| onboardingInd | boolean | C | 0..1 | This IE shall be present, if the UE is registered for onboarding in an SNPN (see clause 5.30.2.10.4 in 3GPP TS 23.501 [40] and clause 4.2.2.2.4 in 3GPP TS 23.502 [3]). - false (default): The UE is not registered in an SNPN for the purpose of onboarding; - true: The UE has registered in the SNPN for the purpose of onboarding. | ENPN |
| oldPduSessionRef | Uri | C | 0..1 | This IE shall be present if this information was received earlier in Notify SM Context Status, see clause 4.3.5.2 of 3GPP TS 23.502 [3]. When present, this IE shall contain the URI of the PDU session resource in the old SMF. The URI shall be an absolute URI, including apiRoot (see clause 6.1.3.6.2). | EnEDGE |
| smPolicyNotifyInd | boolean | O | 0..1 | When present, this IE shall indicate whether the SM Policy Association Establishment and Termination events shall be reported for the PDU session by the PCF for the SM Policy to the PCF for the UE: - true: SM Policy Association Establishment and Termination events shall be reported - false (default): SM Policy Association Establishment and Termination events is not required (NOTE 5) | SPAE |
| pcfUeCallbackInfo | PcfUeCallbackInfo | C | 0..1 | This IE shall be present when the smPolicyNotifyInd IE is present with value true. When present, this IE shall contain the callback information of the PCF for the UE to receive SM Policy Association Establishment and Termination events notification from the PCF for the SM Policy. (NOTE 5) | SPAE |
| satelliteBackhaulCat | SatelliteBackhaulCategory | O | 0..1 | This IE may be present if the AMF supports the 5GSAT feature and the AMF is aware that there is a satellite backhaul towards the 5G AN serving the UE. When present, this IE shall indicate the category of the satellite backhaul used towards the 5G AN serving the UE. | 5GSAT |
| upipSupported | boolean | C | 0..1 | This IE shall be present during the PDU session establishment procedure, if the UE supports User Plane Integrity Protection with EPS and if the AMF supports the related functionality. It may be present otherwise. When present, this IE shall be set as follows: - true: User Plane Integrity Protection with EPS is supported; - false (default): User Plane Integrity Protection with EPS is not supported. | UPIPE |
| uavAuthenticated | boolean | C | 0..1 | This IE shall be present during the PDU session establishment procedure, if the UAV has been authenticated by the USS for the requested DNN subject to aerial services. It may be present otherwise. When present, this IE shall be set as follows: - true: UAV has been authenticated by the USS; - false: UAV has not been authenticated by the USS. If both the requested DNN (i.e. dnn IE) and selected DNN (i.e. selected Dnn IE) are present, the uavAuthenticated shall be related to the selected DNN. |  |
| disasterRoamingInd | boolean | O | 0..1 | This IE may be set during the PDU session establishment. When present, this IE shall be set as follows: - true: the UE is registered for Disaster Roaming service - false (default): the UE is not registered for Disaster Roaming service |  |
| anchorSmfOauth2Required | boolean | O | 0..1 | When present, this IE shall indicate whether the H-SMF or SMF for a PDU session with an I-SMF requires Oauth2-based authorization for accessing its Nsmf_PDUSession service. - true: OAuth2 based authorization is required. - false: OAuth2 based authorization is not required. The absence of this IE means that no indication is available about the usage of Oauth2 for authorization of the anchor SMF's Nsmf_PDUSession service. (NOTE 2) |  |
| smContextSmfOauth2Required | boolean | O | 0..1 | This IE may be present during an I-SMF insertion, change or removal. This IE may be present when V-SMF changes within the same PLMN. When present, this IE shall indicate whether the SMF hosting the SM Context requires Oauth2-based authorization for accessing its Nsmf_PDUSession service. - true: OAuth2 based authorization is required. - false: OAuth2 based authorization is not required. The absence of this IE means that no indication is available about the usage of Oauth2 for authorization of the Nsmf_PDUSession service on the SMF hosting the SM Context. (NOTE 2) |  |
| hrsboAllowedInd | boolean | O | 0..1 | This IE may be present in HR roaming scenarios. When present, this IE shall Indicate whether Session Breakout for HR Session using ULCL/BP in VPLMN is allowed: - true: Allowed. - false (default): Not allowed. | HR-SBO |
| NOTE 1: In shared networks, when the message is sent from the VPLMN to the HPLMN, the PLMN ID that is communicated in this IE shall be that of the selected Core Network Operator. In shared networks, when the AMF and SMF pertain to the same PLMN, the Primary PLMN ID shall be communicated in the ECGI or NCGI to the SMF. The Core Network Operator PLMN ID shall be communicated in the TAI and the Serving Network. NOTE 2: If the SMF is aware that Oauth is enabled for the indicated next hop SMF, e.g. when the anchorSmfOauth2Required IE and/or the smContextSmfOauth2Required IE are received with the value true or when received a "401 Unauthorized" response code from next hop SMF, the SMF shall use the NF instance Identifier to acquire the access token for the Nsmf_PduSession service on the indicated SMF. NOTE 3: If present, this attribute shall be used together with routingIndicator. This attribute is only used by the HPLMN in roaming scenarios. NOTE 4: If present, this attribute shall be used together with the PCF ID received from the AMF for selecting the same PCF instance for the PDU session. NOTE 5: If the AMF has received the callback information of the PCF for the UE together with the information of the PDU sessions (i.e. Slice and DNN combination) that are applicable for notification of SM Policy Association events, the AMF shall identify whether the non-roaming or local breakout PDU session to be created is applicable for SM Policy Association events, i.e, whether the slice and DNN combination of the PDU session is listed in the received PDU session information from the PCF for the UE. If the PDU session is applicable for notification of SM Policy Association events, the AMF shall include the smPolicyNotifyInd IE with the value "true" and the callback information of the PCF for the UE in the request. The SMF shall forward the callback information of the PCF for the UE to the PCF for SM Policy during SM Policy Association Establishment. See clause 4.3.2.2.1 of 3GPP TS 23.502 [3]. NOTE 6: See NOTE 2 of Table 6.1.6.2.3-1. NOTE 7: If the PLMN ID of the SMF holding the SM context received in smContextSmfPlmnId attribute is different from the PLMN ID of the target V-SMF/I-SMF/anchor SMF, the target V-SMF/I-SMF/anchor SMF shall retrieve the SM Context from the SMF via the SEPP. In this case, the smContextSmfPlmnId attribute may also be used for the discovery and selection of the local SEPP. NOTE 8: The smfUri and hSmfUri attributes need not be included in Create SM Context request in procedures other than PDU session establishment procedure and EPS to 5GS mobility procedures if the NF Service Consumer (e.g. AMF) and I-SMF/V-SMF support the "ACSCR" feature. See clause 6.1.8. |  |  |  |  |  |

* * * Next Change * * * *

6.1.6.2.9	Type: PduSessionCreateData

Table 6.1.6.2.9-1: Definition of type PduSessionCreateData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supi | Supi | C | 0..1 | This IE shall be present, except if the UE is emergency registered and UICCless. When present, it shall contain the subscriber permanent identify. |  |
| unauthenticatedSupi | boolean | C | 0..1 | This IE shall be present if the SUPI is present in the message but is not authenticated and is for an emergency registered UE. When present, it shall be set as follows: - true: unauthenticated SUPI; - false (default): authenticated SUPI. |  |
| pei | Pei | C | 0..1 | This IE shall be present if the UE is emergency registered and it is either UIClless or the SUPI is not authenticated. For all other cases, this IE shall be present if it is available. When present, it shall contain the permanent equipment identifier. |  |
| pduSessionId | PduSessionId | C | 0..1 | This IE shall contain the PDU Session ID, except during an EPS to 5GS Idle mode mobility or handover using the N26 interface. |  |
| dnn | Dnn | M | 1 | This IE shall contain the requested DNN. The DNN shall be the full DNN (i.e. with both the Network Identifier and Operator Identifier) for a HR PDU session, and it should be the full DNN in LBO and non-roaming scenarios. If the Operator Identifier is absent, the serving core network operator shall be assumed. |  |
| selectedDnn | Dnn | C | 0..1 | This IE shall be present, if another DNN other than the UE requested DNN is selected for this PDU session. When present, it shall contain the selected DNN. The DNN shall be the full DNN (i.e. with both the Network Identifier and Operator Identifier) for a HR PDU session, and it should be the full DNN in LBO and non-roaming scenarios. If the Operator Identifier is absent, the serving core network operator shall be assumed. |  |
| sNssai | Snssai | C | 0..1 | This IE shall be present, except during an EPS to 5GS idle mode mobility or handover using the N26 interface. When present, it shall contain: - the HPLMN S-NSSAI for a HR PDU session, which is mapped from the requested S-NSSAI by the VPLMN; or - the requested S-NSSAI in the serving PLMN for a PDU session with an I-SMF. |  |
| hplmnSnssai | Snssai | C | 0..1 | This IE shall be present for an LBO PDU session with an I-SMF, except during an EPS to 5GS idle mode mobility or handover using the N26 interface. When present, it shall contain the HPLMN S-NSSAI of the LBO PDU session, which is mapped from the requested S-NSSAI by the VPLMN. | DTSSA |
| vsmfId | NfInstanceId | C | 0..1 | This IE shall be present for a HR PDU session. When present, it shall contain the identifier of the V-SMF. |  |
| ismfId | NfInstanceId | C | 0..1 | This IE shall be present for a PDU session with an I-SMF. When present, it shall contain the identifier of the I-SMF. | DTSSA |
| servingNetwork | PlmnIdNid | M | 1 | This IE shall contain the serving core network operator PLMN ID and, for an SNPN, the NID that together with the PLMN ID identifies the SNPN. |  |
| requestType | RequestType | C | 0..1 | This IE shall be present if the Request type IE is received from the UE for a single access PDU session and if the request refers to an existing PDU session or an existing emergency PDU session. The requestType IE shall not be included for a MA-PDU session establishment request. It may be present otherwise. When present, it shall indicate whether the request refers to a new PDU session or emergency PDU session, or to an existing PDU session or emergency PDU session. For request sent from AMF, this IE shall be set based on the requestType received. |  |
| epsBearerId | array(EpsBearerId) | C | 1..N | This IE shall be present during an EPS to 5GS Idle mode mobility or handover preparation using the N26 interface. When present, it shall contain the list of EPS bearer Id(s) received from the MME. |  |
| pgwS8cFteid | Bytes | C | 0..1 | This IE shall be present during an EPS to 5GS Idle mode mobility or handover preparation using the N26 interface. When present, it shall contain Base64-encoded characters, encoding the PGW S8 F-TEID for Control Plane as specified in Figure 8.22-1 of 3GPP TS 29.274 [16] (starting from octet 1), received from the MME. |  |
| vsmfPduSessionUri | Uri | C | 0..1 | This IE shall be present for a HR PDU session. When present, it shall include the callback URI representing the PDU session in the V-SMF. |  |
| ismfPduSessionUri | Uri | C | 0..1 | This IE shall be present for a PDU session with an I-SMF. When present, it shall include the callback URI representing the PDU session in the I-SMF. | DTSSA |
| vcnTunnelInfo | TunnelInfo | C | 0..1 | This IE shall be present for a HR PDU session, except for EPS to 5GS handover using N26 interface and when Control Plane CIoT 5GS Optimisation is enabled and data delivery via NEF is selected for this PDU session. When present, this IE shall contain the N9 tunnel information of the visited CN side, i.e. V-UPF. |  |
| icnTunnelInfo | TunnelInfo | C | 0..1 | This IE shall be present for a PDU session involving an I-SMF, except when Control Plane CIoT 5GS Optimisation is enabled and data delivery via NEF is selected for this PDU session. When present, this IE shall contain the N9 tunnel information of the I-UPF controlled by the I-SMF. | DTSSA |
| n9ForwardingTunnelInfo | TunnelInfo | C | 0..1 | This IE shall be present during Service Request procedures with I-SMF insertion, if buffered DL data is available at the I-UPF that is controlled by the SMF (see clause 4.23.4 in 3GPP TS 23.502 [3]). When present, this IE shall contain the N9 tunnel information of the I-UPF controlled by the I-SMF. | DTSSA |
| additionalCnTunnelInfo | TunnelInfo | C | 0..1 | This IE shall be present if a MA PDU session is requested or if the PDU session is allowed to be upgraded to a MA PDU session, and the UE is registered over both 3GPP access and Non-3GPP access. When present, it shall contain additional N9 tunnel information of the UPF controlled by the V-SMF or I-SMF. | MAPDU |
| anType | AccessType | M | 1 | This IE shall indicate the Access Network Type to which the PDU session is to be associated. |  |
| additionalAnType | AccessType | C | 0..1 | This IE shall indicate the additional Access Network Type to which the PDU session is to be associated. This IE shall be present if a MA-PDU session is requested and the UE is registered over both 3GPP access and Non-3GPP access. | MAPDU |
| ratType | RatType | C | 0..1 | This IE shall be present and indicate the RAT Type used by the UE, if available. |  |
| ueLocation | UserLocation | C | 0..1 | This IE shall contain the UE location information (see clause 5.2.3.4), if it is available. (NOTE 1) |  |
| ueTimeZone | TimeZone | C | 0..1 | This IE shall contain the UE Time Zone, if it is available. |  |
| addUeLocation | UserLocation | O | 0..1 | Additional UE location. This IE may be present, if anType indicates a non-3GPP access and a valid 3GPP access user location information is available. When present, it shall contain: - the last known 3GPP access user location (see clause 5.2.3.4); and - the timestamp, if available, indicating the UTC time when the addUeLocation information was acquired. (NOTE 1) |  |
| gpsi | Gpsi | C | 0..1 | This IE shall be present if it is available. When present, it shall contain the user's GPSI. |  |
| n1SmInfoFromUe | RefToBinaryData | C | 0..1 | This IE shall be present if the V-SMF or I-SMF has received known N1 SM information from the UE that does not need to be interpreted by the V-SMF or I-SMF. When present, this IE shall reference the n1SmInfoFromUe binary data (see clause 6.1.6.4.4). |  |
| unknownN1SmInfo | RefToBinaryData | C | 0..1 | This IE shall be present if the V-SMF or I-SMF has received unknown N1 SM information from the UE. When present, this IE shall reference the unknownN1SmInfo binary data (see clause 6.1.6.4.4). |  |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one optional feature defined in clause 6.1.8 is supported. |  |
| hPcfId | NfInstanceId | O | 0..1 | This IE may be used by V-SMF to indicate the home PCF selected by the AMF for the UE to the H-SMF, for a HR PDU session. When present, this IE shall contain the identifier of the H-PCF selected by the AMF for the UE (for UE Policy Control). |  |
| pcfId | NfInstanceId | O | 0..1 | This IE may be used by I-SMF to indicate the (V-)PCF selected by the AMF for the UE to the SMF, for a PDU session with an I-SMF. When present, this IE shall contain the identifier of the PCF (for Access and Mobility Policy Control and/or UE Policy Control) in non-roaming scenarios, or the V-PCF (for Access and Mobility Policy Control) in LBO roaming scenarios. | DTSSA |
| pcfGroupId | NfGroupId | O | 0..1 | This IE may be present in non-roaming and HR roaming scenarios. When present, this IE shall contain the identity of the (home) PCF group serving the UE for Access and Mobility Policy and/or UE Policy. |  |
| pcfSetId | NfSetId | O | 0..1 | When present, it shall contain the NF Set ID of the H-PCF indicated by the hPcfId IE or the (V-)PCF indicated by the pcfId IE. |  |
| hoPreparationIndication | boolean | C | 0..1 | This IE shall be present during an EPS to 5GS handover preparation using the N26 interface or during N2 handover preparation with I-SMF insertion. When present, it shall be set as follows: - true: an EPS to 5GS handover preparation or N2 handover preparation with I-SMF is in progress; the PGW-C/SMF shall not switch the DL user plane of the PDU session yet. - false: there is no on-going EPS to 5GS handover preparation or N2 handover preparation with I-SMF in progress. If a handover preparation was in progress, the handover has been completed. The PGW-C/SMF shall switch the DL user plane of the PDU session using the N9 tunnel information that has been received in the vcnTunnelInfo or icnTunnelInfo. It shall be set to "true" during an EPS to 5GS handover preparation using the N26 interface or during N2 handover preparation with I-SMF insertion. |  |
| selMode | DnnSelectionMode | C | 0..1 | This IE shall be present if it is available. When present, it shall be set to: - "VERIFIED", if the requested DNN provided by UE or the selected DNN provided by the network corresponds to an explicitly subscribed DNN; or - "UE_DNN_NOT_VERIFIED", if the requested DNN provided by UE corresponds to the usage of a wildcard subscription; or - "NW_DNN_NOT_VERIFIED", if the selected DNN provided by the network corresponds to the usage of a wildcard subscription. If both the requested DNN (i.e. dnn IE) and selected DNN (i.e. selected Dnn IE) are present, the selMode shall be related to the selected DNN. |  |
| alwaysOnRequested | boolean | C | 0..1 | This IE shall be present and set to true if the UE requests to setup an always-on PDU session and this is allowed by local policy in the V-SMF or I-SMF. When present, it shall be set as follows: - true: request for an always-on PDU session - false (default): not a request for an always-on PDU session |  |
| udmGroupId | NfGroupId | O | 0..1 | When present, it shall indicate the identity of the UDM group serving the UE. |  |
| routingIndicator | string | O | 0..1 | When present, it shall indicate the Routing Indicator of the UE. |  |
| hNwPubKeyId | integer | O | 0..1 | When present, it shall indicate the Home Network Public Key Identifier of the UE. (NOTE 2) |  |
| epsInterworkingInd | EpsInterworkingIndication | O | 0..1 | This IE may be present if the indication has been received from AMF and is allowed to be forwarded to H-SMF by operator configuration. When present, this IE shall indicate whether the PDU session may possibly be moved to EPS and whether N26 interface to be used during EPS interworking procedures. |  |
| vSmfServiceInstanceId | string | O | 0..1 | When present, this IE shall contain the serviceInstanceId of the V-SMF service instance serving the PDU session. This IE may be used by the H-SMF to identify PDU sessions affected by a failure or restart of the V-SMF service (see clauses 6.2 and 6.3 of 3GPP TS 23.527 [24]). |  |
| iSmfServiceInstanceId | string | O | 0..1 | When present, this IE shall contain the serviceInstanceId of I-SMF service instance serving the PDU session. This IE may be used by the SMF to identify PDU sessions affected by a failure or restart of the I-SMF service (see clauses 6.2 and 6.3 of 3GPP TS 23.527 [24]). | DTSSA |
| recoveryTime | DateTime | O | 0..1 | Timestamp when the V-SMF or I-SMF service instance serving the PDU session was (re)started (see clause 6.3 of 3GPP TS 23.527 [24]). |  |
| roamingChargingProfile | RoamingChargingProfile | O | 0..1 | Roaming Charging Profile applicable in the VPLMN (see clauses 5.1.9.1, 5.2.1.7 and 5.2.2.12.2 of 3GPP TS 32.255 [25]). |  |
| chargingId | string | O | 0..1 | Charging ID (see clauses 5.1.9.1 of 3GPP TS 32.255 [25]). The string shall encode the Charging ID (32-bit unsigned integer value, with maximum value "4294967295") in decimal representation. Pattern: '^(0¦([1-9]{1}[0-9]{0,9}))$' (NOTE 4) |  |
| oldPduSessionId | PduSessionId | C | 0..1 | This IE shall be present if this information is received from the UE and the same SMF is selected for SSC mode 3. When present, it shall contain the old PDU Session ID received from the UE. See clauses 4.3.2.2.1 and 4.3.5.2 of 3GPP TS 23.502 [3]. |  |
| epsBearerCtxStatus | EpsBearerContextStatus | C | 0..1 | This IE shall be present during an EPS to 5GS idle mode mobility using the N26 interface, if received in the Create SM Context request. When present, it shall be set to the value received in the Create SM Context request. |  |
| amfNfId | NfInstanceId | C | 0..1 | This IE shall be present if it is received in the Create SM Context request, unless the PDU session is related to regulatory prioritized service. When present, it shall contain the identifier of the serving AMF. |  |
| guami | Guami | C | 0..1 | This IE shall be present if the amfNfId is present. When present, it shall contain the serving AMF's GUAMI. |  |
| maxIntegrityProtectedDataRateUl | MaxIntegrityProtectedDataRate | C | 0..1 | This IE shall be present if it is available. When present, it shall indicate the maximum integrity protected data rate supported by the UE for uplink. |  |
| maxIntegrityProtectedDataRateDl | MaxIntegrityProtectedDataRate | C | 0..1 | This IE shall be present if it is available. When present, it shall indicate the maximum integrity protected data rate supported by the UE for downlink. |  |
| cpCiotEnabled | boolean | C | 0..1 | This IE shall be present with the value "True" if the "5gCiotCpEnabled" attribute is received with "True" value in SM Context Create request, indicating the Control Plane CIoT 5GS Optimisation is enabled for the PDU session (see 3GPP TS 23.502 [3], clause 4.3.2.2.2). When present, it shall be set as follows: - True: Control Plane CIoT 5GS Optimisation is enabled. - False (default): Control Plane CIoT 5GS Optimisation is not enabled. | CIOT |
| cpOnlyInd | boolean | C | 0..1 | This IE shall be present with the value "True", if the PDU session shall only use Control Plane CIoT 5GS Optimisation (see clause 5.31.4.1 of 3GPP TS 23.501 [2]). When present, it shall be set as follows: - True: the PDU session shall only use Control Plane CIoT 5GS Optimisation - False (default): the PDU session is not constrained to only use Control Plane CIoT 5GS Optimisation. | CIOT |
| invokeNef | boolean | C | 0..1 | This IE shall be present with value "True", if Control Plane CIoT 5GS Optimisation is enabled and data delivery via NEF is selected for the PDU session. When present, it shall be set as follows: - True: Data delivery via NEF is selected. - False (default): Data delivery via NEF is not selected. | CIOT |
| maRequestInd | boolean | C | 0..1 | This IE shall be present if a MA-PDU session is requested to be established by the UE. When present, it shall be set as follows: - True: a MA-PDU session is requested - False (default): a MA-PDU session is not requested | MAPDU |
| maNwUpgradeInd | boolean | C | 0..1 | This IE shall only be present if the PDU session is allowed to be upgraded to MA PDU session (see clause 4.22.3 of 3GPP TS 23.502 [3]). When present, it shall be set as follows: - True: the PDU session is allowed to be upgraded to MA PDU session - False (default): the PDU session is not allowed to be upgraded to MA PDU session When maRequestInd is present and set to "true", this IE shall not be present. | MAPDU |
| dnaiList | array(Dnai) | C | 1..N | This IE shall be present over N16a if an I-SMF is inserted into a PDU session during the following procedures: PDU session establishment, Registration, Service Request, Xn based handover, Inter NG-RAN node N2 based handover (see clause 4.23 of 3GPP TS 23.502 [3]). When present, it shall include the list of DNAIs supported by the I-SMF. | DTSSA |
| presenceInLadn | PresenceState | C | 0..1 | This IE shall be present during Xn based handover with I-SMF insertion, if the DNN corresponds to a LADN. When present, it shall be set to "IN" or "OUT" to indicate that the UE is in or out of the LADN service area. | DTSSA |
| secondaryRatUsageInfo | array(SecondaryRatUsageInfo) | O | 1..N | This IE may be present to report usage data for a secondary RAT for QoS flows and/or the whole PDU session. | DTSSA |
| smallDataRateStatus | SmallDataRateStatus | C | 0..1 | This IE shall be present if the small data rate control status is received from AMF, see clause 5.31.14.3 of 3GPP TS 23.501 [2] and clause 4.3.2.2.2 of 3GPP TS 23.502 [3]. | CIOT |
| apnRateStatus | ApnRateStatus | C | 0..1 | This IE shall be present, if the APN rate control status (APN rates are shared by all PDN connections of the UE to this APN) is received from the AMF, see clause 4.7.7.3 in 3GPP TS 23.401 [33] and clause 4.11.5.3 in 3GPP TS 23.502 [3]. | CIOT |
| dlServingPlmnRateCtl | integer | C | 0..1 | This IE shall be present if Serving PLMN Rate Control for downlink data packets is enabled in the PLMN and Control Plane CIoT 5GS Optimisation is enabled for the PDU session. When present, this IE shall contain the maximum allowed number of Downlink NAS Data PDUs per deci hour of the serving PLMN, as specified in clause 5.31.14.2 of 3GPP TS 23.501 [2]. Minimum: 10 | CIOT |
| upSecurityInfo | UpSecurityInfo | C | 0..1 | This IE shall be present if received from NG-RAN during Xn handover procedure with I-SMF Insertion (see clause 5.2.2.7.5). When present, this IE shall contain the User Plane Security Information associated to the PDU session. See clause 9.3.1.60 of 3GPP TS 38.413 [9]. | DTSSA |
| vplmnQos | VplmnQos | C | 0..1 | This IE shall be present for a HR PDU session, if the V-SMF supports the VQOS feature and if VPLMN QoS constraints are required for the PDU session. When present, this IE shall contain the QoS constraints from the VPLMN. | VQOS |
| oldSmContextRef | Uri | C | 0..1 | This IE shall be present, if it is received in the Create SM Context request. When present, this IE shall contain the identifier of the SM Context resource in the old SMF. | EnEDGE |
| redundantPduSessionInfo | RedundantPduSessionInformation | C | 0..1 | This IE shall be present for a PDU session with an I-SMF, if an RSN and/or PDU Session Pair ID was received from the UE. | DCE2ER |
| oldPduSessionRef | Uri | C | 0..1 | This IE shall be present, if it is received in the Create SM Context request. When present, this IE shall contain the URI of the PDU session resource in the old SMF. The URI shall be an absolute URI, including apiRoot (see clause 6.1.3.6.2). | EnEDGE |
| smPolicyNotifyInd | boolean | C | 0..1 | This IE shall be included by I-SMF to SMF, if received from AMF. When present, this IE shall indicate whether the SM Policy Association Establishment and Termination events shall be reported for the PDU session by the PCF for the SM Policy to the PCF for the UE: - true: SM Policy Association Establishment and Termination events shall be reported - false (default): SM Policy Association Establishment and Termination events is not required (NOTE 3) | SPAE |
| pcfUeCallbackInfo | PcfUeCallbackInfo | C | 0..1 | This IE shall be present when the smPolicyNotifyInd IE is present with value true. When present, this IE shall contain the callback information of the PCF for the UE to receive SM Policy Association Establishment and Termination events notification from the PCF for the SM Policy. (NOTE 3) | SPAE |
| satelliteBackhaulCat | SatelliteBackhaulCategory | O | 0..1 | This IE may be present if the V-SMF/I-SMF supports the 5GSAT feature and the satelliteBackhaulCat IE has been received from the AMF. When present, this IE shall indicate the value received from the AMF. | 5GSAT |
| upipSupported | boolean | C | 0..1 | This IE shall be present during the PDU session establishment procedure if the UE supports User Plane Integrity Protection with EPS and if the AMF supports the related functionality. It may be present otherwise. When present, this IE shall be set as follows: - true: User Plane Integrity Protection with EPS is supported; - false (default): User Plane Integrity Protection with EPS is not supported. | UPIPE |
| upCnxState | UpCnxState | C | 0..1 | This IE shall be present to indicate that the User Plane resource for the PDU session is going to be established by the I-SMF/V-SMF, during a service request procedure with I-SMF/V-SMF insertion (see clause 4.23.4.3 of 3GPP TS 23.502 [3]). When present, this IE shall be set as specified in clause 5.2.2.7.6. |  |
| disasterRoamingInd | boolean | O | 0..1 | This IE may be set during the PDU session establishment. When present, this IE shall be set as follows: - true: the UE is registered for Disaster Roaming service - false (default): the UE is not registered for Disaster Roaming service |  |
| hrsboReqInfo | HrsboReqInfo | C | 0..1 | This IE shall be present in HR roaming scenarios if the V-SMF requests HR SBO authorization. When present, this IE shall Include the information for HR-SBO. | HR-SBO |
| NOTE 1: In shared networks, the PLMN ID that is communicated in this IE shall be that of the selected Core Network Operator. NOTE 2: If present, this attribute shall be used together with routingIndicator. This attribute is only used by the HPLMN in roaming scenarios. NOTE 3: If the smPolicyNotifyInd IE is received with the value "true", the SMF shall forward the callback information of the PCF for the UE to the PCF for SM Policy during SM Policy Association Establishment. See clause 4.3.2.2.1 of 3GPP TS 23.502 [3]. NOTE 4: Usage of Charging ID with Uint32 value for roaming scenarios may lead to Charging ID collision between SMFs. |  |  |  |  |  |

* * * Next Change * * * *

6.1.6.2.10	Type: PduSessionCreatedData

Table 6.1.6.2.10-1: Definition of type PduSessionCreatedData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| pduSessionType | PduSessionType | M | 1 | This IE shall indicate the selected PDU type. |  |
| sscMode | string | M | 1 | This IE shall indicate the SSC mode applicable to the PDU session. When present, it shall be encoded as one character in hexadecimal representation, taking a value of "0" to "7", representing the 3 bits of the SSC mode value of the SSC mode IE specified in clause 9.11.4.16 of 3GPP TS 24.501 [7]. Pattern: "^[0-7]$" Example: SSC mode 3 shall be encoded as "3". (NOTE 1). |  |
| hcnTunnelInfo | TunnelInfo | C | 0..1 | This IE shall be present for a HR PDU session, except when Control Plane CIoT 5GS Optimisation is enabled and data delivery via NEF is selected for this PDU session. When present, this IE shall contain the N9 tunnel information of the home CN side, i.e. H-UPF. |  |
| cnTunnelInfo | TunnelInfo | C | 0..1 | This IE shall be present for a PDU session involving an I-SMF, except when Control Plane CIoT 5GS Optimisation is enabled and data delivery via NEF is selected for this PDU session. When present, this IE shall contain the N9 tunnel information of the PSA UPF. | DTSSA |
| additionalCnTunnelInfo | TunnelInfo | C | 0..1 | This IE shall be present if a MA-PDU session is established for a UE registered over both 3GPP access and Non-3GPP access. When present, it shall contain additional N9 tunnel information of the UPF controlled by the H-SMF or SMF. | MAPDU |
| sessionAmbr | Ambr | C | 0..1 | This IE shall be present, except when Control Plane CIoT 5GS Optimisation is enabled for the PDU session. When present, this IE shall contain the Session AMBR granted to the PDU session. |  |
| qosFlowsSetupList | array(QosFlowSetupItem) | C | 1..N | This IE shall be present, except when Control Plane CIoT 5GS Optimisation is enabled for the PDU session. When present, this IE shall contain the full set of QoS flow(s) to establish for the PDU session. It shall contain at least the Qos flow associated to the default Qos rule. In V-SMF/I-SMF insertion scenarios where no QoS Rule(s) associated to a QoS flow can or need to be sent to the UE, the qosRules attribute of the QosFlowSetupItem may be set to an empty string or to the latest QoS Rule(s) associated to the QoS flow. (NOTE 3) |  |
| hSmfInstanceId | NfInstanceId | C | 0..1 | This IE shall be present for a HR PDU session. When present, it shall contain the identifier of the home SMF. |  |
| smfInstanceId | NfInstanceId | C | 0..1 | This IE shall be present for a PDU session with an I-SMF. When present, it shall contain the identifier of the SMF. | DTSSA |
| pduSessionId | PduSessionId | C | 0..1 | This IE shall be present during an EPS to 5GS Idle mode mobility or handover preparation using the N26 interface. When present, it shall be set to the PDU Session ID. |  |
| sNssai | Snssai | C | 0..1 | This IE shall be present during an EPS to 5GS Idle mode mobility or handover using the N26 interface. When present, it shall contain: - the S-NSSAI assigned to the PDU session in the Home PLMN, for a HR PDU session; - the S-NSSAI assigned to the PDU session in the serving PLMN, for a PDU session with an I-SMF. The Snssai shall overwrite the S-NSSAI earlier stored in I-SMF, if they are different. |  |
| enablePauseCharging | boolean | C | 0..1 | This IE shall be present, based on operator's policy, to enable the use of Pause of Charging for the PDU session (see clause 4.4.4 of 3GPP TS 23.502 [3]). When present, it shall be set as follows: - true: enable Pause of Charging; - false (default): disable Pause of Charging. |  |
| ueIpv4Address | Ipv4Addr | C | 0..1 | This IE shall be present if the SMF assigns a UE IPv4 address to the PDU session. |  |
| ueIpv6Prefix | Ipv6Prefix | C | 0..1 | This IE shall be present if the SMF assigns a UE IPv6 prefix to the PDU session. |  |
| n1SmInfoToUe | RefToBinaryData | C | 0..1 | This IE shall be present if the SMF needs to send N1 SM information to the UE that does not need to be interpreted by the V-SMF or I-SMF. When present, this IE shall reference the n1SmInfoToUe binary data (see clause 6.1.6.4.4). |  |
| epsPdnCnxInfo | EpsPdnCnxInfo | C | 0..1 | This IE shall be present if the PDU session may be moved to EPS during its lifetime. |  |
| epsBearerInfo | array(EpsBearerInfo) | C | 1..N | This IE shall be present if the PDU session may be moved to EPS during its lifetime. |  |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one optional feature defined in clause 6.1.8 is supported. |  |
| maxIntegrityProtectedDataRate | MaxIntegrityProtectedDataRate | C | 0..1 | This IE shall be present if the upSecurity IE is present and indicates that integrity protection is preferred or required. When present, it shall indicate the maximum integrity protected data rate for uplink. If the maxIntegrityProtectedDataRateDl IE is absent, this IE applies to both uplink and downlink. (NOTE 6) |  |
| maxIntegrityProtectedDataRateDl | MaxIntegrityProtectedDataRate | C | 0..1 | This IE may be present if the upSecurity IE is present and indicates that integrity protection is preferred or required. When present, it shall indicate the maximum integrity protected data rate for downlink. (NOTE 6) |  |
| alwaysOnGranted | boolean | C | 0..1 | This IE shall be present if the alwaysOnRequested IE was received in the request or if the SMF determines, based on local policy, that the PDU session needs to be established as an always-on PDU session. When present, it shall be set as follows: - true: always-on PDU session granted. - false (default): always-on PDU session not granted. |  |
| gpsi | Gpsi | C | 0..1 | This IE shall be present if no GPSI IE is provided in the request, e.g. for a PDU session moved from another access or another system, and the SMF knows that a GPSI is already associated with the PDU session. When present, it shall contain the user's GPSI associated with the PDU session. |  |
| upSecurity | UpSecurity | O | 0..1 | When present, this IE shall indicate the security policy for integrity protection and encryption for the user plane of the PDU session. If this IE is present, it shall not indicate that integrity protection is preferred or required, if the maxIntegrityProtectedDataRate IE is not present (e.g. if UE Integrity Protection Maximum Data Rate is not available in the SMF). (NOTE 6) |  |
| roamingChargingProfile | RoamingChargingProfile | O | 0..1 | Roaming Charging Profile selected by the HPLMN (see clauses 5.1.9.1, 5.2.1.7 and 5.2.2.12.2 of 3GPP TS 32.255 [25]). |  |
| hSmfServiceInstanceId | string | O | 0..1 | When present, this IE shall contain the serviceInstanceId of the H-SMF service instance serving the PDU session, for a HR PDU session. This IE may be used by the V-SMF to identify PDU sessions affected by a failure or restart of the H-SMF service (see clause 6.2 of 3GPP TS 23.527 [24]). |  |
| smfServiceInstanceId | string | O | 0..1 | When present, this IE shall contain the serviceInstanceId of the SMF service instance serving the PDU session, for a PDU session with an I-SMF. This IE may be used by the I-SMF to identify PDU sessions affected by a failure or restart of the SMF service (see clause 6.2 of 3GPP TS 23.527 [24]). | DTSSA |
| recoveryTime | DateTime | O | 0..1 | Timestamp when the SMF service instance serving the PDU session was (re)started (see clause 6.3 of 3GPP TS 23.527 [24]). |  |
| dnaiList | array(Dnai) | C | 1..N | This IE shall be present over N16a, if available and an I-SMF has been inserted into a PDU session, during the following procedures: PDU session establishment, Registration, Service Request, Xn based handover, Inter NG-RAN node N2 based handover (see clause 4.23 of 3GPP TS 23.502 [3]). When present, it shall include the list of DNAIs of interest for the PDU session for local traffic steering at the I-SMF. If the I-SMF and the SMF support the DTSSA-Ext1 feature, when present, this IE should include the full list of DNAIs of interest for PDU session, including DNAIs that may not be supported by the I-SMF and excluding the ones supported by the Anchor SMF. | DTSSA DTSSA-Ext1 |
| ipv6MultiHomingInd | boolean | C | 0..1 | This IE shall be present over N16a, if available and an I-SMF has been inserted into the PDU session during the following procedures: PDU session establishment, Registration, Service Request, Xn based handover, Inter NG-RAN node N2 based handover (see clause 4.23 of 3GPP TS 23.502 [3]). When present, it shall be set as follows: - true: IPv6 multi-homing is permitted. - false (default): IPv6 multi-homing is not allowed. | DTSSA |
| maAcceptedInd | boolean | C | 0..1 | This IE shall be present if a request to establish a MA PDU session was accepted or if a single access PDU session was upgraded into a MA PDU session (see clauses 4.22.2 and 4.22.3 of 3GPP TS 23.502 [3]). When present, it shall be set as follows: - true: MA PDU session - false (default): single access PDU session | MAPDU |
| homeProvidedChargingId | string | O | 0..1 | When present, this IE shall contain the Home provided Charging ID (see 3GPP TS 32.255 [25]). This IE shall be present during an EPS to 5GS Idle mode mobility or Handover of a HR PDU session. (NOTE 5) The string shall encode the Charging ID (32-bit unsigned integer value, with maximum value "4294967295") in decimal representation. Pattern: '^(0¦([1-9]{1}[0-9]{0,9}))$' (NOTE 8) |  |
| nefExtBufSupportInd | boolean | C | 0..1 | This IE shall be present with value "true", if NEF has indicated Extended Buffering Support for mobile terminated data in SMF-NEF connection establishment response. When present, this IE shall be set as following: - true: Extended Buffering supported by NEF - false (default): Extended Buffering not supported by NEF | CIOT |
| smallDataRateControlEnabled | boolean | C | 0..1 | This IE shall be present and set to "true" if small data rate control is applicable on the PDU session. When present, it shall be set as follows: - true: small data rate control is applicable. - false (default): small data rate control is not applicable. | CIOT |
| ueIpv6InterfaceId | string | C | 0..1 | This IE shall be present if the H-SMF/SMF has assigned IPv6 interface identifier to the UE during the PDU session establishment for the Home-routed Roaming scenario or for a PDU session with an I-SMF. When present, it shall encode the UE IPv6 Interface Identifier to be used by the UE for its link-local address configuration with 16 hexadecimal digits. Pattern: "^[A-Fa-f0-9]{16}$" |  |
| ipv6Index | IpIndex | C | 0..1 | This IE shall be present if IPv6 Index has been received from PCF during SM Policy Creation. (NOTE 4) | DTSSA |
| dnAaaAddress | IpAddress | O | 0..1 | When present, this IE shall contain the address of DN-AAA server for UE IP Address allocation that has been received from UDM. (NOTE 4). | DTSSA |
| redundantPduSessionInfo | RedundantPduSessionInformation | C | 0..1 | This IE shall be present for a PDU session with an I-SMF, if Dual Connectivity based end to end Redundant User Plane Paths shall apply as specified in clause 5.33.2.1 of 3GPP TS 23.501 [2], regardless of whether the redundantPduSessionInfo IE was received or not in the request. If an RSN and/or PDU Session Pair ID was received from the UE, the same RSN and/or PDU Session Pair ID shall be returned in the response; additionally, if either the RSN or PDU Session Pair ID was not received from the UE, the anchor SMF shall determine and also return an RSN or PDU Session Pair ID respectively in the response. | DCE2ER |
| nspuSupportInd | boolean | C | 0..1 | This IE shall be present and set to "true" if enablePauseCharging is set to "true" and if the (H-)SMF and PSA UPF support Notify Start Pause of Charging via user plane feature as specified in clause 5.30 of 3GPP TS 29.244 [29]. When present, it shall be set as follows: - true: Notify Start Pause of Charging via user plane feature is supported. |  |
| interPlmnApiRoot | Uri | C | 0..1 | This IE should be present if the PDU session may be subject to inter-PLMN mobility and different PDU session context URIs shall be used for intra-PLMN and inter-PLMN signaling requests targeting the PDU session context. When present, it shall contain the apiRoot of the PDU session context to be used in inter-PLMN signalling request targeting the PDU session context. (NOTE 7) |  |
| intraPlmnApiRoot | Uri | C | 0..1 | This IE should be present if the PDU session may be subject to inter-PLMN mobility and different PDU session context URIs shall be used for intra-PLMN and inter-PLMN signaling requests targeting the PDU session context. When present, it shall contain the apiRoot of the PDU session context to be used in intra-PLMN signalling request targeting the PDU session context. (NOTE 7) |  |
| hrsboRspInfo | HrsboRspInfo | C | 0..1 | This IE shall be present in HR roaming scenarios if the H-SMF supports the HR-SBO feature and it receives a request for HR-SBO. When present, this IE shall Include the information for HR-SBO. The absence of this IE shall indicate that the HR-SBO is not allowed. | HR-SBO |
| NOTE 1: This IE contains information that the V-SMF or I-SMF only needs to transfer to the UE (without interpretation). It is sent as a separate IE rather than within the n1SmInfoToUE binary data because the Selected SSC mode IE is defined as a "V" IE (i.e. without a Type field) in the NAS PDU Session Establishment Accept message. NOTE 2: In scenarios with a V-SMF/I-SMF insertion, the V-SMF/I-SMF may receive in the Create Response some IEs it has already received during the earlier SM context retrieval from the SMF (e.g. due to the condition of presence of IEs in the Create Response). In such a case, the V-SMF/I-SMF shall overwrite the IEs earlier received with the new IEs received in the Create Response. NOTE 3: The V-SMF/I-SMF shall ignore any QoS Rule(s) associated to a QoS flow received in PduSessionCreatedData during V-SMF/I-SMF insertion scenarios where no QoS Rule(s) can be sent to the UE, i.e. during Registration, Inter NG-RAN node N2 based handover, and EPS to 5GS Idle mode mobility/handover using N26 interface procedures with V-SMF/I-SMF insertion, or during Service Request and Xn based handover procedures with I-SMF insertion. In such scenarios, the (H-)SMF shall initiate a subsequent PDU session modification procedure if it needs to change the QoS Rules associated to the QoS flows. NOTE 4: The I-SMF may use IPv6 index to assist in selecting how the IPv6 prefix is to be allocated for local PSA when IPv6 multi-homing is applied for the PDU session. If the IPv6 index indicates UE IP address allocation should be performed towards DN-AAA server, the DN-AAA server address may be included from the SMF to the I-SMF. NOTE 5: The chargingId IE in SmContext (see clause 6.1.6.2.39) shall be set to the value received in the homeProvidedChargingId IE during an EPS to 5GS Idle mode mobility or Handover of a HR PDU session. NOTE 6: During inter-system mobility from EPS to 5GS, the UE Integrity Protection Maximum Data Rate is not available at the SMF during PDU Session Creation. The UE will provide UE Integrity Protection Maximum Data Rate to the network within a subsequent UE triggered PDU session modification procedure, as specified in clause 4.3.3.2 of 3GPP TS 23.502 [3]. NOTE 7: During an inter-PLMN mobility, after retrieving the SM context from the old V-SMF, I-SMF or anchor SMF, the target V-SMF or I-SMF shall replace the apiRoot of the pduSessionRef with the interPlmnApiRoot (if available) if the anchor SMF is not in the target PLMN, or with the intraPlmnApiRoot (if available) otherwise. The Operator Identifier in the DNN indicates the PLMN ID of the anchor SMF. NOTE 8: Usage of Charging ID with Uint32 value for roaming scenarios may lead to Charging ID collision between SMFs. |  |  |  |  |  |

* * * Next Change * * * *

6.1.6.2.x	Type: HrsboReqInfo

Table 6.1.6.2.x-1: Definition of type HrsboReqInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| hrsboAuthReqInd | boolean | M | 1 | This IE shall Indicate whether authorization of Session Breakout for HR Session using ULCL/BP in VPLMN is requested: - true: Requested. - false: Not requested. |
| vEasdfAddr | IpAddress | C | 0..1 | This IE shall be present if available. If present, this IE shall contain the IP Address of the V-EASDF. (NOTE) |
| NOTE: The V-EASDF address is sent from H-SMF to UE in n1SmInfoToUe attribute. |  |  |  |  |

* * * Next Change * * * *

6.1.6.2.y	Type: HrsboRspInfo

Table 6.1.6.2.y-1: Definition of type HrsboRspInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| hrsboAuthResult | boolean | M | 1 | This IE shall Indicate whether HR-SBO request is authorized - true: authorized. - false: Not authorized. |
| hDnsAddr | IpAddress | C | 0..1 | This IE shall be present if available. If present, this IE shall contain the DNS server address of HPLMN. |

* * * Next Change * * * *

6.1.8	Feature Negotiation

The feature negotiation mechanism specified in clause 6.6 of 3GPP TS 29.500 [4] shall be used to negotiate the optional features applicable between the SMF and the NF Service Consumer, for the Nsmf_PDUSession service, if any.

The NF Service Consumer shall indicate the optional features it supports for the Nsmf_PDUSession service, if any, by including the supportedFeatures attribute in the HTTP POST request when requesting the SMF to create an SM context or a PDU session resource. In scenarios with a change of NF Service Consumer (e.g. change of AMF, V-SMF or I-SMF change), the new NF Service Consumer shall indicate the optional features it supports for the Nsmf_PDUSession service, if any, by including the supportedFeatures attribute in the HTTP POST request when requesting the SMF to update an SM context or a PDU session resource to change the NF Service Consumer.

The SMF shall determine the supported features for the created SM context or PDU session resource, or for the updated SM context or PDU session resource in scenarios with a change of NF Service Consumer, as specified in clause 6.6 of 3GPP TS 29.500 [4] and shall indicate the supported features by including the supportedFeatures attribute in the representation of the SM context or PDU session resource it returns in the HTTP response confirming the creation or the modification of the resource.

The syntax of the supportedFeatures attribute is defined in clause 5.2.2 of 3GPP TS 29.571 [13].

The following features are defined for the Nsmf_PDUSession service.

Table 6.1.8-1: Features of supportedFeatures attribute used by Nsmf_PDUSession service

| Feature Number | Feature | M/O | Description |
| --- | --- | --- | --- |
| 1 | CIOT | O | Cellular IoT Support of this feature implies the support of all the CIoT features specified in clause 5.31 of 3GPP TS 23.501 [2], including in particular corresponding SMF PDUSession service's extensions to support: - NB-IoT and LTE-M RAT types; - Control Plane CIoT 5GS Optimisation; - Rate control of user data; - Idle mode mobility with data forwarding between EPS and 5GS using N26 interface. The SMF shall indicate its support of this feature in supportedFeatures attribute in its profile registered in NRF. A NF service consumer (e.g. AMF) shall only select SMF(s) that supports this feature for PDU sessions with Control Plane CIoT 5GS Optimisation enabled. |
| 2 | MAPDU | O | Multi-Access PDU Session An SMF that supports this feature shall support the procedures specified in3GPP TS 23.501 [2] and 3GPP TS 23.502 [3] related to Access Traffic Steering, Switching and Splitting. |
| 3 | DTSSA | O | Deployments Topologies with specific SMF Service Areas A NF Service Consumer and an SMF that support this feature shall support the procedures specified in clause 5.34 of 3GPP TS 23.501 [2] and in clause 4.23 of 3GPP TS 23.502 [3]. |
| 4 | CARPT | O | SMF derived CN Assisted RAN parameters Tuning. A NF Service Consumer (e.g. AMF) and an SMF that support this feature shall support exchanging SMF derived CN assisted RAN parameters in Notify SM Context Status service operation (see clause 5.2.2.5.1). |
| 5 | CTXTR | O | This feature bit indicates whether the NF Service Consumer (e.g. AMF) and SMF supports Network Function/NF Service Context Transfer Procedures specified in clause 4.26 of 3GPP TS 23.502 [3]. The SMF shall only trigger these context transfer procedures if the NF Service Consumer has indicated support of this feature. |
| 6 | VQOS | O | VPLMN QoS An SMF that supports this feature shall support: - the handling of QoS constraints from the VPLMN during a HR PDU session establishment as specified in clause 4.3.2.2.2 of 3GPP TS 23.502 [3]; and - QoS modification requests initiated by the VPLMN, as specified in clause 4.3.3.3 of 3GPP TS 23.502 [3]. |
| 7 | HOFAIL | M | This feature bit indicates whether the NF Service Consumer (e.g. AMF, V-SMF, I-SMF) and SMF supports the Notify (SM Context) Status procedure to indicate a handover failure with the Resource Status set to "UPDATED" between 3GPP access and non-3GPP access as specified in clauses 5.2.2.5.1 and 5.2.2.10.1. The SMF shall only trigger such a resource status notify procedure if the NF Service Consumer has indicated support of this feature. |
| 8 | ES3XX | M | Extended Support of HTTP 307/308 redirection An NF Service Consumer (e.g. AMF, V-SMF, I-SMF) that supports this feature shall support handling of HTTP 307/308 redirection for any service operation of the PDUSession service. An NF Service Consumer that does not support this feature does only support HTTP redirection as specified for 3GPP Release 15. |
| 9 | DCE2ER | O | Dual Connectivity based end to end Redundant User Plane Paths An NF service consumer (e.g. I-SMF) and SMF that supports this feature shall support the procedures specified in clause 5.33.2.1 of 3GPP TS 23.501 [2]. |
| 10 | AASN | M | This feature bit indicates whether the NF Service Consumer (e.g. AMF) and the SMF support the Notify SM Context Status procedure to indicate that the PDU session is established towards an alternative anchor SMF, as specified in clause 5.2.2.5.1. The SMF shall only trigger such a Notify SM Context Status procedure if the NF Service Consumer has indicated support of this feature. |
| 11 | EnEDGE | O | Enhancement of Edge Computing in 5G Core network A NF Service Consumer and an SMF that support this feature shall support to signal the target DNAI in Notify (SM Context) Status and Create SM Context service operations, support to signal the URI of the SM Context resource in Notify SM Context Status, Create SM Context and create service operations / signal the URI of the PDU Session resource in Notify (SM Context) Status, Create SM Context and create service operations to retrieve the AF Coordination Information, as specified in 3GPP TS 23.501 [2], 3GPP TS 23.502 [3] and 3GPP TS 23.548 [39]. |
| 12 | SCPBU | O | Simultaneous Change of PSA and BP or UL CL This feature bit indicates whether the NF Service Consumer (e.g. I-SMF) and the SMF support the n4InfoExt3 IE included in VsmfUpdateData, VsmfUpdatedData or VsmfUpdateError to support the simultaneous change of PSA and BP or UL CL controlled by I-SMF. The SMF shall only include the n4InfoExt3 IE in VsmfUpdateData if the NF Service Consumer has indicated support of this feature. |
| 13 | ENPN | O | Enhanced support of Non-Public Networks Support of this feature implies the support of the Remote Provisioning of UEs in Onboarding Network procedures, as specified in clause 5.30.2.10.4 of 3GPP TS 23.501 [2] and clause 4.3.2.2.1 of 3GPP TS 23.502 [3]. The SMF shall indicate its support of this feature in supportedFeatures attribute in its profile registered in NRF. A NF service consumer (e.g. AMF) shall select SMF(s) that supports this feature to setup PDU sessions for Remote Provisioning of UEs in Onboarding Network. |
| 14 | SPAE | O | SM Policy Association Events This feature bit indicates whether the NF Service Consumer (e.g. AMF) and the SMF supports the SM Policy Association establishment and termination event notification information handling, i.e. whereby the PCF for UE subscribes to SM Policy Association events to the PCF for SM Policy via the AMF and SMF, as specified in clause 4.3.2.2.1 and clause 4.3.3.2 of 3GPP TS 23.502 [3]. |
| 15 | 5GSAT | O | This feature bit indicates whether the NF Service Consumer (e.g. AMF, V-SMF, I-SMF) and SMF support the reporting of satellite backhaul information, as specified in clause 5.8.2.15 of 3GPP TS 23.501 [2]. |
| 16 | UPIPE | O | User Plane Integrity Protection with EPS An NF service consumer (e.g. AMF) and SMF that supports this feature shall support the User Plane Integrity Protection with EPS specified in clauses 4.11.1 and 4.11.5.3 of 3GPP TS 23.502 [3]. |
| 17 | BIUMR | O | This feature bit indicates whether the NF Service Consumer (e.g. AMF, V-SMF, I-SMF) and SMF supports Binding Indication Update for multiple resource contexts specified in clauses 6.12.1 and 5.2.3.2.6 of 3GPP TS 29.500 [4]. |
| 18 | ACSCR | O | Absence of smfUri and hSmfUri attributes in Create SM Context Request for procedures with I-SMF/V-SMF insertion/change other than PDU session establishment and EPS to 5GS mobility procedures. This feature bit indicates that the NF Service Consumer (e.g. AMF) supports not including, and the I-SMF/V-SMF supports not receiving, the smfUri and hSmfUri attributes in the Create SM Context request in procedures with I-SMF/V-SMF insertion/change other than PDU session establishment and EPS to 5GS mobility procedures. An NF Service Consumer and I-SMF/V-SMF complying with this release of the specification shall support this feature if the DTSSA feature is supported. The support of this feature may remove the need for the AMF to fetch the smfUri or hsmfUri from the NRF where the anchor SMF profile is registered, e.g. enable the AMF to skip an inter-PLMN NF Discovery procedure towards the HPLMN during a V-SMF insertion/change when the AMF can determine by other means (e.g. using the anchorSmfSupportedFeatures attribute in PDU session context received over N14) whether the HPLMN supports the DTSSA feature, and accordingly, to fasten the execution of mobility (e.g. handover) scenarios. |
| 19 | PSETR | O | This feature bit indicates that the SMF is able to (re)select an alternative peer SMF (when available) when it detects the peer SMF has failed. See also clause 6.8 in 3GPP TS 23.527 [24]. An SMF implementation (complying with this release of the specification) should support the PSETR feature (i.e. support reselecting a peer SMF service instance when the peer SMF fails) |
| 20 | DLSET | O | This feature bit indicates that the PDU session resources served by the SMF are not exclusively bound to a SMF service instance, i.e. they are shared by multiple SMF service instances. See also clause 6.8 in 3GPP TS 23.527 [24]. |
| 21 | N9FSC | O | N9 Forwarding between Branching Points or UL CLs controlled by the same or different I-SMFs for EAS Session Continuity. An NF Service Consumer and I-SMF/SMF that support this feature shall support the procedures specified in clauses 4.23.9.4 and 4.23.9.5 of 3GPP TS 23.502 [3] related to the N9 forwarding tunnel establishment between Branching Points or UL CLs controlled by the same or different I-SMFs to support EAS session continuity. |
| 22 | DTSSA-Ext1 | O | This feature bit indicates that the full list of DNAIs of interest for PDU session, including DNAIs that may not be supported by the I-SMF and excluding the ones supported by the anchor SMF, can be provisioned by the anchor SMF or handled by the I-SMF, which enables the (target) I-SMF to receive such information earlier during an I-SMF insertion or change procedures, so that the I-SMF can decide to insert UL CL/BP and/or a local PSA earlier to save some signalling transactions. |
| x | HR-SBO | O | Home Routed Session BreakOut An NF service consumer (e.g. AMF and V-SMF) and SMF that supports this feature shall support local traffic routing in VPLMN for HR-SBO specified in clause 6.7 of 3GPP TS 23.548 [39]. |
| Feature number: The order number of the feature within the supportedFeatures attribute (starting with 1). Feature: A short name that can be used to refer to the bit and to the feature. M/O: Defines if the implementation of the feature is mandatory ("M") or optional ("O"). Description: A clear textual description of the feature. |  |  |  |

* * * Next Change * * * *

A.2	Nsmf_PDUSession API

openapi: 3.0.0

info:

version: '1.3.0-alpha.2'

title: 'Nsmf_PDUSession'

description: |

SMF PDU Session Service.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

[…]

SmContextCreateData:

description: Data within Create SM Context Request

type: object

properties:

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

unauthenticatedSupi:

type: boolean

default:  false

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

pduSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

selectedDnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

sNssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

hplmnSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

servingNfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

guami:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

serviceName:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/ServiceName'

servingNetwork:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

requestType:

$ref: '#/components/schemas/RequestType'

n1SmMsg:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

anType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

additionalAnType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

ratType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

presenceInLadn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceState'

ueLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

ueTimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

addUeLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

smContextStatusUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

hSmfUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

hSmfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

smfUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

smfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

additionalHsmfUri:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

minItems: 1

additionalHsmfId:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

minItems: 1

additionalSmfUri:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

minItems: 1

additionalSmfId:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

minItems: 1

oldPduSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

pduSessionsActivateList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

minItems: 1

ueEpsPdnConnection:

$ref: '#/components/schemas/EpsPdnCnxContainer'

hoState:

$ref: '#/components/schemas/HoState'

pcfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

pcfGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

pcfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

nrfUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

selMode:

$ref: '#/components/schemas/DnnSelectionMode'

backupAmfInfo:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BackupAmfInfo'

minItems: 1

traceData:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TraceData'

udmGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

routingIndicator:

type: string

hNwPubKeyId:

type: integer

epsInterworkingInd:

$ref: '#/components/schemas/EpsInterworkingIndication'

indirectForwardingFlag:

type: boolean

directForwardingFlag:

type: boolean

targetId:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/NgRanTargetId'

epsBearerCtxStatus:

$ref: '#/components/schemas/EpsBearerContextStatus'

cpCiotEnabled:

type: boolean

default: false

cpOnlyInd:

type: boolean

default: false

invokeNef:

type: boolean

default: false

maRequestInd:

type: boolean

default: false

maNwUpgradeInd:

type: boolean

default: false

n2SmInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

n2SmInfoType:

$ref: '#/components/schemas/N2SmInfoType'

n2SmInfoExt1:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

n2SmInfoTypeExt1:

$ref: '#/components/schemas/N2SmInfoType'

smContextRef:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

smContextSmfPlmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

smContextSmfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

smContextSmfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

smContextSmfServiceSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfServiceSetId'

smContextSmfBinding:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/SbiBindingLevel'

upCnxState:

$ref: '#/components/schemas/UpCnxState'

smallDataRateStatus:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SmallDataRateStatus'

apnRateStatus:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApnRateStatus'

extendedNasSmTimerInd:

type: boolean

default: false

dlDataWaitingInd:

type: boolean

default: false

ddnFailureSubs:

$ref: '#/components/schemas/DdnFailureSubs'

smfTransferInd:

type: boolean

default: false

oldSmfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

oldSmContextRef:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

wAgfInfo:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/WAgfInfo'

tngfInfo:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/TngfInfo'

twifInfo:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/TwifInfo'

ranUnchangedInd:

type: boolean

samePcfSelectionInd:

type: boolean

default: false

targetDnai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

nrfManagementUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

nrfDiscoveryUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

nrfAccessTokenUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

nrfOauth2Required:

type: object

description: 'Map indicating whether the NRF requires Oauth2-based authorization for accessing its services. The key of the map shall be the name of an NRF service, e.g. "nnrf-nfm" or "nnrf-disc"'

additionalProperties:

type: boolean

minProperties: 1

smfBindingInfo:

type: string

pvsInfo:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ServerAddressingInfo'

minItems: 1

onboardingInd:

type: boolean

default: false

oldPduSessionRef:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

smPolicyNotifyInd:

type: boolean

default: false

pcfUeCallbackInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PcfUeCallbackInfo'

satelliteBackhaulCat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SatelliteBackhaulCategory'

upipSupported:

type: boolean

default: false

uavAuthenticated:

type: boolean

disasterRoamingInd:

type: boolean

default: false

anchorSmfOauth2Required:

type: boolean

smContextSmfOauth2Required:

type: boolean

hrsboAllowedInd:

type: boolean

default: false

required:

- servingNfId

- servingNetwork

- anType

- smContextStatusUri

[…]

PduSessionCreateData:

description: Data within Create Request

type: object

properties:

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

unauthenticatedSupi:

type: boolean

default: false

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

pduSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

selectedDnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

sNssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

hplmnSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

vsmfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

ismfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

servingNetwork:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

requestType:

$ref: '#/components/schemas/RequestType'

epsBearerId:

type: array

items:

$ref: '#/components/schemas/EpsBearerId'

minItems: 1

pgwS8cFteid:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

vsmfPduSessionUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

ismfPduSessionUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

vcnTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

icnTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

n9ForwardingTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

additionalCnTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

anType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

additionalAnType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

ratType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

ueLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

ueTimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

addUeLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

n1SmInfoFromUe:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

unknownN1SmInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

hPcfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

pcfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

pcfGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

pcfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

hoPreparationIndication:

type: boolean

selMode:

$ref: '#/components/schemas/DnnSelectionMode'

alwaysOnRequested:

type: boolean

default: false

udmGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

routingIndicator:

type: string

hNwPubKeyId:

type: integer

epsInterworkingInd:

$ref: '#/components/schemas/EpsInterworkingIndication'

vSmfServiceInstanceId:

type: string

iSmfServiceInstanceId:

type: string

recoveryTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

roamingChargingProfile:

$ref: 'TS32291_Nchf_ConvergedCharging.yaml#/components/schemas/RoamingChargingProfile'

chargingId:

type: string

pattern: '^(0|([1-9]{1}[0-9]{0,9}))$'

oldPduSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

epsBearerCtxStatus:

$ref: '#/components/schemas/EpsBearerContextStatus'

amfNfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

guami:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

maxIntegrityProtectedDataRateUl:

$ref: '#/components/schemas/MaxIntegrityProtectedDataRate'

maxIntegrityProtectedDataRateDl:

$ref: '#/components/schemas/MaxIntegrityProtectedDataRate'

cpCiotEnabled:

type: boolean

default: false

cpOnlyInd:

type: boolean

default: false

invokeNef:

type: boolean

default: false

maRequestInd:

type: boolean

default: false

maNwUpgradeInd:

type: boolean

default: false

dnaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

presenceInLadn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceState'

secondaryRatUsageInfo:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SecondaryRatUsageInfo'

minItems: 1

smallDataRateStatus:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SmallDataRateStatus'

apnRateStatus:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApnRateStatus'

dlServingPlmnRateCtl:

type: integer

minimum: 10

upSecurityInfo:

$ref: '#/components/schemas/UpSecurityInfo'

vplmnQos:

$ref: '#/components/schemas/VplmnQos'

oldSmContextRef:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

redundantPduSessionInfo:

$ref: '#/components/schemas/RedundantPduSessionInformation'

oldPduSessionRef:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

smPolicyNotifyInd:

type: boolean

default: false

pcfUeCallbackInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PcfUeCallbackInfo'

satelliteBackhaulCat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SatelliteBackhaulCategory'

upipSupported:

type: boolean

default: false

upCnxState:

$ref: '#/components/schemas/UpCnxState'

disasterRoamingInd:

type: boolean

default: false

hrsboReqInfo:

$ref: '#/components/schemas/HrsboReqInfo'

required:

- dnn

- servingNetwork

- anType

oneOf:

- required: [ vsmfId, vsmfPduSessionUri ]

- required: [ ismfId, ismfPduSessionUri ]

PduSessionCreatedData:

description: Data within Create Response

type: object

properties:

pduSessionType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionType'

sscMode:

type: string

pattern: '^[0-7]$'

hcnTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

cnTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

additionalCnTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

sessionAmbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ambr'

qosFlowsSetupList:

type: array

items:

$ref: '#/components/schemas/QosFlowSetupItem'

minItems: 1

hSmfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

smfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

pduSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

sNssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

enablePauseCharging:

type: boolean

default: false

ueIpv4Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

ueIpv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

n1SmInfoToUe:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

epsPdnCnxInfo:

$ref: '#/components/schemas/EpsPdnCnxInfo'

epsBearerInfo:

type: array

items:

$ref: '#/components/schemas/EpsBearerInfo'

minItems: 1

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

maxIntegrityProtectedDataRate:

$ref: '#/components/schemas/MaxIntegrityProtectedDataRate'

maxIntegrityProtectedDataRateDl:

$ref: '#/components/schemas/MaxIntegrityProtectedDataRate'

alwaysOnGranted:

type: boolean

default: false

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

upSecurity:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UpSecurity'

roamingChargingProfile:

$ref: 'TS32291_Nchf_ConvergedCharging.yaml#/components/schemas/RoamingChargingProfile'

hSmfServiceInstanceId:

type: string

smfServiceInstanceId:

type: string

recoveryTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

dnaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

ipv6MultiHomingInd:

type: boolean

default: false

maAcceptedInd:

type: boolean

default: false

homeProvidedChargingId:

type: string

pattern: '^(0|([1-9]{1}[0-9]{0,9}))$'

nefExtBufSupportInd:

type: boolean

default: false

smallDataRateControlEnabled:

type: boolean

default: false

ueIpv6InterfaceId:

type: string

pattern: '^[A-Fa-f0-9]{16}$'

ipv6Index:

$ref: 'TS29519_Policy_Data.yaml#/components/schemas/IpIndex'

dnAaaAddress:

$ref: '#/components/schemas/IpAddress'

redundantPduSessionInfo:

$ref: '#/components/schemas/RedundantPduSessionInformation'

nspuSupportInd:

type: boolean

interPlmnApiRoot:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

intraPlmnApiRoot:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

hrsboRspInfo:

$ref: '#/components/schemas/HrsboRspInfo'

required:

- pduSessionType

- sscMode

oneOf:

- required: [ hSmfInstanceId ]

- required: [ smfInstanceId ]

[…]

AnchorSmfFeatures:

description: Anchor SMF supported features

type: object

properties:

psetrSupportInd:

type: boolean

HrsboReqInfo:

description: HR-SBO Information in request message

type: object

properties:

hrsboAuthReqInd:

type: boolean

vEasdfAddr:

$ref: '#/components/schemas/IpAddress'

required:

- hrsboAuthReqInd

HrsboRspInfo:

description: HR-SBO Information in response message

type: object

properties:

hrsboAuthResult:

type: boolean

hDnsAddr:

$ref: '#/components/schemas/IpAddress'

required:

- hrsboAuthResult

[…]

* * * End of Changes * * * *