3GPP TSG-CT Meeting #99	CP-230053

# Rotterdam, Netherlands, 20th – 21st March 2023			revision of C4-230641

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.503 | CR | 0978 | rev | 2 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Time Sync Data in AccessAndMobilitySubscriptionData |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Work item code: | TRS_URLLC |  |  |  |  |  | Date: |  |  | 2023-03-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to 23.502 clause 4.28.2.1 step 1 (see S2-2211311), AccessAndMobilitySubscriptionData are extended to contain Time Synchronization Data. |  |  |  |  |  |  |  |  |
| Summary of change: |  | AccessAndMobilitySubscriptionData are extended with Time Synchronization Data. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirement from 23.502 cannot be implemented |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.1.6.1, 6.1.6.2.4, 6.1.6.2.xx (new), A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards compatible new features with impact to the following APIs: TS29503_Nudm_SDM.yaml TS29504_Nudr_DR.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev2: correct OpenAPI errors detected after CT4#114 |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	3GPP TS 33.501: "Security Architecture and Procedures for 5G System".

[7]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces Stage 3".

[8]	3GPP TS 23.003: "Numbering, addressing and identification".

[9]	3GPP TS 29.504: "5G System; Unified Data Repository Services; Stage 3".

[10]	3GPP TS 29.505: "5G System; Usage of the Unified Data Repository Services for Subscription Data; Stage 3".

[11]	3GPP TS 32.255: "Charging management; 5G data connectivity domain charging".

[12]	3GPP TS 32.298: "Charging management; Charging Data Record (CDR) parameter description".

[13]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[14]	OpenAPI Initiative, "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[15]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[16]	IETF RFC 7807: "Problem Details for HTTP APIs".

[17]	IETF RFC 7396: "JSON Merge Patch".

[18]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[19]	3GPP TS 29.510: "Network Function Repository Services; Stage 3".

[20]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station in idle mode".

[21]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[22]	3GPP TS 29.338: "Diameter based protocols to support Short Message Service (SMS) capable Mobile Management Entities (MMEs)"

[23]	ITU-T Recommendation E.164: "The international public telecommunication numbering plan".

[24]	3GPP TS 29.509: "Authentication Server Services; Stage 3".

[25]	IETF RFC 7232: "Hypertext Transfer Protocol (HTTP/1.1): Conditional Requests".

[26]	IETF RFC 7234: "Hypertext Transfer Protocol (HTTP/1.1): Caching".

[27]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[28]	ETSI TS 102 225: "Smart Cards; Secured packet structure for UICC based applications".

[29]	IETF RFC 7542: "The Network Access Identifier".

[30]	3GPP TR 21.900: "Technical Specification Group working methods".

[31]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[32]	3GPP TS 23.632: "User Data Interworking, Coexistence and Migration"

[33]	3GPP TS 29.519: "Policy Data, Application Data and Structured Data for Exposure; Stage 3".

[34]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[35]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[36]	3GPP TS 29.518: "Access and Mobility Management Services".

[37]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS); Stage 2".

[38]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[39]	3GPP TS 29.515: "5G System; Gateway Mobile Location Services; Stage 3".

[40]	3GPP TS 29.508: "5G System; Session Management Event Exposure Service; Stage 3".

[41]	IETF RFC 6902: "JavaScript Object Notation (JSON) Patch".

[42]	BBF TR-069: "CPE WAN Management Protocol".

[43]	BBF TR-369: "User Services Platform (USP)".

[44]	3GPP TS 29.524: "5G System; Cause codes mapping between 5GC interfaces; Stage 3".

[45]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[46]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[47]	3GPP TS 22.071: "Location Services (LCS); Service description; Stage 1".

[48]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[49]	3GPP TS 24.302: "Access to the 3GPP Evolved Packet Core (EPC) via non-3GPP access networks".

[50]	IETF RFC 7230: "Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing".

[51]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[52]	3GPP TS 29.328: "IP Multimedia (IM) Subsystem Sh interface; Signalling flows and message contents".

[53]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[54]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[55]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

[56]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[57]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[58]	3GPP TS 29.520: "5G System; Network Data Analytics Services; Stage 3".

[59]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[60]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

[61]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[62]	3GPP TS 29.563: "5G System; Home Subscriber Server (HSS) services for interworking with Unified Data Management (UDM); Stage 3"

[63]	3GPP TS 33.558: "Security aspects of enhancement of support for enabling edge applications; Stage 2".

[64]	3GPP TS 33.503: " Security Aspects of Proximity based Services (ProSe) in the 5G System (5GS)".

[65]	3GPP TS 29.562: "5G System; Home Subscriber Server (HSS) Services; Stage 3"

[66]	3GPP TS 23.540: " 5G System; Technical realization of Service Based Short Message Service; Stage 2".

[x]	3GPP TS 29.514: " 5G System; Policy Authorization Service; Stage 3".

* * * Next Change * * * *

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nudm_SDM service API.

Table 6.1.6.1-1: Nudm_SDM specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| Nssai | 6.1.6.2.2 | Network Slice Selection Assistance Information |
| SdmSubscription | 6.1.6.2.3 | A subscription to notifications |
| AccessAndMobilitySubscriptionData | 6.1.6.2.4 | Access and Mobility Subscription Data |
| SmfSelectionSubscriptionData | 6.1.6.2.5 | SMF Selection Subscription Data |
| DnnInfo | 6.1.6.2.6 | Data Network Name and associated information (LBO roaming allowed flag) |
| SnssaiInfo | 6.1.6.2.7 | S-NSSAI and associated information (DNN Info) |
| SessionManagementSubscriptionData | 6.1.6.2.8 | User subscribed session management data |
| DnnConfiguration | 6.1.6.2.9 | User subscribed data network configuration |
| PduSessionTypes | 6.1.6.2.11 | Default/allowed session types for a data network |
| SscModes | 6.1.6.2.12 | Default/allowed SSC modes for a data network |
| SmsSubscriptionData | 6.1.6.2.13 |  |
| SmsManagementSubscriptionData | 6.1.6.2.14 | SMS Management Subscription Data |
| SubscriptionDataSets | 6.1.6.2.15 |  |
| UeContextInSmfData | 6.1.6.2.16 | UE Context In SMF Data |
| PduSession | 6.1.6.2.17 |  |
| IdTranslationResult | 6.1.6.2.18 | SUPI that corresponds to a given GPSI |
| ModificationNotification | 6.1.6.2.21 |  |
| IpAddress | 6.1.6.2.22 | IP address (IPv4, or IPv6, or IPv6 prefix) |
| UeContextInSmsfData | 6.1.6.2.23 |  |
| SmsfInfo | 6.1.6.2.24 |  |
| AcknowledgeInfo | 6.1.6.2.25 |  |
| SorInfo | 6.1.6.2.26 | Steering Of Roaming Information |
| SharedData | 6.1.6.2.27 | Subscription Data shared by multiple UEs |
| PgwInfo | 6.1.6.2.28 | Information about the DNNs/APNs and PGW-C+SMF FQDNs used in interworking with EPS |
| TraceDataResponse | 6.1.6.2.29 | Contains Trace Data or a shared data Id identifying shared Trace Data |
| SteeringContainer | 6.1.6.2.30 |  |
| SdmSubsModification | 6.1.6.2.31 | Modification instruction for a subscription to notifications |
| EmergencyInfo | 6.1.6.2.32 | Information about emergency session |
| UpuInfo | 6.1.6.2.33 | UE Parameters Update Information |
| GroupIdentifiers | 6.1.6.2.34 |  |
| NiddInformation | 6.1.6.2.35 | Non-IP Data Delivery information |
| CagData | 6.1.6.2.36 |  |
| CagInfo | 6.1.6.2.37 |  |
| DataSetName | 6.1.6.3.3 |  |
| PduSessionContinuityInd | 6.1.6.3.7 |  |
| AdditionalSnssaiData | 6.1.6.2.38 | Additional information specific to a slice |
| VnGroupData | 6.1.6.2.39 |  |
| AppDescriptor | 6.1.6.2.40 |  |
| AppPortId | 6.1.6.2.41 | Application Port Id |
| LcsPrivacyData | 6.1.6.2.42 |  |
| Lpi | 6.1.6.2.43 |  |
| UnrelatedClass | 6.1.6.2.44 |  |
| PlmnOperatorClass | 6.1.6.2.45 |  |
| ValidTimePeriod | 6.1.6.2.46 |  |
| LcsMoData | 6.1.6.2.47 |  |
| EcRestrictionDataWb | 6.1.6.2.48 | Enhance Coverage Restriction Data |
| ExpectedUeBehaviourData | 6.1.6.2.49 | Expected UE Behaviour Data |
| SuggestedPacketNumDl | 6.1.6.2.52 | Suggested Number of Downlink Packets |
| FrameRouteInfo | 6.1.6.2.54 | Frame Route Information |
| SorUpdateInfo | 6.1.6.2.55 |  |
| EnhancedCoverageRestrictionData | 6.1.6.2.56 | Enhanced Coverage Restriction Data |
| EdrxParameters | 6.1.6.2.57 | eDRX Parameters |
| PtwParameters | 6.1.6.2.58 | Paging Time Window Parameters |
| OperationMode | 6.1.6.3.12 | Operation Mode |
| SorUpdateIndicator | 6.1.6.3.13 | SoR Update Indicator |
| ExternalUnrelatedClass | 6.1.6.2.62 |  |
| AfExternal | 6.1.6.2.63 |  |
| LcsClientExternal | 6.1.6.2.64 |  |
| LcsClientGroupExternal | 6.1.6.2.65 |  |
| ServiceTypeUnrelatedClass | 6.1.6.2.66 |  |
| UeId | 6.1.6.2.67 |  |
| DefaultUnrelatedClass | 6.1.6.2.68 |  |
| ContextInfo | 6.1.6.2.69 | Contains the HTTP Headers received by the NFs |
| UeContextInAmfData | 6.1.6.2.70 |  |
| V2xSubscriptionData | 6.1.6.2.71 | V2X Subscription Data |
| LcsBroadcastAssistanceTypesData | 6.1.6.2.72 | LCS Broadcast Assistance Data Types |
| DatasetNames | 6.1.6.2.73 | Data Set Names |
| PlmnRestriction | 6.1.6.2.74 |  |
| ProseSubscriptionData | 6.1.6.2.76 | ProSe Subscription Data |
| AerialUeSubscriptionInfo | 6.1.6.2.78 | Aerial UE Subscription Information |
| SmSubsData | 6.1.6.2.79 |  |
| ExtendedSmSubsData | 6.1.6.2.80 |  |
| AmfInfo | 6.1.6.2.81 | AMF information |
| ProSeAllowedPlmn | 6.1.6.2.82 | Contains the allowed PLMN information for ProSe Service |
| ImmediateReport | 6.1.6.2.83 | UDM SDM Immediate Report |
| MbsSubscriptionData | 6.1.6.2.84 | 5MBS Subscription Data |
| UcSubscriptionData | 6.1.6.2.85 | User Consent Subscription Data |
| UeContextInSmfDataSubFilter | 6.1.6.2.86 | UE Context In Smf Data Subscription Filter |
| UeIdentifiers | 6.1.6.2.87 | List of UE identifiers |
| SupiInfo | 6.1.6.2.88 | List of the SUPIs |
| TimeSyncData | 6.1.6.2.xx | Time Synchronization Data |
| DefaultDnnIndicator | 6.1.6.3.2 |  |
| LboRoamingAllowed | 6.1.6.3.2 |  |
| UeUsageType | 6.1.6.3.2 |  |
| MpsPriorityIndicator | 6.1.6.3.2 |  |
| McsPriorityIndicator | 6.1.6.3.2 |  |
| 3GppChargingCharacteristics | 6.1.6.3.2 | 3GPP Charging Characteristics |
| MicoAllowed | 6.1.6.3.2 |  |
| SmsSubscribed | 6.1.6.3.2 |  |
| SharedDataId | 6.1.6.3.2 |  |
| IwkEpsInd | 6.1.6.3.2 | Interworking with EPS Indication |
| SecuredPacket | 6.1.6.3.2 |  |
| UpuRegInd | 6.1.6.3.2 |  |
| ExtGroupId | 6.1.6.3.2 |  |
| NbIoTUePriority | 6.1.6.3.2 |  |
| CodeWord | 6.1.6.3.2 |  |
| AfId | 6.1.6.3.2 |  |
| LcsClientId | 6.1.6.3.2 |  |
| DataSetName | 6.1.6.3.3 |  |
| PduSessionContinuityInd | 6.1.6.3.7 |  |
| LocationPrivacyInd | 6.1.6.3.8 |  |
| PrivacyCheckRelatedAction | 6.1.6.3.9 |  |
| LcsClientClass | 6.1.6.3.10 |  |
| LcsMoServiceClass | 6.1.6.3.11 |  |
| OperationMode | 6.1.6.3.12 |  |
| CodeWordInd | 6.1.6.3.14 |  |
| MdtUserConsent | 6.1.6.3.15 | MDT User Consent |
| SharedDataTreatmentInstruction | 6.1.6.3.16 |  |
| GpsiType | 6.1.6.3.17 | Type of GPSI (MSISDN or External-ID) |
| SorTransparentContainer | 6.1.6.3.2 |  |
| AerialUeIndication | 6.1.6.3.18 | Indication on whether Aerial service for UE is allowed or not. |
| UpuTransparentContainer | 6.1.6.3.2 |  |
| ProseDirectAllowed | 6.1.6.3.19 | Contains the ProSe Services that can be authorised by the given PLMN identity. |
| UcPurpose | 6.1.6.3.20 | Purpose of the user consent |
| UserConsent | 6.1.6.3.21 | User Consent |

Table 6.1.6.1-2 specifies data types re-used by the Nudm_SDM service API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nudm_SDM service API.

Table 6.1.6.1-2: Nudm_SDM re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Dnn | 3GPP TS 29.571 [7] | Data Network Name with Network Identifier only; this type is used as key in a map of: - DnnConfigurations; see clause 6.1.6.2.8; - EpsIwkPgws; see clause 6.2.6.2.2; - ExpectedUeBehaviourData; see clause 6.1.6.2.8; |
| DurationSec | 3GPP TS 29.571 [7] | Time value in seconds |
| ProblemDetails | 3GPP TS 29.571 [7] | Common data type used in response bodies |
| Snssai | 3GPP TS 29.571 [7] | Single NSSAI |
| Uri | 3GPP TS 29.571 [7] | Uniform Resource Identifier |
| Gpsi | 3GPP TS 29.571 [7] | Generic Public Subscription Identifier |
| RatType | 3GPP TS 29.571 [7] | Radio Access Technology Type |
| Area | 3GPP TS 29.571 [7] |  |
| ServiceAreaRestriction | 3GPP TS 29.571 [7] |  |
| CoreNetworkType | 3GPP TS 29.571 [7] |  |
| SupportedFeatures | 3GPP TS 29.571 [7] | see 3GPP TS 29.500 [4] clause 6.6 |
| PlmnId | 3GPP TS 29.571 [7] | PLMN Identity |
| PduSessionType | 3GPP TS 29.571 [7] |  |
| SubscribedDefaultQos | 3GPP TS 29.571 [7] | Subscribed Default QoS |
| Ambr | 3GPP TS 29.571 [7] |  |
| AmbrRm | 3GPP TS 29.571 [7] |  |
| SliceMbrRm | 3GPP TS 29.571 [7] |  |
| PduSessionId | 3GPP TS 29.571 [7] | PduSessionId is used as key in a map of PduSessions; see clause 6.1.6.2.16. |
| NfInstanceId | 3GPP TS 29.571 [7] |  |
| Supi | 3GPP TS 29.571 [7] |  |
| RfspIndexRm | 3GPP TS 29.571 [7] |  |
| SscMode | 3GPP TS 29.571 [7] |  |
| Ipv4Addr | 3GPP TS 29.571 [7] |  |
| Ipv6Addr | 3GPP TS 29.571 [7] |  |
| Ipv6Prefix | 3GPP TS 29.571 [7] |  |
| SorMac | 3GPP TS 29.509 [24] |  |
| SteeringInfo | 3GPP TS 29.509 [24] |  |
| AckInd | 3GPP TS 29.509 [24] |  |
| CounterSor | 3GPP TS 29.509 [24] |  |
| UpuMac | 3GPP TS 29.509 [24] |  |
| UpuData | 3GPP TS 29.509 [24] |  |
| UpuAckInd | 3GPP TS 29.509 [24] |  |
| CounterUpu | 3GPP TS 29.509 [24] |  |
| TraceData | 3GPP TS 29.571 [7] | Trace control and configuration parameters |
| NotifyItem | 3GPP TS 29.571 [7] |  |
| UpSecurity | 3GPP TS 29.571 [7] |  |
| ServiceName | 3GPP TS 29.510 [19] |  |
| OdbPacketServices | 3GPP TS 29.571 [7] |  |
| GroupId | 3GPP TS 29.571 [7] | This type is also used as key of a map in attributes: - vnGroupInfo and sharedVnGroupDataIds; see clause 6.1.6.2.4, 6.1.6.2.8, 6.1.6.2.27; |
| DateTime | 3GPP TS 29.571 [7] |  |
| CagId | 3GPP TS 29.571 [7] |  |
| StnSr | 3GPP TS 29.571 [7] | Session Transfer Number for SRVCC |
| CMsisdn | 3GPP TS 29.571 [7] | Correlation MSISDN |
| OsId | 3GPP TS 29.519 [33] |  |
| Uint16 | 3GPP TS 29.571 [7] |  |
| RgWirelineCharacteristics | 3GPP TS 29.571 [7] |  |
| GeographicArea | 3GPP TS 29.572 [34] |  |
| LcsServiceType | 3GPP TS 29.572 [34] |  |
| ScheduledCommunicationTime | 3GPP TS 29.571 [7] | Scheduled Communication Time |
| LocationArea | 6.5.6.2.10 |  |
| StationaryIndication | 3GPP TS 29.571 [7] | Stationary Indication |
| TrafficProfile | 3GPP TS 29.571 [7] | Traffic Profile |
| ScheduledCommunicationType | 3GPP TS 29.571 [7] | Scheduled Communication Type |
| BatteryIndication | 3GPP TS 29.571 [7] | Battery Indication |
| AcsInfo | 3GPP TS 29.571 [7] | ACS Information |
| IPv4AddrMask | 3GPP TS 29.571 [7] |  |
| Fqdn | 3GPP TS 29.571 [7] | Fully Qualified Domain Name |
| NefId | 3GPP TS 29.510 [19] |  |
| PatchResult | 3GPP TS 29.571 [7] |  |
| NrV2xAuth | 3GPP TS 29.571 [7] |  |
| LteV2xAuth | 3GPP TS 29.571 [7] |  |
| ProseServiceAuth | 3GPP TS 29.571 [7] |  |
| BitRate | 3GPP TS 29.571 [7] |  |
| MdtConfiguration | 3GPP TS 29.571 [7] |  |
| Uint64 | 3GPP TS 29.571 [7] |  |
| WirelineArea | 3GPP TS 29.571 [7] |  |
| WirelineServiceAreaRestriction | 3GPP TS 29.571 [7] |  |
| NfGroupId | 3GPP TS 29.571 [7] |  |
| EcsServerAddr | 3GPP TS 29.571 [7] |  |
| RedirectResponse | 3GPP TS 29.571 [7] | Response body of the redirect response message |
| Bytes | 3GPP TS 29.571 [7] | Binary data encoded as a base64 character string |
| SpatialValidityCond | 3GPP TS 29.571 [7] |  |
| AccessType | 3GPP TS 29.571 [7] | Access Type |
| NsSrg | 3GPP TS 29.571 [7] | NSSRG value |
| MbsSessionId | 3GPP TS 29.571 [7] | 5MBS Session Id |
| RoamingRestrictions | 3GPP TS 29.571 [7] | Roaming Restrictions related information |
| PlmnIdNid | 3GPP TS 29.571 [7] |  |
| Uinteger | 3GPP TS 29.571 [7] | Unsigned Integer |
| Tai | 3GPP TS 29.571 [7] | Tracking Area Identity |
| TemporalValidity | 3GPP TS 29.514 [x] | Temporal Validity contains start time and stop time |

* * * Next Change * * * *

##### 6.1.6.2.4	Type: AccessAndMobilitySubscriptionData

Table 6.1.6.2.4-1: Definition of type AccessAndMobilitySubscriptionData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supportedFeatures | SupportedFeatures | O | 0..1 | See clause 6.1.8 |  |
| gpsis | array(Gpsi) | O | 0..N | List of Generic Public Subscription Identifier; see 3GPP TS 29.571 [7] |  |
| internalGroupIds | array(GroupId) | O | 1..N | List of internal group identifier; see 3GPP TS 23.501 [2] clause 5.9.7 |  |
| sharedVnGroupDataIds | map(SharedDataId) | O | 1..N | A map of identifiers of shared 5G VN group data (list of key-value pairs whereGroupId serves as key; see clause 6.1.6.1). This attribute is only applicable to the Nudm interface and shall not be included over the Nudr interface. |  |
| hssGroupId | NfGroupId | O | 0..1 | Identity of the HSS group associated with the subscription, which may be used by the UDM in discovering the HSS; see 3GPP TS 29.510 [19]. This attribute may be included if the coreNetworkTypeRestrictions does not indicate a value of "EPC". This attribute is only applicable to the Nudr interface and shall not be included over the Nudm interface. |  |
| subscribedUeAmbr | AmbrRm | O | 0..1 |  |  |
| nssai | Nssai | O | 0..1 | Network Slice Selection Assistance Information |  |
| ratRestrictions | array(RatType) | O | 0..N | List of RAT Types that are restricted in 5GC and EPC; see 3GPP TS 29.571 [7] (NOTE 2) Contains unique items |  |
| forbiddenAreas | array(Area) | O | 0..N | List of forbidden areas in 5GS (NOTE 6, NOTE 7) |  |
| serviceAreaRestriction | ServiceAreaRestriction | O | 0..1 | Subscribed Service Area Restriction (NOTE 7) |  |
| coreNetworkTypeRestrictions | array(CoreNetworkType) | O | 0..N | List of Core Network Types that are restricted. The use of the value "5GC" is deprecated on Nudm and shall be discarded by the receiving AMF. |  |
| accessTypeRestrictions | array(AccessType) | O | 0..2 | List of Access Types that are restricted. If non-3GPP access is restricted, then the UDM shall reject/deregister the AMF non-3GPP registration. If 3GPP access is restricted, then the UDM shall reject/deregister the AMF 3GPP registration. Otherwise, the UDM shall pass the IE to the AMF. |  |
| rfspIndex | RfspIndexRm | O | 0..1 | Index to RAT/Frequency Selection Priority; |  |
| subsRegTimer | DurationSecRm | O | 0..1 | Subscribed periodic registration timer; (see clause 5.20 of 3GPP TS 23.501 [2], clause 4.15.3.2.3b and 4.15.6.3a of 3GPP TS 23.502 [3] and 3GPP TS 29.571 [7]) |  |
| ueUsageType | UeUsageType | O | 0..1 |  |  |
| mpsPriority | MpsPriorityIndicator | O | 0..1 |  |  |
| mcsPriority | McsPriorityIndicator | O | 0..1 |  |  |
| activeTime | DurationSecRm | O | 0..1 | subscribed active time for PSM UEs (see clause 5.20 of 3GPP TS 23.501 [2] and clause 4.15.3.2.3b and 4.15.6.3a of 3GPP TS 23.502 [3]). |  |
| sorInfo | SorInfo | O | 0..1 | On Nudm, this IE shall be present if the UDM shall send the information for Steering of Roaming during registration or the subscription data update to the UE. The UDM may detect the need to send sorInfo by retrieving context information from the UDR. (NOTE 4) |  |
| sorInfoExpectInd | Boolean | C | 0..1 | Contains the indication on whether or not the UE is expecting to receive SoR information at initial registration. - When set to true; it indicates that the UE is expecting to receive SoR information at initial registration in a VPLMN, i.e. the UDM shall send SoR information to the AMF on Nudm even when nothing was received from UDR or SOR-AF. In case the UDM was not able to obtain SoR information, SoR information sent on Nudm shall contain the indication that "no change" is needed. - When set to false: it indicates that the UE is not expecting to receive SoR information at initial registration, i.e. the UDM shall send SoR information to the AMF based on operator policy. This attribute may be present on Nudr interface and shall be absent on UDM interface. The UDM shall ignore this attribute if the UE is not roaming out of its HPLMN. |  |
| sorafRetrieval | boolean | C | 0..1 | Contains the indication on whether or not SoR information shall be retrieved from the SOR-AF. - When set to true: it indicates that the UDM shall retrieve SoR information from the SOR-AF. - When set to false or absent: it indicates that the retrieval of SorInfo from the SOR-AF is not required. This attribute may be present on Nudr interface and shall be absent on Nudm interface. The UDM shall ignore this attribute if it is received in Nudr but the UE is not roaming out of its HPLMN. |  |
| sorUpdateIndicatorList | array(SorUpdateIndicator) | C | 1..N | When present, it contains the list of SoR Update Indicators; - It shall indicate that the AMF shall retrieve SoR information when the UE performs Registration with NAS Registration Type "Initial Registration" if the value "INITIAL_REGISTRATION" is included; - And/or it shall indicate that the AMF shall retrieve SoR information when the UE performs Registration with NAS Registration Type "Emergency Registration" if the value "EMERGENCY_REGISTRATION" is included. When absent on Nudm interface, it indicates that the AMF is not requested to retrieve SoR information when the UE performs Registration with either NAS Registration Type "Initial Registration" or NAS Registration Type "Emergency Registration". The UDM shall ignore this attribute if the UE is not roaming out of its HPLMN. |  |
| upuInfo | UpuInfo | O | 0..1 | This IE shall be present if the UDM shall send the information for UE Parameters Update after the UE has been successfully authenticated and registered to the 5G system. |  |
| routingIndicator | string | O | 0..1 | This IE may be sent in Nudm_SDM notification as defined in 3GPP TS 23.502 Clause 4.20.2, if UE Parameter Update was sent for Routing Indicator Update, but without requesting the UE to re-register. |  |
| micoAllowed | MicoAllowed | O | 0..1 | Indicates whether the UE subscription allows MICO mode. |  |
| sharedAmDataIds | array(SharedDataId) | O | 0..N | Identifier of shared Access And Mobility Subscription data | SharedData |
| odbPacketServices | OdbPacketServices | O | 0..1 | Operator Determined Barring for Packet Oriented Services (NOTE 3). |  |
| subscribedDnnList | array(Dnn) | O | 0..N | List of the subscribed DNNs for the UE (including optionally the Wildcard DNN). Used to determine the list of LADN available to the UE as defined in clause 5.6.5 of TS 23.501 [2]. When present, this IE shall contain the Network Identifier only. |  |
| serviceGapTime | DurationSec | O | 0..1 | Used to set the Service Gap timer for Service Gap Control (see TS 23.501 [2] clause 5.26.16 and TS 23.502 [3] clause 4.2.2.2.2). |  |
| mdtUserConsent | MdtUserConsent | O | 0..1 | When present, this IE shall indicate whether the user has given his consent for MDT activation or not (see clause 4.9 of 3GPP TS 32.422 [48]). When absent, "CONSENT_NOT_GIVEN" is the default value. |  |
| mdtConfiguration | MdtConfiguration | C | 0..1 | This IE shall be present if the MDT task is activated. When present, this IE shall contain MDT configuration data for UE (see clause 4.1.2.17 of 3GPP TS 32.422 [48]). |  |
| traceData | TraceData | O | 0..1 | Trace requirements about the UE, only sent to AMF in the HPLMN or one of its equivalent PLMN(s) |  |
| cagData | CagData | O | 0..1 | Closed Access Group Data. Shall be absent if both - no CAG is subscribed for the serving PLMN and - an acknowledgement from the UE is not pending. | CAGFeature |
| stnSr | StnSr | O | 0..1 | This IE shall be present if the UE is subscribed to 5G SRVCC. When present, it indicates the STN-SR (Session Transfer Number for SRVCC) of the UE. |  |
| cMsisdn | CMsisdn | O | 0..1 | This IE shall be present if the UE is subscribed to 5G SRVCC. When present, it indicates the C-MSISDN (Correlation MSISDN) of the UE. |  |
| nbIoTUePriority | NbIoTUePriority | O | 0..1 | Indicates NB IoT UE priority which is used by the NG-RAN to prioritise resource allocation between UEs accessing via NB-IoT(see clause 5.31.17 of 3GPP TS 23.501 [2]). |  |
| nssaiInclusionAllowed | boolean | O | 0..1 | Indicates that the UE is allowed to include NSSAI in the RRC connection establishment in clear text for 3GPP access, as specified in clause 5.15.9 of 3GPP TS 23.501 [2] and clause 4.2.2.2.2 of 3GPP TS 23.502 [3]. true: indicates that NSSAI can be included in RRC connection establishment by the UE. false or absent: indicates that NSSAI cannot be included. |  |
| rgWirelineCharacteristics | RgWirelineCharacteristics | O | 0..1 | Indicates the RG Level Wireline Access Characteristics as specified in 3GPP TS 23.316 [37]. |  |
| ecRestrictionDataWb | EcRestrictionDataWb | O | 0..1 | Indicates Enhanced Coverage Restriction Data for WB-N1 mode. If absent, indicates Enhanced Coverage is not restricted for WB-N1 mode. |  |
| ecRestrictionDataNb | boolean | O | 0..1 | If present, this IE shall indicate whether Enhanced Coverage for NB-N1 mode is restricted or not. true: Enhanced Coverage for NB-N1 mode is restricted. false or absent: Enhanced Coverage for NB-N1 mode is allowed. |  |
| expectedUeBehaviourList | ExpectedUeBehaviourData | O | 0..1 | Indicates Expected UE Behaviour parameters associated with AMF(see clause 5.20 of 3GPP TS 23.501 [2] and clause 4.15.6.3 of 3GPP TS 23.502 [3]). This attribute is only applicable to the Nudm interface and shall not be included over the Nudr interface. |  |
| primaryRatRestrictions | array(RatType) | O | 0..N | List of RAT Types that are restricted for use as primary RAT in 5GC and EPC; see 3GPP TS 29.571 [7] (NOTE 2) Contains unique items |  |
| secondaryRatRestrictions | array(RatType) | O | 0..N | List of RAT Types that are restricted for use as secondary RAT in 5GC and EPC; see 3GPP TS 29.571 [7] (NOTE 2) Contains unique items |  |
| edrxParametersList | array(EdrxParameters) | O | 1..N | List of subscribed the extended idle mode DRX parameters (see clause 5.31.7.2.1 of 3GPP TS 23.501 [2]). |  |
| ptwParametersList | array(PtwParameters) | O | 1..N | List of subscribed the Paging Time Window parameters (see clause 5.31.7.2.1 of 3GPP TS 23.501 [2]). |  |
| iabOperationAllowed | boolean | O | 0..1 | Indicates that the UE is allowed for IAB operation as specified in 3GPP TS 23.501 [2]. true: indicates that the UE is allowed for IAB operation. false or absent: indicates that the UE is not allowed for IAB operation. |  |
| adjacentPlmnRestrictions | map(PlmnRestriction) | O | 1..N | A map (list of key-value pairs where PlmnId converted to string serves as key; see 3GPP TS 29.571 [7]) of PlmnRestrictions for adjacent PLMNs |  |
| wirelineForbiddenAreas | array(WirelineArea) | O | 0..N | List of forbidden areas for 5G-BRG/5G-CRG/FN-CRG (NOTE 6, NOTE 7) |  |
| wirelineServiceAreaRestriction | WirelineServiceAreaRestriction | O | 0..1 | Subscribed Service Area Restriction for 5G-BRG/5G-CRG/FN-CRG (NOTE 7) |  |
| pcfSelectionAssistanceInfos | array(PcfSelectionAssistanceInfo) | C | 1..N | List of combination of DNN and S-NSSAI that indicates that the same PCF needs to be selected by the AMF. (NOTE 5) |  |
| aerialUeSubInfo | AerialUeSubscriptionInfo | O | 0..1 | This IE shall contain the subscribed Aerial UE Subscription Information when present. |  |
| roamingRestrictions | RoamingRestrictions | O | 0..1 | This IE is used by Nudr_DataRepository API starting with 3GPP Rel-17 and onwards (see Table 5.2.3.3.1-3 in 3GPP TS 29.505 [10]). If present, this IE contains information on the roaming restrictions. If this IE is absent e.g. if the UDR is not provisioned with the serving PLMN ID, then the UDM may use the UE's home subscribed profile as the default profile to handle the roaming and SNPN scenarios. |  |
| remoteProvInd | boolean | O | 0..1 | Indicates whether the UE is allowed only for Remote Provisioning as specified in clause 5.30.2.10.3.2 of 3GPP TS 23.501 [2]. UE subscription either allows, or does not allow the UE to access the PLMN as the Onboarding Network using PLMN credentials. - false (default), or the attribute is absent: indicates that the UE is not restricted to the Onboarding only; - true: indicates that the UE is allowed only for the Onboarding. |  |
| timeSyncData | TimeSyncData | O | 0..1 | Contains subscription data for Access Stratum Time Synchronization (see 3GPP TS 23.502 [3] clause 4.28.2.1 and 3GPP TS 23.501 [2] clause 5.27.1.11). |  |
| NOTE 1: AccessAndMobilitySubscriptionData can be UE-individual data or shared data. UE-individual data take precedence over shared data unless treatment instructions associated to the shared data indicate otherwise. E.g.: When an attribute of type array is present but empty within UE-Individual data and present (with any cardinality) in shared data and no treatment instructions are applicable, the empty array takes precedence. Similarly, when a nullable attribute is present with value null within the individual data and present (with any value) in shared data and no treatment instructions are applicable, the null value takes precedence (i.e. for the concerned UE the attribute is considered absent). NOTE 2: If the primaryRatRestrictions and secondaryRatRestrictions attributes are supported by the sender, the sender shall include the list of RAT Types that are restricted, if any, in the ratRestrictions attribute, shall include the list of RAT Types that are restricted for use as primary RAT, if any, in the primaryRatRestrictions attribute and shall include the list of RAT Types that are restricted for use as secondary RAT, if any, in the secondaryRatRestrictions attribute. If the primaryRatRestrictions and secondaryRatRestrictions attributes are supported by the receiver, the receiver shall use the data in the primaryRatRestrictions attribute, if received, as the list of RAT Types that are restricted for use as primary RAT, and shall use the data in the secondaryRatRestrictions attribute, if received, as the list of RAT Types that are restricted for use as secondary RAT, otherwise the receiver shall use the data in the ratRestrictions attribute, if received, as the list of RAT Types that are restricted. If the secondaryRatRestictions attribute is included in the subscription profile, the content may be sent to MME during inter RAT handover from NR SA to EN-DC, for the purpose of adequate SGW selection at MME based on subscription profile, and to avoid allocating unnecessary resources for secondary RAT at EPC if it is restricted. NOTE 3: The AMF shall take responsibility to perform PDU session related actions subject to change of OdbPacketService, e.g. release existing PDU session. NOTE 4: The UDM shall ignore the content of sorInfo received on Nudr if "sorafRetrieval" is set to true. NOTE 5: If multiple PDU sessions to one DNN and S-NSSAI are established in EPS, the AMF shall select the same PCF by configuration or by using existing method, e.g. same PCF selection in usage monitoring. NOTE 6: When UE is only permitted in a small area, i.e. the UE is forbidden in most of TAIs in the PLMN, AreaCode (e.g. "All TAIs except xxx") may be used to avoid provisoning and transferring large number of TAIs. AeraCode may be used otherwise. NOTE 7: AreaCode for VPLMN(s) may be provisioned under agreement between operators. |  |  |  |  |  |

* * * Next Change * * * *

##### 6.1.6.2.xx	Type: TimeSyncData

Table 6.1.6.2.xx-1: Definition of type TimeSyncData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| authorized | boolean | M | 1 | Access Stratum Time Synchronization Service Authorization this IE shall be set as follows: - true: The service is authorized - false: The service is not authorized |
| uuTimeSyncErrBdgt | Uinteger | O | 0..1 | Indicates the Uu time synchronization error budget for the time synchronization service in units of nanoseconds. |
| tempVals | array(TemporalValidity) | O | 1..N | Contains start and stop times |
| coverageArea | array(Tai) | O | 1..N | List of TAIs |

* * * Next Change * * * *

# A.2	Nudm_SDM API

openapi: 3.0.0

**********text not shown for clarity****************

AccessAndMobilitySubscriptionData:

type: object

properties:

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

gpsis:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

hssGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

internalGroupIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

minItems: 1

sharedVnGroupDataIds:

description: A map(list of key-value pairs) where GroupId serves as key of SharedDataId

type: object

additionalProperties:

$ref: '#/components/schemas/SharedDataId'

minProperties: 1

subscribedUeAmbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AmbrRm'

nssai:

$ref: '#/components/schemas/Nssai'

ratRestrictions:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

uniqueItems: true

forbiddenAreas:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Area'

serviceAreaRestriction:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ServiceAreaRestriction'

coreNetworkTypeRestrictions:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/CoreNetworkType'

accessTypeRestrictions:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

maxItems: 2

rfspIndex:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RfspIndexRm'

subsRegTimer:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSecRm'

ueUsageType:

$ref: '#/components/schemas/UeUsageType'

mpsPriority:

$ref: '#/components/schemas/MpsPriorityIndicator'

mcsPriority:

$ref: '#/components/schemas/McsPriorityIndicator'

activeTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSecRm'

sorInfo:

$ref: '#/components/schemas/SorInfo'

sorInfoExpectInd:

type: boolean

sorafRetrieval:

type: boolean

default: false

sorUpdateIndicatorList:

type: array

items:

$ref: '#/components/schemas/SorUpdateIndicator'

minItems: 1

upuInfo:

$ref: '#/components/schemas/UpuInfo'

routingIndicator:

type: string

pattern: '^[0-9]{1,4}$'

micoAllowed:

$ref: '#/components/schemas/MicoAllowed'

sharedAmDataIds:

type: array

items:

$ref: '#/components/schemas/SharedDataId'

minItems: 1

odbPacketServices:

$ref: 'TS29571_CommonData.yaml#/components/schemas/OdbPacketServices'

subscribedDnnList:

type: array

items:

anyOf:

- $ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

- $ref: 'TS29571_CommonData.yaml#/components/schemas/WildcardDnn'

serviceGapTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

mdtUserConsent:

$ref: '#/components/schemas/MdtUserConsent'

mdtConfiguration:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MdtConfiguration'

traceData:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TraceData'

cagData:

$ref: '#/components/schemas/CagData'

stnSr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/StnSr'

cMsisdn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/CMsisdn'

nbIoTUePriority:

$ref: '#/components/schemas/NbIoTUePriority'

nssaiInclusionAllowed:

type: boolean

default: false

rgWirelineCharacteristics:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RgWirelineCharacteristics'

ecRestrictionDataWb:

$ref: '#/components/schemas/EcRestrictionDataWb'

ecRestrictionDataNb:

type: boolean

default: false

expectedUeBehaviourList:

$ref: '#/components/schemas/ExpectedUeBehaviourData'

primaryRatRestrictions:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

uniqueItems: true

secondaryRatRestrictions:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

uniqueItems: true

edrxParametersList:

type: array

items:

$ref: '#/components/schemas/EdrxParameters'

minItems: 1

ptwParametersList:

type: array

items:

$ref: '#/components/schemas/PtwParameters'

minItems: 1

iabOperationAllowed:

type: boolean

default: false

adjacentPlmnRestrictions:

description: A map (list of key-value pairs where PlmnId serves as key) of PlmnRestriction

type: object

additionalProperties:

$ref: '#/components/schemas/PlmnRestriction'

minProperties: 1

wirelineForbiddenAreas:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/WirelineArea'

wirelineServiceAreaRestriction:

$ref: 'TS29571_CommonData.yaml#/components/schemas/WirelineServiceAreaRestriction'

pcfSelectionAssistanceInfos:

type: array

items:

$ref: '#/components/schemas/PcfSelectionAssistanceInfo'

minItems: 1

aerialUeSubInfo:

$ref: '#/components/schemas/AerialUeSubscriptionInfo'

roamingRestrictions:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RoamingRestrictions'

remoteProvInd:

type: boolean

default: false

timeSyncData:

$ref: '#/components/schemas/TimeSyncData'

TimeSyncData:

type: object

required:

- authorized

properties:

authorized:

type: boolean

uuTimeSyncErrBdgt:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

tempVals:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/TemporalValidity'

minItems: 1

coverageArea:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

minItems: 1

**********text not shown for clarity****************

* * * End of Change * * * *