3GPP TSG-CT WG4 Meeting #114	C4-230448

# Athens, Greece, 27th Feb – 03rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.274 | CR | 2072 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Encoding of PGW FQDN IE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | RPCPSET |  |  |  |  |  | Date: |  |  | 2023-01-18 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The encoding of the PGW FQDN IE is incorrect: - the Type is wrongly encoded on 2 octets instead of 1 octet - no Instance field is defined in octet 4. See clause 8.2.1 for the definition of GTP-C IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The encoding of the PGW FQDN IE is corrected to comply with the encoding of all GTP-C IEs defined in clause 8.2.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interoperability issues preventing the use of the PGW FQDN IE (a receiver will decode the type in the first octet while the sender would encode the type in 2 octets). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.146 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 8.146	PGW FQDN

The PGW FQDN IE shall contain an FQDN representing the PGW Set or PGW. It shall be encoded as shown in Figure 8.146-1.

|  |  | Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 |  | 3 | 2 | 1 |  |
|  | 1 | Type = 215 (decimal) |  |  |  |  |  |  |  |  |  |
|  | 2 to 3 | Length = n |  |  |  |  |  |  |  |  |  |
|  | 4 | Spare |  |  |  |  | Instance |  |  |  |  |
|  | 5 to m | FQDN |  |  |  |  |  |  |  |  |  |
|  | (m+1) to (n+4) | These octet(s) is/are present only if explicitly specified |  |  |  |  |  |  |  |  |  |

Figure 8.146-1: PGW FQDN

FQDN encoding shall be identical to the encoding of a FQDN within a DNS message of clause 3.1 of IETF RFC 1035 [27] but excluding the trailing zero byte.

NOTE:	The FQDN field in the IE is not encoded as a dotted string as commonly used in DNS master zone files.

* * * End of Changes * * * *