3GPP TSG-CT WG1 Meeting #140	C1-230923

# Athens, Greece, 27 February – 3 March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.302 | CR | 0741 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Redefining SNPN list with trusted 5G Connectivity IE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CT1#139, the new SNPN List with trusted 5G Connectivity information element was introduced for the structure of IEEE Std 802.11 generic container. The new IE is type 6 IE of format TLV and contains a length of one octet while containing two lists: - One list for SNPN information list, having a length of one octet; and - Another list for GIN list, having a length of one octet. It is recommended to modify the new SNPN List with trusted 5G Connectivity information element to type 6 IE of format TLV-E with a length equal to 2 octets. Otherwise, there may be a risk to limit the amount of information if current and future releases require more. Two EN has been introduced for since the SNPN and NID are ot defined as they are defined in TS 24.501 and TS 24.502. There is no reason for this inconsistency and therefore the format of should be according to the formats in TS 24.502. Furthermore, there is a redundant and gerneral information about the format of each IE to subclause 11.2.2.1 of TS 24,007, which is not needed. If there is any need for that information, then it should not be a general information by specific type information of the format for that IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modified the length of SNPN List with trusted 5G Connectivity information element from 1 octet to 2 octets. - Removal of unnecessary text between the figures ad tables. - Aligning the SNPN ID and NID ID to the formats which are used according to TS 24.502 and removing the ENs. - Changed length of SNPN list and GIN list from one octet to two octets. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Too small of length for SNPN List with trsuted 5G Connectivity information element may result in not having enough room for two IEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | H.2.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************************ Next Change ************************

### H.2.4.7	SNPN List with trusted 5G Connectivity IE

The SNPN List with trusted 5G Connectivity information element is used by the network to indicate the SNPNs that can be selected from the WLAN. The SNPN List with trusted 5G Connectivity information element optionally includes per SNPN access information, per SNPN supported GINs, and a GIN list. The SNPN List with trusted 5G Connectivity information element is type 6 information element of format TLV-E according to 3GPP TS 24.007 [48] and is coded as shown in figure H.2.4.7-1.

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNPN List with trusted 5G Connectivity IE |  |  |  |  |  |  |  | octet 1 |
| Length of SNPN List with trusted 5G Connectivity value contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| SNPN information List |  |  |  |  |  |  |  | octet 4 octet d |
| GIN list |  |  |  |  |  |  |  | octet d+1* octet Z* |

Figure H.2.4.7-1: SNPN List with trusted 5G Connectivity information element

| Length of SNPN information list |  | octet 4 octet 5 |
| --- | --- | --- |
| SNPN information entry 1 |  | octet 6 octet b |
|  | .. | octet b+1* octet c-1* |
| SNPN information entry N |  | octet c* octet d* |

Figure H.2.4.7-2: SNPN information List

| Length of SNPN information entry | octet 6 octet 7 |
| --- | --- |
| SNPN identity | octet 8 octet 16 |
| SNPN access information | octet 17* octet 18* |
| Supported GINs | octet 19* octet b* |

Figure H.2.4.7-3: SNPN information entry

| 7 | 6 | 5 | 4 | 3 |  | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCC digit 2 |  |  |  |  | MCC digit 1 |  |  |  | octet 8 |
| MNC digit 3 |  |  |  |  | MCC digit 3 |  |  |  | octet 9 |
| MNC digit 2 |  |  |  |  | MNC digit 1 |  |  |  | octet 10 |
| NID value digit 1 |  |  |  |  | NID assignment mode |  |  |  | octet 11 |
| NID value digit 3 |  |  |  |  | NID value digit 2 |  |  |  | octet 12 |
| NID value digit 5 |  |  |  |  | NID value digit 4 |  |  |  | octet 13 |
| NID value digit 7 |  |  |  |  | NID value digit 6 |  |  |  | octet 14 |
| NID value digit 9 |  |  |  |  | NID value digit 8 |  |  |  | octet 15 |
| Spare |  |  |  |  | NID value digit 10 |  |  |  | octet 16 |

Figure H.2.4.7-4: SNPN identity

| 7 | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of SNPN access information |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 17 |
| Spare |  | Spare |  | Spare |  | Spare |  | Spare |  | OB |  | CHWC |  | CH | octet 18 |

Figure H.2.4.7-5: SNPN access information

| 7 | 6 | 5 | 4 | 3 |  | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of Supported GINs value |  |  |  |  |  |  |  |  | octet 19 |
| G(8) | G(7) | G(6) | G(5) |  | G(4) | G(3) | G(2) | G(1) | octet 20* |
| .. |  |  |  |  |  |  |  |  | octet 21* octet b-1* |
| G(n) | G(n-1) | G(n-2) | G(n-3) |  | G(n-4) | G(n-5) | G(n-6) | G(n-7) | octet b* |

Figure H.2.4.7-6: Supported GINs

:

| Length of GIN list | octet d+1 octet d+2 |
| --- | --- |
| GIN information entry 1 | octet d+3 octet e |
| .. | octet e+1* octet f-1* |
| GIN information entry N | octet f* octet Z* |

Figure H.2.4.7-7: GIN list

:

| 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of GIN information entry |  |  |  |  |  |  |  | octet d+3 octet d+4 |  |
| MCC digit 2 |  |  |  | MCC digit 1 |  |  |  | octet d+5 |  |
| MNC digit 3 |  |  |  | MCC digit 3 |  |  |  | octet d+6 |  |
| MNC digit 2 |  |  |  | MNC digit 1 |  |  |  | octet d+7 |  |
| NID list |  |  |  |  |  |  |  | octet d+8 octet e |  |

Figure H.2.4.7-8: GIN information entry

| Length of NID list | octet d+8 octet d+9 |
| --- | --- |
| NID identity 1 | octet d+10 octet d+15 |
| .. | octet d+16* octet e-1* |
| NID identity N | octet e* octet e+5* |

Figure H.2.4.7-9: NID list

| 7 | 6 | 5 | 4 | 3 |  | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NID value digit 1 |  |  |  |  | NID assignment mode |  |  |  | octet d+10 |
| NID value digit 3 |  |  |  |  | NID value digit 2 |  |  |  | octet d+11 |
| NID value digit 5 |  |  |  |  | NID value digit 4 |  |  |  | octet d+12 |
| NID value digit 7 |  |  |  |  | NID value digit 6 |  |  |  | octet d+13 |
| NID value digit 9 |  |  |  |  | NID value digit 8 |  |  |  | octet d+14 |
| Spare |  |  |  |  | NID value digit 10 |  |  |  | octet d+15 |

Figure H.2.4.7-10: NID identity

Table H.2.4.7-1: SNPN List with trusted 5G Connectivity information element

SNPN List with trusted 5G Connectivity information element (octets 1 to Z)

SNPN identity (octets 8 to 16) MCC (octet 8, octet 9 bits 0 to 3) or (octet d+5, octet d+6 bits 0 to 3) The MCC (Mobile country code) field is coded as in ITU-T Recommendation E.212 [63], Annex A MNC (octet 10, octet 9 bits 4 to 7) or (octet d+7, octet d+6 bits 4 to 7). The coding of the MNC (Mobile network code) field is the responsibility of each administration but BCD coding shall be used. The MNC shall consist of 2 or 3 digits. If a network operator decides to use only two digits in the MNC, bits 4 to 7 of octet 9 or octet d+6 shall be coded as "1111". NID assignment mode (octet 11 bits 0 to 3) or (octet d+10 bits 0 to 3) This field contains the binary encoding of the assignment mode of the NID as defined in 3GPP TS 23.003 [3]. NID value (octet 11 bits 4 to 7, octets 12 to 15, octet 16 bits 0 to 3) or (octet d+10 bits 4 to 7, octets d+11 to d+14, octet d+15 bits 0 to 3) This field contains the binary encoding of each hexadecimal digit of the NID value as defined in 3GPP TS 23.003 [3] Bits 4 to 7 of octet 16 or Bits 4 to 7 of octet d+15 are spare and shall be coded as zero.

SNPN access information (octets 17 to 18) CH (octet 18 bit 0) This bit indicates whether the SNPN supports access using credentials from a credentials holder. 0 access using credentials from a credentials holder supported. 1 access using credentials from a credentials holder not supported. CHWC (octet 18 bit 1) This bit indicates whether the SNPN allows registration attempts with credentials from a credentials holder from UEs that are not explicitly configured to select the SNPN. 0 registration attempts from UEs without configuration to select the SNPN not allowed. 1 registration attempts from UEs without configuration to select the SNPN allowed. OB (octet 18 bit 2) This bit indicates whether the SNPN allows onboarding. 0 onboarding is not allowed. 1 onboarding is allowed. octet 18 bits 3 to 7 are spare.

Supported GINs value (octets 19 to b) G(1) to G(n) is a bitmap indicating whether the SNPN identity in the SNPN information entry supports a corresponding GIN in the GIN list. G(1) bit corresponds to GIN index value one in the GIN list, G(2) corresponds to GIN index value two in the GIN list and so on. A bit set to 1 indicates that the GIN is supported by the SNPN and a bit set to 0 indicates that the GIN is not supported by the SNPN. The length of the Supported GINs value field in octets is the ceiling of the number of GINs in the GIN list divided by 8. The filling bits, if any, are set to 0.

GIN list (octets d+1 to Z) The GIN list contains one or more GIN information entries. Each GIN information entry contains either one GIN, which is identified by a PLMN ID (MCC, MNC) and a single NID (NID list with a single NID entry), or multiple GINs that share the same PLMN ID with each GIN identified by the shared PLMN ID and a NID entry in the NID list. The GIN index m is defined as d1+d2+…+d(n-1)+i for the GIN included in the n-th entry of the GIN list and the i-th entry of its corresponding GIN information entry, where d(k) is the number of GIN index values used in the k-th GIN information entry.

************************ Next Change ************************