3GPP TSG-CT Meeting #99	CP-230305

# Rotterdam, Netherlands, 20th – 21st March 2023					 (revision of C3-230760, 0603)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.122 | CR | 0662 | rev | 2 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of the procedure when the NEF reject the AF update request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile Communications Group Co.,Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | China Mobile Communications Group Co.,Ltd. |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2023-03-12 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is definded in 3GPP TS 23.502 CR 3659 when the NEF reject the Nnef_AFsessionWithQoS_Update request while the NEF determined not to invoke the TSCTSF for the initial Nnef_AFsessionWithQoS_Create request and the NEF need to invoke the TSCTSF during the update procedure as defined in clause 4.15.6.6a of 3GPP TS 23.502.: - Problem detail is added indicating the reason of rejection; |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the response sent by the NEF during the rejection, including: - Add a problem detail to indicate the reason of rejection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with stage 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.14.3.3.3.2, 5.14.3.3.3.3, 5.14.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact the API files. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Changes from C3-230760: - Remove the "NOTE b" part in the table 5.14.3.3.3.2-1 and table 5.14.3.3.3.3-1 which clashed with CR CR#0656 of TS 29.122. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

5.14.3.3.3.2	PUT

The PUT method allows changing the service information of an active subscription. The properties "ueIpv4Addr" or "ueIpv6Addr" shall remain unchanged from previously provided value.

This method shall support request and response data structures, and response codes, as specified in the table 5.14.3.3.3.2-1.

Table 5.14.3.3.3.2-1: Data structures supported by the PUT request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | AsSessionWithQoSSubscription | 1 | Set up AS session with required QoS. |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  | AsSessionWithQoSSubscription | 1 | 200 OK | The subscription was modified successfully. The SCEF shall return an updated subscription in the response payload body. |
|  | none |  | 204 No Content | The subscription was updated successfully. |
|  | none |  | 307 Temporary Redirect | Temporary redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SCEF. Redirection handling is described in clause 5.2.10. |
|  | none |  | 308 Permanent Redirect | Permanent redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SCEF. Redirection handling is described in clause 5.2.10. |
|  | ProblemDetails | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 also apply. |  |  |  |  |

Table 5.14.3.3.3.2-2: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SCEF. |

Table 5.14.3.3.3.2-3: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SCEF. |

* * * Next Change * * * *

5.14.3.3.3.3	PATCH

The PATCH method allows to change the service information of an active subscription.

This method shall support request and response data structures, and response codes, as specified in the table 5.14.3.3.3.3-1.

Table 5.14.3.3.3.3-1: Data structures supported by the PATCH request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | AsSessionWithQoSSubscriptionPatch | 1 | Partial update of an AS session with required QoS. |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  | AsSessionWithQoSSubscription | 1 | 200 OK | The subscription was modified successfully. The SCEF shall return an updated subscription in the response payload body. |
|  | none |  | 204 No Content | The subscription was modified successfully. |
|  | none |  | 307 Temporary Redirect | Temporary redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SCEF. Redirection handling is described in clause 5.2.10. |
|  | none |  | 308 Permanent Redirect | Permanent redirection, during subscription modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SCEF. Redirection handling is described in clause 5.2.10. |
|  | ProblemDetails | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.6-1 also apply. |  |  |  |  |

Table 5.14.3.3.3.3-2: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SCEF. |

Table 5.14.3.3.3.3-3: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SCEF. |

* * * Next Change * * * *

5.14.5.3	Application Errors

The application errors defined for AsSessionWithQoS API are listed in table 5.14.5.3-1.

Table 5.14.5.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |
| INVALID_SESSION_UPDATE | 403 Forbidden | Indicates that the session is not allowed to be updated since one or more of the received parameters can not be served in current session. The AF can retry with a new session. |  |

* * * End of Changes * * * *