3GPP TSG-CT #99	CP-230319

# Rotterdam, NL, 20  – 21 March 2023									  				 (was CP-230290)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.122 | CR | 0952 | rev | 13 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Signal level enhanced network selection requirements for PLMN selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Ericsson, LG Electronics, InterDigital, Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Deutsche Telekom, BT, China Mobile, KDDI, KPN, Telecom Italia, Vodafone, Ericsson, Huawei, HiSilicon, IDEMIA, InterDigital, NEC, MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Work item code: | SENSE |  |  |  |  |  | Date: |  |  | 2023-03-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Regarding SA1 requirements in 3GPP TS 22.011, SENSE work requires that for UEs supporting any, or a combination, of NB-IoT, GERAN EC-GSM-IoT and Category M1 or M2 of E-UTRA the “Operator controlled signal threshold per access technology” shall be considered for the PLMN selection: For UEs supporting any, or a combination, of NB-IoT, GERAN EC-GSM-IoT [18] and Category M1 or M2 of E-UTRA [17], the 5G system shall support a mechanism to have an Operator controlled signal threshold per access technology on the USIM to be used for network selection. The signal threshold is specific for a certain Access Technology and shall apply to all PLMNs with the corresponding access technology combinations. …. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In accordance with 3GPP TS 22.011, add SENSE requirements to PLMN selection to introduce the “Operator controlled signal threshold” per access technology. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The “Operator controlled signal threshold per access technology” is not considered in PLMN selection for UEs supporting any, or a combination, of NB-IoT, GERAN EC-GSM-IoT and Category M1 or M2 of E-UTRAN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | CR to 3GPP TS 31.102 to cover USIM parameters is submitted. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev13: Further clarification on applicability of the feature SENSE, Introduce NOTE 2. |  |  |  |  |  |  |  |  |

************************* Change *******************

## 3.X	Signal level enhanced network selection

Signal level enhanced network selection applies only to NB-IoT, GERAN EC-GSM-IoT and Category M1 or M2 of E-UTRA. An MS supporting any, or a combination, of NB-IoT, GERAN EC-GSM-IoT and Category M1 or M2 of E-UTRA shall apply signal level enhanced network selection if the following conditions are fulfilled:

1)	The MS is in automatic PLMN selection mode;

2)	The MS supports the "Operator controlled signal threshold per access technology"  as specified in 3GPP TS 22.011 [19];

3)	The MS is configured for using signal level enhanced network selection as specified in 3GPP TS 24.368 [50]; and

4)	The "Operator controlled signal threshold per access technology" is configured in the USIM.

NOTE 1:	The usage of the "Operator controlled signal threshold per access technology" is intended only for IoT stationary devices (see 3GPP TS 22.011 [19]).

NOTE 2:	"Operator controlled signal threshold per access technology" is not expected to be supported by non-IoT devices.

The MS can be configured with an "Operator controlled signal threshold per access technology" stored in the USIM (see 3GPP TS 31.102 [40]) consisting of one or more entries, each containing:

a)	a home operator controlled signal threshold; and

b)	an access technology.

The "Operator controlled signal threshold per access technology" is specific for a certain access technology and when applicable, applies to all allowable PLMNs with the corresponding access technology combination.

Editor's note (WI SENSE, CR 0952):	It is FFS whether HPLMN can use CP-SOR procedure to update the signal level enhanced network selection in the USIM.