3GPP TSG-CT WG1 Meeting #104	C1-172470

# Zhangjiajie, P.R of China, 15-19 May 2017

Source:	Qualcomm Incorporated

Title:	Pseudo-CR on Registration over a non-3GPP access

Spec:	3GPP TR 24.890 v0.1.0

Agenda item:	15.2.1.5

Document for:	Agreement

1. Introduction

Stage 2 has agreed on the principles of registration for non-3GPP access. CT1 should start the dicussion on the stage 3 aspects.

2. Reason for Change

In 5GS, the UE may be registered over the non-3GPP access with the same PLMN as over the 3GPP access. In this case, the registation is performed with the same AMF over both accesses and it is associated with the same security context. This is a new level of integration between non-3GPP and 3GPP access with respect to EPC. It needs to be clarified how this integration works at the stage 3 level.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 24.890 v0.1.0.

* * * First Change * * * *

# 9	Access to the 5G core network via non-3GPP access networks

## 9.x.y	Registration over on-3GPP access

### 9.x.y.1	General

The UE registers separately for 3GPP access and non-3GPP access, respectively.

- The UE shall not perform periodic registration update over a non-3GPP access.

Editor's note:	It is FFS whether a registration timer is used for the registration over non-3GPP access

- The UE cannot be paged over a non-3GPP access.

### 9.x.y.2	Registration over non-3GPP access

When the UE is registered with a PLMN over a non-3GPP access, the AMF and the UE maintain:

- -	Registration state and state machine over non-3GPP access;

- -	Security context;

- -	Temporary UE identity; and

- -	Registration area for non-3GPP access, which is associated with a fixed well-known N3GPP TAI.

Editor’s note: What information the N3IWF maintains for a registered UE is FFS.

### 9.x.y.3	Registration over non-3GPP access and 3GPP access

- When the UE is registered with a PLMN over a non-3GPP access and over a 3GPP access, the AMF and the UE maintain:

- -	Separate registration states and separate instances of state machines over 3GPP access and non-3GPP access, respectively;

Editor’s note: The state machine over non-3GPP access is FFS. All registration sub-states and transitions in the state machine for the 3GPP access are not applicable to the non-3GPP access and vice-versa.

- -	Separate registration areas; and

- -	Same security context and Temporary UE identity, only if the UE is registered with the same PLMN over both accesses. Otherwise, the security contexts and the Temporary UE identity shall be separate.

Editor’s note: To what extent the security context is the same when the UE is registered with the same PLMN over both accesses is FFS.

- When the UE initiates registration over a non-3GPP access to a PLMN for which it has a valid Temporary UE identity, the UE shall use this Temporary UE identity during the registration. When the UE is currently registered with the same PLMN over a 3GPP access, the Temporary UE identity is considered as valid.

Editor’s note: Other conditions for considering a Temporary UE identity as valid are FFS.

### 9.x.y.4	De-registration over non-3GPP access

The UE and the AMF may initiate de-registration for a non-3GPP access by:

- -	Initiating a de-registration request over a non-3GPP access; or

- -	Initiating a de-registration request over a 3GPP access with an indication that it applies to all accesses.

When the UE moves all the PDU sessions over a non-3GPP access to a 3GPP access, the UE and the AMF need not initiate de-registration over the non-3GPP access.

* * * End of Changes * * * *