3GPP TSG-CT1 Meeting #116	C1-192085

# Xi’an, China, 8th Apr 2019 - 12th Apr 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 0944 | rev | - | Current version: | 16.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of 5G NAS security contexts |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-CT |  |  |  |  |  | Date: |  |  | 2019-03-28 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 33.501 subclause 6.9.2.3.4 specifies that the UE shall verify the NAS COUNT and NAS MAC when receiving NASC (NAS Container) in the HO Command: "If the UE also receives a NASC (NAS Container) in the HO Command message, the UE shall update its NAS security context as follows: NOTE 1: The purpose of this NASC could be compared to a NAS SMC message. - The UE shall verify the freshness of the downlink NAS COUNT in the NASC. - If the NASC indicates a new KAMF has been calculated (i.e., K_AMF_change_flag is one), - The UE shall compute the horizontally derived KAMF using the KAMF from the current 5G NAS security context identified by the ngKSI included in the NASC and the downlink NAS COUNT in the NASC, as specified in Annex A.13. - The UE shall assign the ngKSI included in the NASC to the ngKSI of the new derived KAMF. The UE shall further configure NAS security based on the horizontally derived KAMF and the selected NAS security algorithms in the NASC. - The UE shall further verify the NAS MAC in the NASC as described in Clause 6.9.2.3.3 and if the verification is successful, the UE shall further set the NAS COUNTs to zero. - If KAMF change is not indicated, - If the verification is successful, the UE shall configure the NAS security based on the parameters included in the NASC but shall not set the NAS COUNTs to zero. - The UE shall verify the NAS MAC in the NASC. - The UE shall further set the downlink NAS COUNT value of the currently active NAS security context to the received downlink NAS COUNT value in the NASC." However, TS 24.501 subclause 4.4.2.3 does not specify that the UE shall verify the NAS COUNT and MAC when receiving an Intra N1 mode NAS transparent container during N1 mode to N1 mode handover. Please note that in such error case the NAS is not able to create a new security context and RRC has no updated AS keys for encryption & ciphering of the RRCReconfigurationComplete message. It is therefore proposed in this CR to update TS 24.501 subclause 4.4.2.2, 4.4.2.3 and 4.4.2.4 to inform RRC once an invalid NAS transparent container is received so that RRC can perform appropriate error handling (e.g. initiating the RRC connection re-establishment procedure on the source cell). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update subclause 4.4.2.2, 4.4.2.3, 4.4.2.4 to include the needed validation procedures related to the received specific NAS transparent container according to the SA4 requirements. - Update subclause 9.8 to include the MAC header in the Intra N1 mode NAS transparent container and in the S1 mode to N1 mode NAS transparent container. - Update subclause 9.10 to include the Sequence Number in the Intra N1 mode NAS transparent container, in the S1 mode to N1 mode NAS transparent container, and in the N1 mode to S1 mode NAS transparent container. Interoperability impact analysis The changes in this CR are backwards compatible |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Integrity of the NAS transparent container during N1 mode to N1 mode handover, during inter-system change from S1 mode to N1 mode in 5GMM-CONNECTED mode, or inter-system change from N1 mode to S1 mode in 5GMM-CONNECTED mode is not verified as required according to the SA3 requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.2.2, 4.4.2.3, 4.4.3.3, 9.8 and 9.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS, TS ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** First change ***

#### 4.4.2.2	Establishment of a mapped 5G NAS security context during inter-system change from S1 mode to N1 mode in 5GMM-CONNECTED mode

In order for the UE operating in single-registration mode to derive a mapped 5G NAS security context for an inter-system change from S1 mode to N1 mode in 5GMM-CONNECTED mode, the AMF shall construct a mapped 5G NAS security context from the EPS security context received from the source MME as indicated in 3GPP TS 33.501 [24]. The AMF shall select the 5G NAS security algorithms and derive the 5G NAS keys (i.e. KNASenc and KNASint). The AMF shall define an ngKSI for the newly derived K'AMF key such that the value field is taken from the eKSI of the KASME key and the type field is set to indicate a mapped security context and associate this ngKSI with the newly created mapped 5G NAS security context. The AMF shall then include the message authentication code, selected NAS algorithms, NCC, NAS sequence number, replayed UE security capabilities and generated ngKSI in the S1 mode to N1 mode NAS transparent container IE (see subclause 9.11.2.9).

When the UE operating in single-registration mode receives the command to perform inter-system change to N1 mode in 5GMM-CONNECTED mode, the UE shall derive a mapped K'AMF, as indicated in 3GPP TS 33.501 [24], using the K'ASME from the EPS security context. Furthermore, the UE shall also derive the 5G NAS keys from the mapped K'AMF using the selected NAS algorithm identifiers included in the S1 mode to N1 mode NAS transparent container IE and associate this mapped 5G NAS security context with the ngKSI value received. The UE shall then verify the received NAS MAC. In case the received NAS MAC is not verified successfully (see subclause 4.4.3.3) the UE shall delete the new mapped 5G NAS security context and inform RRC that the configuration received in the S1 mode to N1 mode NAS transparent container is invalid.

When the UE operating in single-registration mode has a PDN connection for emergency bearer services and has no current EPS security context, the AMF shall set 5G-IA0 and 5G-EA0 as the selected 5G NAS security algorithms in the S1 mode to N1 mode NAS transparent container IE. The AMF shall create a locally generated K'AMF. The AMF shall set the ngKSI value of the associated security context to "000" and the type of security context flag to "mapped security context" in the S1 mode to N1 mode NAS transparent container IE.

When the UE operating in single-registration mode receives the command to perform inter-system change to N1 mode in 5GMM-CONNECTED mode (see 3GPP TS 38.331 [30]) and has a PDN connection for emergency bearer services, if 5G-IA0 and 5G-EA0 as the selected 5G NAS security algorithms are included in the S1 mode to N1 mode NAS transparent container IE, the UE shall create a locally generated K'AMF. Furthermore, the UE shall set the ngKSI value of the associated security context to the KSI value received.

If the inter-system change from S1 mode to N1 mode in 5GMM-CONNECTED mode is not completed successfully, the AMF and the UE operating in single-registration mode shall delete the new mapped 5G NAS security context.

*** Next change ***

#### 4.4.2.3	Establishment of a 5G NAS security context during N1 mode to N1 mode handover

During an N1 mode to N1 mode handover, the target AMF may derive a new 5G NAS security context for which the target AMF creates a new 5G NAS security context as indicated in 3GPP TS 33.501 [24].

When a new 5G NAS security context is derived using the same KAMF, the target AMF includes the 8 least significant bits of the downlink NAS COUNT in the Intra N1 mode NAS transparent container, and indicates that a new KAMF  shall not be derived (see subclause 9.11.2.6). The AMF shall increment the downlink NAS COUNT by one after creating the Intra N1 mode NAS transparent container.

When a new 5G NAS security context is created from a new KAMF, the target AMF includes the 8 least significant bits of the downlink NAS COUNT in the Intra N1 mode NAS transparent container and indicates that a new KAMF shall be derived (see subclause 9.11.2.6). The AMF shall then set both the uplink and downlink NAS COUNT counters of this 5G NAS security context to zero. The AMF shall increment the downlink NAS COUNT by one after creating the Intra N1 mode NAS transparent container.

The target AMF also includes the ngKSI with the same value as the ngKSI currently being used with the UE, the message authentication code, and the selected NAS algorithms in the Intra N1 mode NAS transparent container.

When the UE receives a command to perform handover to NG-RAN including an Intra N1 mode NAS transparent container (see subclause 9.11.2.6), the UE derives a new 5G NAS security context as described in 3GPP TS 33.501 [24]. When the Intra N1 mode NAS transparent container indicates that a new KAMF needs to be derived, the UE shall set both the downlink NAS COUNT and uplink NAS COUNT to zero after creating the new 5G NAS security context.

If the received Intra N1 mode NAS transparent container does not have a valid NAS COUNT (see subclause 4.4.3.2) or the received NAS MAC is not verified successfully (see subclause 4.4.3.3) the UE shall apply the 5G NAS security context in use before receiving the handover command, and inform RRC that the configuration received in the Intra N1 mode NAS transparent container is invalid.

NOTE:	During N1 mode to N1 mode handover, the Intra N1 mode NAS transparent container (see subclause 9.11.2.6) is equivalent to sending a SECURITY MODE COMMAND message to the UE in order to derive and use a new 5G NAS security context, optionally created with a new KAMF.

*** Next change ***

#### 4.4.2.4	Establishment of an EPS security context during inter-system change from N1 mode to S1 mode in 5GMM-CONNECTED mode

In order for the UE operating in single-registration mode to derive a mapped EPS security context for an inter-system change from N1 mode to S1 mode in 5GMM-CONNECTED mode, the AMF shall prepare a mapped EPS security context for the target MME as indicated in 3GPP TS 33.501 [24].

The AMF shall derive a K'ASME using the KAMF key and the downlink NAS COUNT of the current 5G security context, include the corresponding NAS sequence number in the N1 mode to S1 mode NAS transparent container (see subclause 9.11.2.7) and then increments its stored downlink NAS COUNT value by one. The AMF shall select the NAS algorithms identifiers to be used in the target MME after the inter-system change from N1 mode to S1 mode in 5GMM-CONNECTED mode, for encryption and integrity protection. The uplink and downlink NAS COUNT associated with the newly derived K'ASME key are set to 0. The eKSI for the newly derived K'ASME key shall be defined such as the value field is taken from the ngKSI and the type field is set to indicate a mapped security context.

When the UE operating in single-registration mode receives a command to perform inter-system change from N1 mode to S1 mode in 5GMM-CONNECTED mode including the N1 mode to S1 mode NAS transparent container for handover to E-UTRAN (see 3GPP TS 24.301 [15]), the UE shall derive the mapped EPS security context, i.e. derive K'ASME from KAMF using a downlink NAS COUNT based on the NAS sequence number received in the N1 mode to S1 mode NAS transparent container (see subclause 9.11.2.7) as described in 3GPP TS 33.501 [24]. The eKSI for the newly derived K'ASME key is defined such that the value field is taken from the ngKSI and the type field is set to indicate a mapped security context. The UE shall also derive the NAS keys as specified in 3GPP TS 33.401 [23A] using the EPS NAS security algorithms identifiers that are stored in the UE's 5G NAS security context.

If the received N1 mode to S1 mode NAS transparent container does not have a valid NAS COUNT (see subclause 4.4.3.2) the UE shall delete the new mapped EPS security context and inform RRC that the configuration received in the N1 mode to S1 mode NAS transparent container is invalid.

If the inter-system change from N1 mode to S1 mode in 5GMM-CONNECTED mode is not completed successfully, the AMF and the UE shall delete the new mapped EPS security context.

*** Next change ***

#### 4.4.3.3	Integrity protection and verification

The sender shall use its locally stored NAS COUNT as input to the integrity protection algorithm.

The receiver shall use the NAS sequence number included in the received message and an estimate for the NAS overflow counter as defined in subclause 4.4.3.1 to form the NAS COUNT input to the integrity verification algorithm.

The algorithm to calculate the integrity protection information is specified in 3GPP TS 33.501 [24], and in case of the:

- SECURITY PROTECTED 5GS NAS MESSAGE message, the integrity protection shall include octet 7 to n, i.e. the Sequence number IE and the NAS message IE.

- Intra N1 mode NAS transparent container, the integrity protection shall include octet 7 to 9.

- S1 mode to N1 mode NAS transparent container, the integrity protection shall include octet 7 to 12.

In addition to the data that is to be integrity protected, the BEARER ID, DIRECTION bit, NAS COUNT and 5G NAS integrity key are input to the integrity protection algorithm. These parameters are described in 3GPP TS 33.501 [24].

After successful integrity protection validation, the receiver shall update its corresponding locally stored NAS COUNT with the value of the estimated NAS COUNT for this NAS message.

Integrity verification is not applicable when 5G-IA0 is used.

*** Next change ***

## 9.8	Message authentication code

The message authentication code (MAC) information element contains the integrity protection information for the message. The MAC IE shall be included in the SECURITY PROTECTED 5GS NAS MESSAGE message if a valid 5G NAS security context exists and security functions are started.

The message authentication code (MAC) is also included in the in the Intra N1 mode NAS transparent container and in the S1 mode to N1 mode NAS transparent container.

The usage of MAC is specified in subclause 4.4.3.3.

*** Next change ***

## 9.10	Sequence number

This IE includes the NAS message sequence number (SN) which consists of the eight least significant bits of the NAS COUNT for a SECURITY PROTECTED 5GS NAS MESSAGE message.

The NAS message sequence number (SN) with the eight least significant bits of the NAS COUNT is also included in the in the Intra N1 mode NAS transparent container and in the S1 mode to N1 mode NAS transparent container.

The usage of SN is specified in subclause 4.4.3.

*** End change ***