3GPP TSG-CT WG1 Meeting #122-e	C1-200765

# Electronic meeting, 20-28 February 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.041 | CR | 211 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of ePWS message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | ePWS |  |  |  |  |  | Date: |  |  | 2020.02.17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This specification is stage 2 specification for broadcast of warning message. Threrefore, to avoid confusion, it has to be clarified how can handle the concurrent message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified how to handle the concurrent broadcast of warning message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If it is not approved, the other working group which refer to this specification has confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 9.2.16	WRITE-REPLACE-WARNING-REQUEST Request/Indication

| PARAMETER | REFERENCE | PRESENCE |
| --- | --- | --- |
| Message Type | 9.3.28 | M |
| Message Identifier | 9.3.1 | M |
| Serial-Number | 9.3.3 | M |
| Repetition-Period E-UTRAN | 9.3.36 | M |
| No-of-Broadcasts-Requested | 9.3.9 | M |
| List of TAIs | 9.3.29 | O |
| Warning Area List | 9.3.30 | O |
| Extended Repetition-Period | 9.3.37 | O |
| Warning-Type | 9.3.24 | O |
| Warning-Security-Information | 9.3.25 | O |
| Data Coding Scheme | 9.3.18 | O (NOTE) |
| Warning Message Content E-UTRAN | 9.3.35 | O |
| OMC ID | 9.3.31 | O |
| Concurrent Warning Message Indicator | 9.3.32 | O |
| Send Write-Replace-Warning-Indication | 9.3.39 | O |
| Global eNB ID | 9.3.46 | O |
| List of NG-RAN TAIs | 9.3.54 | O |
| Warning Area List NG-RAN | 9.3.55 | O |
| Warning Message Content NG-RAN | 9.3.51 | O |
| Global RAN Node ID | 9.3.53 | O |
| RAT Selector NG-RAN | 9.3.56 | O |
| Warning Area Coordinates | 9.3.63 | O |
| NOTE: The Data Coding Scheme IE is not required for ETWS primary notification but it is mandatory for ETWS secondary notification and CMAS warning messages when Warning Message Content E-UTRAN IE is present. |  |  |

This primitive is sent by the CBC to the MME or to the PWS-IWF.

The WRITE-REPLACE-WARNING-REQUEST Request/Indication primitive is to request the MME and the AMF to start or overwrite a warning message broadcast. The response to this Request/Indication primitive is sent by the MME or the AMF in a corresponding WRITE-REPLACE-WARNING-CONFIRM primitive.

If the message is sent to the MME, then the List of TAIs IE, the Warning Area List IE, the Warning Message Content E-UTRAN IE and the Global eNB ID IE may be used and the List of NG-RAN TAIs IE, the Warning Area List NG-RAN IE, the Warning Message Content NG-RAN IE, the Global RAN Node ID IE and the RAT Selector NG-RAN IE shall not be used.

If the message is sent to the PWS-IWF, then the List of NG-RAN TAIs IE, the Warning Area List NG-RAN IE, the Warning Message Content NG-RAN IE, the Global RAN Node ID IE and the RAT Selector NG-RAN IE may be used and the List of TAIs IE, the Warning Area List IE, the Repetition-Period E-UTRAN IE, the Extended Repetition Period IE, the Warning Message Content E-UTRAN IE, and the Global eNB ID IE shall not be used. The WRITE-REPLACE-WARNING-REQUEST Request/Indication primitive shall then be forwarded to the AMF.

The Concurrent Warning Message Indicator is required based on requirement of some operators or regulations of some countries.

NOTE:	For ETWS Primary Notification, the Repetition Period IE and the Number of Broadcasts Requested IE are ignored by eNB if included in WRITE-REPLACE-WARNING-REQUEST message (see 3GPP TS 36.413 [34]).