3GPP TSG-CT WG1 Meeting #122	C1-200893

# Electronic meeting, 20-28 February 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 1701 | rev | 3 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancement on CPSR for CIoT CP data transport |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Vodafone, ZTE, China Mobile, China Telecom, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2020-02-06 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As discussed in disc paper C1-196336, following proposals were provided to consider further enhancements on the current CPSR message in order to shorten the message overhead as far as possible, to improve the UE battery life and signaling efficiency: - The CPSR message was defined as a non-standard L3 NAS message with shortest message header. - Similar as done for 4G SR message, a new dedicated security header type is defined for CPSR message only. Hence, the message ID is not needed. This saves 1 octet. - Similar as done for 4G SR message, the ngKSI and Sequence number is merged into one octet. - The security protection was directly applied, this saves 2 octets (double EPD + security header type is avoided). Furthermore, based on SA3 reply LS (C1-198439/S3-193715), SA3 prefers ngKSI needs to be included in the CPSR message and also strongly recommand that the short MAC cannot be used. Hence, full MAC will be used. With using full MAC, comparing to the current CPSR message coding, this proposal can always save 3 octets message overhead in all cases. NOTE: As CPSR message shall always be security protected, hence it is reasonable to only cover security protected message for compare. Cases; Security protected message overhead of the current CPSR; Security protected message overhead of this proposal; Octets saved. Best cases; 14; 11; 3. Worst cases; 29; 26; 3. Reason for change for rev#2: - The Control plane service type IE was remained in the CPSR message coding. - Furthermore, with this enhancement proposal, following two ENs can be removed: "Editor's note: Whether other optimizations besides using a new EPD can be used in order to further reduce the message header of the message is FFS. Editor's note: Whether Control plane service type IE can be removed is FFS." |
| Summary of change: |  | 1. It proposes to define the CPSR message as a non-standard L3 NAS message with shortest message overhead to improve the CIoT device battery life and signaling efficiency. 2. Two related ENs are deleted. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For (in)frequent CIoT small data transport over CP, the current CPSR message coding will take more message overhead which impacts the CIoT device battery life and signaling efficiency. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.3.3, 4.4.3.4, 4.4.6, 8.2.30.1, 9.1.1, 9.3, 9.7, 9.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1 was postponed in CT1#121 Nov. 2019 meeting. Rev 2 is to keep the Control plane service type IE in the CPSR message coding and to remove to two related ENs. Rev 3 is to make the Sequence number IE as one octet and to make the ngKSI IE has falf octet. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

4.4.3.3	Integrity protection and verification

The sender shall use its locally stored NAS COUNT as input to the integrity protection algorithm.

The receiver shall use the NAS sequence number included in the received message (or estimated from the 5 bits of the NAS sequence number received in the message) and an estimate for the NAS overflow counter as defined in subclause 4.4.3.1 to form the NAS COUNT input to the integrity verification algorithm.

The algorithm to calculate the integrity protection information is specified in 3GPP TS 33.501 [24], and in case of the:

a)	SECURITY PROTECTED 5GS NAS MESSAGE message, the integrity protection shall include octet 7 to n, i.e. the Sequence number IE and the NAS message IE.

b)	Intra N1 mode NAS transparent container IE, the integrity protection shall include octet 7 to 9.

c)	S1 mode to N1 mode NAS transparent container IE, the integrity protection shall include octet 7 to 10.

d)	CONTROL PLANE SERVICE REQUEST message, the integrity protection shall include octet 1 to 4 and octet 9 to n, if any.

In addition to the data that is to be integrity protected, the BEARER ID, DIRECTION bit, NAS COUNT and 5G NAS are input to the integrity protection algorithm. These parameters are described in 3GPP TS 33.501 [24].

After successful integrity protection validation, the receiver shall update its corresponding locally stored NAS COUNT with the value of the estimated NAS COUNT for this NAS message.

Integrity verification is not applicable when 5G-IA0 is used.

* * * Next Change * * * *

4.4.3.4	Ciphering and deciphering

The sender shall use its locally stored NAS COUNT as input to the ciphering algorithm.

The receiver shall use the NAS sequence number included in the received message (or estimated from the 5 bits of the NAS sequence number received in the message) and an estimate for the NAS overflow counter as defined in subclause 4.4.3.1 to form the NAS COUNT input to the deciphering algorithm.

The input parameters to the NAS ciphering algorithm are the BEARER ID, DIRECTION bit, NAS COUNT, NAS encryption key and the length of the key stream to be generated by the encryption algorithm.

When applying initial NAS message protection to the REGISTRATION REQUEST or SERVICE REQUEST message as described in subclause 4.4.6, the length of the key stream is set to the length of the entire plain NAS message that is included in the NAS message container IE, i.e. the value part of the NAS message container IE, that is to be ciphered.

* * * Next Change * * * *

4.4.6	Protection of initial NAS signalling messages

The 5GS supports protection of initial NAS messages as specified in 3GPP TS 33.501 [24]. The protection of initial NAS messages applies to the REGISTRATION REQUEST, SERVICE REQUEST and CONTROL PLANE SERVICE REQUEST message, and is achieved as follows:

a)	If the UE does not have a valid 5G NAS security context, the UE sends a REGISTRATION REQUEST message including cleartext IEs only. After activating a 5G NAS security context resulting from a security mode control procedure:

1)	if the UE needs to send non-cleartext IEs, the UE shall include the entire REGISTRATION REQUEST message (i.e. containing both cleartext IEs and non-cleartext IEs) in the NAS message container IE and shall include the NAS message container IE in the SECURITY MODE COMPLETE message; or

2)	if the UE does not need to send non-cleartext IEs, the UE shall include the entire REGISTRATION REQUEST message (i.e. containing cleartext IEs only) in the NAS message container IE and shall include the NAS message container IE in the SECURITY MODE COMPLETE message.

b)	If the UE has a valid 5G NAS security context and:

1)	the UE needs to send non-cleartext IEs in a REGISTRATION REQUEST or SERVICE REQUEST message, the UE includes the entire REGISTRATION REQUEST or SERVICE REQUEST message (i.e. containing both cleartext IEs and non-cleartext IEs) in the NAS message container IE and shall cipher the value part of the NAS message container IE. The UE shall then send a REGISTRATION REQUEST or SERVICE REQUEST message containing the cleartext IEs and the NAS message container IE;

2)	the UE needs to send non-cleartext IEs in a CONTROL PLANE SERVICE REQUEST message:

i)	if CIoT small data container IE is the only non-cleartext IE to be sent, the UE shall cipher the value part of the CIoT small data container IE. The UE shall then send a CONTROL PLANE SERVICE REQUEST message containing the cleartext IEs and the CIoT small data container IE;

ii)	otherwise, the UE includes non-cleartext IEs in the NAS message container IE and shall cipher the value part of the NAS message container IE. The UE shall then send a CONTROL PLANE SERVICE REQUEST message containing the cleartext IEs and the NAS message container IE; or

3)	the UE does not need to send non-cleartext IEs in a REGISTRATION REQUEST or SERVICE REQUEST or CONTROL PLANE SERVICE REQUEST message, the UE sends the REGISTRATION REQUEST or SERVICE REQUEST or CONTROL PLANE SERVICE REQUEST message without including the NAS message container IE.

When the initial NAS message is a REGISTRATION REQUEST message, the cleartext IEs are:

-	Extended protocol discriminator;

-	Security header type;

-	Spare half octet;

-	Registration request message identity;

-	5GS registration type;

-	ngKSI;

-	5GS mobile identity;

-	UE security capability;

-	Additional GUTI;

-	UE status; and

-	EPS NAS message container.

When the initial NAS message is a SERVICE REQUEST message, the cleartext IEs are:

-	Extended protocol discriminator;

-	Security header type;

-	Spare half octet;

-	ngKSI;

-	Service request message identity;

-	Service type; and

-	5G-S-TMSI.

When the initial NAS message is a CONTROL PLANE SERVICE REQUEST message, the cleartext IEs are:

-	Extended protocol discriminator;

-	Security header type;

-	ngKSI and sequence number;

-	Message authentication code; and

-	Control plane service type.

When the UE sends a REGISTRATION REQUEST or SERVICE REQUEST message that includes a NAS message container IE, the UE shall set the security header type of the initial NAS message to "integrity protected".

When the UE sends a CONTROL PLANE SERVICE REQUEST message, the UE shall set the security header type of the initial NAS message to "security header for the CONTROL PLANE SERVICE REQUEST message".

If the UE does not need to send non-cleartext IEs in the initial NAS message, the UE shall send the initial NAS message i.e. REGISTRATION REQUEST or SERVICE REQUEST or CONTROL PLANE SERVICE REQUEST message with cleartext IEs only i.e. without including the NAS message container IE in the initial NAS message.

When the AMF receives an integrity protected initial NAS message which includes a NAS message container IE, the AMF shall decipher the value part of the NAS message container IE. If the received initial NAS message is a REGISTRATION REQUEST message or a SERVICE REQUEST message, the AMF shall consider the NAS message that is obtained from the NAS message container IE as the initial NAS message that triggered the procedure.

When the initial NAS message is a DEREGISTRATION REQUEST message, the UE always sends the NAS message unciphered.

If the UE:

a)	has 5G-EA0 as a selected 5G NAS security algorithm; and

b)	selects a PLMN other than registered PLMN and EPLMN;

the UE shall discard the 5G NAS security context and send an initial NAS message including cleartext IEs only as described in this subclause for the case when the UE does not have a valid 5G NAS security context.

ndicate which PDU session(s) have pending user data to be sent via user-plane resources.

* * * Next Change * * * *

8.2.30.1	Message definition

The CONTROL PLANE SERVICE REQUEST message is sent by the UE to the AMF when the UE is using 5GS services with control plane CIoT 5GS optimization. See table 8.2.30.1.1.

Message type:	CONTROL PLANE SERVICE REQUEST

Significance:	dual

Direction:		UE to network

Table 8.2.30.1.1: CONTROL PLANE SERVICE REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | ngKSI | NAS key set identifier 9.11.3.32 | M | V | 1/2 |
|  | Control plane service type | Control plane service type 9.11.3.18D | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Sequence number | Sequence number 9.10 | M | V | 1 |
|  | Message authentication code | Message authentication code 9.8 | M | V | 4 |
| xx | CIoT small data container | CIoT small data container 9.11.3.18B | O | TLV | 4-257 |
| 8- | Payload container type | Payload container type 9.11.3.40 | O | TV | 1 |
| 7B | Payload container | Payload container 9.11.3.39 | O | TLV-E | 4-65538 |
| 12 | PDU session ID | PDU session identity 2 9.11.3.41 | C | TV | 2 |
| 50 | PDU session status | PDU session status 9.11.3.44 | O | TLV | 4-34 |
| F- | Release assistance indication | Release assistance indication 9.11.3.46A | O | TV | 1 |
| 40 | Uplink data status | Uplink data status 9.11.3.57 | O | TLV | 4-34 |
| 71 | NAS message container | NAS message container 9.11.3.33 | O | TLV-E | 4-n |
| 24 | Additional information | Additional information 9.11.2.1 | O | TLV | 3-n |

* * * Next Change * * * *

9.1.1	NAS message format

Within the protocols defined in the present document, every 5GS NAS message, except the CONTROL PLANE SERVICE REQUEST message, is a standard L3 message as defined in 3GPP TS 24.007 [11]. This means that the message consists of the following parts:

1)	if the message is a plain 5GS NAS message:

a)	extended protocol discriminator;

b)	security header type associated with a half spare octet or PDU session identity;

c)	procedure transaction identity;

d)	message type;

e)	other information elements, as required.

2)	if the message is a security protected 5GS NAS message:

a)	extended protocol discriminator;

b)	security header type associated with a half spare octet;

c)	message authentication code;

d)	sequence number;

e)	plain 5GS NAS message, as defined in item 1

The organization of a plain 5GS NAS message is illustrated in the example shown in figure 9.1.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Extended protocol discriminator |  |  |  |  |  |  |  | octet 1 |
| Security header type associated with a spare half octet; or PDU session identity |  |  |  |  |  |  |  | octet 2 |
| Procedure transaction identity |  |  |  |  |  |  |  | octet 2a* |
| Message type |  |  |  |  |  |  |  | octet 3 |
|  |  |  |  |  |  |  |  | octet 4 |
| Other information elements as required |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet n |

Figure 9.1.1.1: General message organization example for a plain 5GS NAS message

The PDU session identity and the procedure transaction identity are only used in messages with extended protocol discriminator 5GS session management. Octet 2a with the procedure transaction identity shall only be included in these messages.

The organization of a security protected 5GS NAS message is illustrated in the example shown in figure 9.1.2.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Extended protocol discriminator |  |  |  |  |  |  |  | octet 1 |
| Security header type associated with a spare half octet |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  | octet 3 |
| Message authentication code |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet 6 |
| Sequence number |  |  |  |  |  |  |  | octet 7 |
|  |  |  |  |  |  |  |  | octet 8 |
| Plain 5GS NAS message |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet n |

Figure 9.1.1.2: General message organization example for a security protected 5GS NAS message

Unless specified otherwise in the message descriptions of clause 8 and annex D, a particular information element shall not be present more than once in a given message.

* * * Next Change * * * *

9.3	Security header type

Bits 1 to 4 of the second octet of every 5GMM message contain the Security header type IE. This IE includes control information related to the security protection of a 5GMM message. The total size of the Security header type IE is 4 bits.

The Security header type IE can take the values shown in table 9.3.1.

Table 9.3.1: Security header type

| Security header type (octet 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 4 | 3 | 2 | 1 |  |
| 0 | 0 | 0 | 0 | Plain 5GS NAS message, not security protected |
|  |  |  |  | Security protected 5GS NAS message: |
| 0 | 0 | 0 | 1 | Integrity protected |
| 0 | 0 | 1 | 0 | Integrity protected and ciphered |
| 0 | 0 | 1 | 1 | Integrity protected with new 5G NAS security context (NOTE 1) |
| 0 | 1 | 0 | 0 | Integrity protected and ciphered with new 5G NAS security context (NOTE 2) |
|  |  |  |  | Non-standard L3 message: |
| 1 | 1 | 0 | 0 | Security header for the CONTROL PLANE SERVICE REQUEST message |
| 1 | 1 | 0 | 1 | These values are not used in this version of the protocol. |
| to |  |  |  | If received they shall be interpreted as '1100'. (NOTE 3) |
| 1 | 1 | 1 | 1 |  |
| All other values are reserved. |  |  |  |  |
| NOTE 1: This codepoint may be used only for a SECURITY MODE COMMAND message. NOTE 2: This codepoint may be used only for a SECURITY MODE COMPLETE message. NOTE 3: When bits 3 and 4 are set to '11', bits 1 and 2 can be used for future extensions of the CONTROL PLANE SERVICE REQUEST message. |  |  |  |  |

A 5GMM message received with the security header type encoded as 0000 shall be treated as not security protected, plain 5GS NAS message. A protocol entity sending a not security protected 5GMM message shall send the message as plain 5GS NAS message and encode the security header type as 0000.

* * * Next Change * * * *

9.7	Message type

The Message type IE and its use are defined in 3GPP TS 24.007 [11]. Tables 9.7.1 and 9.7.2 define the value part of the message type IE used in the 5GS mobility management protocol and 5GS session management protocol.

Table 9.7.1: Message types for 5GS mobility management

| Bits |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |  |  |
| 0 |  | 1 |  | - |  | - |  | - |  | - |  | - |  | - |  |  |  | 5GS mobility management messages |  |
| 0 |  | 1 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  |  |  | Registration request |  |
| 0 |  | 1 |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  | 0 |  |  |  | Registration accept |  |
| 0 |  | 1 |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  | 1 |  |  |  | Registration complete |  |
| 0 |  | 1 |  | 0 |  | 0 |  | 0 |  | 1 |  | 0 |  | 0 |  |  |  | Registration reject |  |
| 0 |  | 1 |  | 0 |  | 0 |  | 0 |  | 1 |  | 0 |  | 1 |  |  |  | Deregistration request (UE originating) |  |
| 0 |  | 1 |  | 0 |  | 0 |  | 0 |  | 1 |  | 1 |  | 0 |  |  |  | Deregistration accept (UE originating) |  |
| 0 |  | 1 |  | 0 |  | 0 |  | 0 |  | 1 |  | 1 |  | 1 |  |  |  | Deregistration request (UE terminated) |  |
| 0 |  | 1 |  | 0 |  | 0 |  | 1 |  | 0 |  | 0 |  | 0 |  |  |  | Deregistration accept (UE terminated) |  |
| 0 |  | 1 |  | 0 |  | 0 |  | 1 |  | 1 |  | 0 |  | 0 |  |  |  | Service request |  |
| 0 |  | 1 |  | 0 |  | 0 |  | 1 |  | 1 |  | 0 |  | 1 |  |  |  | Service reject |  |
| 0 |  | 1 |  | 0 |  | 0 |  | 1 |  | 1 |  | 1 |  | 0 |  |  |  | Service accept |  |
| 0 |  | 1 |  | 0 |  | 1 |  | 0 |  | 0 |  | x |  | x |  |  |  | Network slice-specific authentication command |  |
| 0 |  | 1 |  | 0 |  | 1 |  | 0 |  | 0 |  | x |  | x |  |  |  | Network slice-specific authentication complete |  |
| 0 |  | 1 |  | 0 |  | 1 |  | 0 |  | 0 |  | x |  | x |  |  |  | Network slice-specific authentication result |  |
| 0 |  | 1 |  | 0 |  | 1 |  | 0 |  | 1 |  | 0 |  | 0 |  |  |  | Configuration update command |  |
| 0 |  | 1 |  | 0 |  | 1 |  | 0 |  | 1 |  | 0 |  | 1 |  |  |  | Configuration update complete |  |
| 0 |  | 1 |  | 0 |  | 1 |  | 0 |  | 1 |  | 1 |  | 0 |  |  |  | Authentication request |  |
| 0 |  | 1 |  | 0 |  | 1 |  | 0 |  | 1 |  | 1 |  | 1 |  |  |  | Authentication response |  |
| 0 |  | 1 |  | 0 |  | 1 |  | 1 |  | 0 |  | 0 |  | 0 |  |  |  | Authentication reject |  |
| 0 |  | 1 |  | 0 |  | 1 |  | 1 |  | 0 |  | 0 |  | 1 |  |  |  | Authentication failure |  |
| 0 |  | 1 |  | 0 |  | 1 |  | 1 |  | 0 |  | 1 |  | 0 |  |  |  | Authentication result |  |
| 0 |  | 1 |  | 0 |  | 1 |  | 1 |  | 0 |  | 1 |  | 1 |  |  |  | Identity request |  |
| 0 |  | 1 |  | 0 |  | 1 |  | 1 |  | 1 |  | 0 |  | 0 |  |  |  | Identity response |  |
| 0 |  | 1 |  | 0 |  | 1 |  | 1 |  | 1 |  | 0 |  | 1 |  |  |  | Security mode command |  |
| 0 |  | 1 |  | 0 |  | 1 |  | 1 |  | 1 |  | 1 |  | 0 |  |  |  | Security mode complete |  |
| 0 |  | 1 |  | 0 |  | 1 |  | 1 |  | 1 |  | 1 |  | 1 |  |  |  | Security mode reject |  |
| 0 |  | 1 |  | 1 |  | 0 |  | 0 |  | 1 |  | 0 |  | 0 |  |  |  | 5GMM status |  |
| 0 |  | 1 |  | 1 |  | 0 |  | 0 |  | 1 |  | 0 |  | 1 |  |  |  | Notification |  |
| 0 |  | 1 |  | 1 |  | 0 |  | 0 |  | 1 |  | 1 |  | 0 |  |  |  | Notification response |  |
| 0 |  | 1 |  | 1 |  | 0 |  | 0 |  | 1 |  | 1 |  | 1 |  |  |  | UL NAS transport |  |
| 0 |  | 1 |  | 1 |  | 0 |  | 1 |  | 0 |  | 0 |  | 0 |  |  |  | DL NAS transport |  |

Table 9.7.2: Message types for 5GS session management

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 1 | 1 | - | - | - | - | - | - |  | 5GS session management messages |
| 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |  | PDU session establishment request |
| 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |  | PDU session establishment accept |
| 1 | 1 | 0 | 0 | 0 | 0 | 1 | 1 |  | PDU session establishment reject |
| 1 | 1 | 0 | 0 | 0 | 1 | 0 | 1 |  | PDU session authentication command |
| 1 | 1 | 0 | 0 | 0 | 1 | 1 | 0 |  | PDU session authentication complete |
| 1 | 1 | 0 | 0 | 0 | 1 | 1 | 1 |  | PDU session authentication result |
| 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 |  | PDU session modification request |
| 1 | 1 | 0 | 0 | 1 | 0 | 1 | 0 |  | PDU session modification reject |
| 1 | 1 | 0 | 0 | 1 | 0 | 1 | 1 |  | PDU session modification command |
| 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 |  | PDU session modification complete |
| 1 | 1 | 0 | 0 | 1 | 1 | 0 | 1 |  | PDU session modification command reject |
| 1 | 1 | 0 | 1 | 0 | 0 | 0 | 1 |  | PDU session release request |
| 1 | 1 | 0 | 1 | 0 | 0 | 1 | 0 |  | PDU session release reject |
| 1 | 1 | 0 | 1 | 0 | 0 | 1 | 1 |  | PDU session release command |
| 1 | 1 | 0 | 1 | 0 | 1 | 0 | 0 |  | PDU session release complete |
| 1 | 1 | 0 | 1 | 0 | 1 | 1 | 0 |  | 5GSM status |

* * * Next Change * * * *

9.9	Plain 5GS NAS message

This IE includes a complete plain 5GS NAS message as specified in subclauses 8.2 and 8.3. The SECURITY PROTECTED 5GS NAS MESSAGE message (see subclause 8.2.28) and the CONTROL PLANE SERVICE REQUEST message are not plain 5GS NAS messages and shall not be included in this IE.

* * * End of Change * * * *