3GPP TSG-CT WG1 Meeting #122-e	C1-200386

- Electronic meeting; 24-28 February 2020

Source:	Huawei, HiSilicon, Ericsson

Title:	Correction for the list of the V2X services authorized for PPPR over V2X PC5 in E-UTRA

Spec:	3GPP TS 24.587 v1.0.1

Agenda item:	16.2.13

Document for:	Agreement

1. Introduction

This p-CR analyzes the list of the V2X services authorized for PPPR over V2X PC5 in E-UTRA and proposes to update the specification to keep compatibility to EPS and allow interworking with EPS for V2X services in 5GS.

2. Reason for Change

Based on stage 2 in 3GPP TS 23.287, the list of the V2X services authorized for PPPR over V2X PC5 in E-UTRA is the same as defined in TS 23.285 of EPS, quote of clause 5.4.1.1.1:

For LTE based PC5, the QoS handling is defined in TS 23.285 [8], based on ProSe Per-Packet Priority (PPPP) and ProSe Per-Packet Reliability (PPPR).

TS 23.285 states:

When PC5 is used for the transmission of V2X messages, the following principles are followed for both network scheduled operation mode and UE autonomous resources selection mode:

...

-	ProSe Per-Packet Reliability (PPPR) applies to the V2X communication over PC5 as defined in TS 23.303 [5]. PPPR has a value range of 1 to 8, where the higher value represents the lower reliability requirement for that message. The UE may be configured with a list of V2X services, e.g. PSID or ITS-AIDs allowed to use the specific PPPR value.

...

When the network scheduled operation mode is used, following additional principles apply:

...

-	UE may provide PPPR information reflecting reliability requirement described in TS 22.186 [28] to the eNB for resources request.

...

The following information for V2X communications over PC5 reference point is provisioned to the UE:

...

3)	Policy/parameters:

...

-	The list of V2X services, e.g. PSID or ITS-AIDs of the V2X applications, allowed to use a specific PPPR value.

...

Based on the above description, the list of the V2X services authorized for PPPR over V2X PC5 in E-UTRA is optional.

Based on stage 3 in 3GPP TS 24.386, the list of the V2X services authorized for PPPR over V2X PC5 in E-UTRA is optional, quote of clause 5.2.4 :

j)	optionally, a list of the V2X services authorized for ProSe Per-Packet Reliability (PPPR). Each entry of the list contains a V2X service identifier and a ProSe Per-Packet Reliability (PPPR) value; and

In order to allow interworking with EPS and also keep compatibility for different 3GPP releases it is therefore proposed that the same is applicable to5GS.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.587 v1.0.1.

* * * First Change * * * *

### 5.2.3	Configuration parameters for V2X communication over PC5

The configuration parameters for V2X communication over PC5 consist of:

a)	an expiration time for the validity of the configuration parameters for V2X communication over PC5;

Editor’s note:	The expiration time as part of the configuration parameters can be re-evaluated when the reply to the LS in C1-196967 arrives.

b)	a list of PLMNs and RATs in which the UE is authorized to use V2X communication over PC5 when the UE is served by E-UTRA or served by NR. Each entry of the list contains a PLMN ID and RATs in which the UE is authorized to use V2X communication over PC5;

c)	an indication of whether the UE is authorized to use V2X communication over PC5 when the UE is not served by E-UTRA and not served by NR;

d)	list of RATs in which the UE is authorized to use V2X communication over PC5 when the UE is not served by E-UTRA and not served by NR;

e)	per geographical area:

1)	radio parameters for V2X communication over PC5 applicable when the UE is not served by E-UTRA, not served by NR and is located in the geographical area, with an indication of whether these radio parameters are "operator managed" or "non-operator managed";

f)	a list of V2X service identifier to Tx profiles mapping rules. Each mapping rule contains one or more V2X service identifiers and a Tx profile;

g)	configuration parameters for privacy support, consisting of:

1)	a list of V2X services requiring privacy. Each entry of the list contains one or more V2X service identifiers and one or more geographical areas where the privacy is required; and

2)	a privacy timer value;

Editor’s note:	The encoding of the Privacy timer is FFS.

h)	configuration parameters for a V2X communication over PC5 in E-UTRA, consisting of:

1)	a list of V2X service identifier to destination layer-2 ID mapping rules. Each mapping rule contains one or more V2X service identifiers and the destination layer-2 ID;

2)	optionally, a default destination layer-2 ID;

3)	a list of PPPP to PDB mapping rules. Each mapping rule contains a ProSe Per-Packet Priority (PPPP) and a Packet Delay Budget (PDB);

4)	a list of V2X service identifier to V2X E-UTRA frequency mapping rules. Each mapping rule contains one or more V2X service identifiers and the V2X E-UTRA frequencies with associated geographical areas; and

5)	optionally, a list of the V2X services authorized for ProSe Per-Packet Reliability (PPPR). Each entry of the list contains one or more V2X service identifiers and a ProSe Per-Packet Reliability (PPPR) value; and

i)	configuration parameters for a V2X communication over PC5 in NR, consisting of:

1)	a list of V2X service identifier to V2X NR frequency mapping rules. Each mapping rule contains one or more V2X service identifiers and the V2X NR frequencies with associated geographical areas;

2)	a list of V2X service identifier to destination layer-2 ID for broadcast mapping rules. Each mapping rule contains one or more V2X service identifiers and the destination layer-2 ID for broadcast;

3)	optionally, a default destination layer-2 ID for broadcast;

4)	a list of V2X service identifier to destination layer-2 ID for groupcast mapping rules. Each mapping rule contains one or more V2X service identifiers and the destination layer-2 ID for groupcast;

5)	a list of V2X service identifier to default destination layer-2 ID for unicast initial signaling mapping rules. Each mapping rule contains one or more V2X service identifiers and the default destination layer-2 ID for initial signalling to establish unicast connection;

6)	a PC5 QoS mapping configuration which is a list of PC5 QoS mapping rules. Each PC5 QoS mapping rule contains an input consisting of one or more V2X service identifiers and optionally V2X application requirements for the V2X service, and an output consisting of PC5 QoS parameters as specified in subclause 5.4.2 of 3GPP TS 23.287 [2]. Specification of the V2X application requirements for the V2X service is out of scope of the present specification; and

7)	an SLRB configurations which is a list of SLRB mapping rules applicable when the UE is not served by E-UTRA and is not served by NR. Each SLRB mapping rule contains a PC5 QoS profile and an SLRB. The PC5 QoS profile contains the following parameters:

i)	the PC5 QoS profile contains a PQI;

ii)	if the PQI of the PC5 QoS profile identifies a GBR QoS, the PC5 QoS profile contains a PC5 flow bit rates consisting of a guaranteed flow bit rate (GFBR) and a maximum flow bit rate (MFBR);

iii)	if the PQI of the PC5 QoS profile does not identify a GBR QoS, the PC5 QoS profile contains the PC5 link aggregated bit rate consisting of a per link aggregate maximum bit rate (PC5 LINK-AMBR);

iv)	the PC5 QoS profile contains a range; and

v)	the PC5 QoS profile can contain the priority level, the averaging window, and the maximum data burst volume. If one or more of the priority level, the averaging window or the maximum data burst volume are not contained in the PC5 QoS profile, their default values apply.

* * * End of Change * * * *