3GPP TSG-CT WG1 Meeting #122-e	C1-200554

- Electronic meeting; 20-28 February 2020

Source:	Huawei, HiSilicon

Title:	On-demand location reporting procedure

Spec:	3GPP TS 24.545 v0.2.0

Agenda item:	16.2.20

Document for:	Agreement

1. Introduction

This p-CR defines the on-demand location reporting procedure for 3GPP TS 24.545.

2. Reason for Change

The specification needs to define the stage 3 details of on-demand location reporting procedure defined in 3GPP TS 23.434 clause 9.3.4.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.545 v0.2.0.

* * * First Change * * * *

#### 6.2.2.2	Client procedure

In order to report the location information, the SLM-C shall send a HTTP POST request message according to procedures specified in IETF RFC 2616 [6]. In the HTTP POST request message, the SLM-C:

b)	shall set the Request-URI to the URI included in the received HTTP response message for location report configuration;

c)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-location-info+xml";

d)	shall include an application/vnd.3gpp.seal-location-info+xml MIME body and in the <location-info> root element:

1)	shall include a <identity> element with a <VAL-user-id> child element set to the identy of the VAL user for location report; and

2)	shall include a <report> element and, if the report was triggered by a location request, include the <report-id> attribute set to the value of the <request-id> attribute in the received request. The <report> element:

i)	shall include a <trigger-id> child element set to the value of each <trigger-id> value of the triggers that have been met; and

ii)	shall include the location reporting elements corresponding to the triggers that have been met;

e)	shall set the minimum-report-interval timer to the minimum-report-interval time and start this timer; and

* * * Next Change * * * *

### 6.2.3	On-demand location reporting procedure

* * * Next Change * * * *

#### 6.2.3.1	Client procedure

Upon receiving an HTTP POST request containing:

- an Accept header field set to "application/vnd.3gpp.seal-location-info+xml";

- a Content-Type header field set to "application/vnd.3gpp.seal-location-info+xml";

- an application/vnd.3gpp.seal-location-info+xml MIME body with a <request> element included in the <location-info> root element;

the SLM-C:

a)	may send a location report as specified in clause 6.2.2.2.2

.

* * * Next Change * * * *

#### 6.2.3.2	Server procedure

If the SLM-S needs to request the SLM-C to report its location, the SLM-S shall generate an HTTP POST request according to procedures specified in IETF RFC 2616 [6]. The SLM-S:

- shall include a Request-URI set to the URI corresponding to the identity of the SLM-C;

- shall include an Accept header field set to "application/vnd.3gpp.seal-location-info+xml";

- shall include a Content-Type header field set to "application/vnd.3gpp.seal-location-info+xml";

- shall include an application/vnd.3gpp.seal-location-info+xml MIME body and in the <location-info> root element:

- shall include a <requested-identity> element with a <VAL-user-id> child element set to the identity of the VAL user whose location is requested;

- shall include a <request> element; and

- shall send the HTTP POST request as specified in IETF RFC 2616 [6].

* * * End of Change * * * *