3GPP TSG-CT WG1 Meeting #122-e	C1-200648

- Electronic meeting; 20-28 February 2020

Source:	Samsung

Title:	Procedure to notify configuration management event

Spec:	3GPP TS 24.546 v1.0.0

Agenda item:	16.2.20

Document for:	Agreement

1. Introduction

This p-CR provides procedure to notify configuration management events.

2. Reason for Change

The stage#2 specification has provided procedure to notify configuration modification events from SCM-S to SCM-C.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.546 v1.0.0.

* * * First Change * * * *

#### 6.2.8.2	Notifications

##### 6.2.8.2.1	Client procedure

###### 6.2.8.2.1.1	Receiving configuration update notification

Upon receiving an HTTP POST request over a call back URI in the configuration update event subscription message, the SCM-C:

a)	shall identify the subscription identity received in the "Identity" parameter of the HTTP POST request. If the subscription identity is not valid, the SCM-C:

1) shall send an HTTP 406 (Not Acceptable) response and skip rest of the steps;

b)	shall send an HTTP 200 (OK) message; and

c)	shall notify the VAL user about the modification of configuration document based on the "Event" parameter.

Based on VAL user’s request, the SCM-C may also retrieve the configuration document as specified in clause 6.2.3 or in clause 6.2.4.

* * * Next Change * * * *

##### 6.2.8.2.2	Server procedure

###### 6.2.8.2.2.1	Sending group modify notification

Upon successful modification of VAL user profile document or VAL UE configuration document, the SCM-S sends a notification to SCM-C. The SCM-S:

a)	shall check whether valid configuration update event subscription exists for event SUBSCRIBE_USER_PROFILE_MODIFICATION (0x01) OR SUBSCRIBE_UE_CONFIG_MODIFICATION (0x02) as defined in clause X.1.2 or not;

1)	if valid subscription does not exist, shall skip rest of the steps;

b)	shall generate an HTTP POST message to notify configuration update notification. In HTTP POST message:

1)	shall set the request URI to call back URI received in the creating subscription procedure;

2)	shall set the Content-Type header to "application/json"; and

3)	shall include an HTTP request entity-body with the parameters specified in clause Y.2 serialized into a JavaScript Object Notation (JSON) structure; and

c)	shall sent an HTTP POST request towards SCM-C.

* * * End of Change * * * *