3GPP TSG-CT WG1 Meeting #122-e	C1-200959

- Electronic meeting; 20-28 February 2020

Source:	Ericsson

Title:	Clarifications of identity definitions and activation procedures

Spec:	3GPP TS 24.174V1.2.1

Agenda item:	16.3.3

Document for:	Agreement

1. Introduction

<Introduction part >

2. Reason for Change

The definitions of identity C and identity D are a bit hard to digest. It is proposed to add information what kinds of identities these identities can be.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.174 V1.2.1.

* * * First Change * * * *

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

native identity: identity used by the UE (e.g. MSISDN, tel URI or SIP URI) that is associated to the IMSI which is currently used by the device for IMS registration.

alternative identity: identity used by the UE (e.g. MSISDN, tel URI or SIP URI) that is not associated to the IMSI which is currently used by the device for IMS registration and which is associated to a different IMSI for the same IMS subscription owned by the same IMS network.

external alternative identity: identity used by the UE (e.g. MSISDN, tel URI or SIP URI) that is not associated to the IMSI which is currently used by the device for IMS registration and which is associated to a different IMSI for another IMS subscription owned by the same or a different IMS network.

virtual identity: identity used by the UE (e.g. MSISDN, tel URI or SIP URI) that is not associated as native identity with any IMSI that is associated with ISIM or USIM in a device. The virtual identity can be used by a single user only or by several users having IMS subscriptions in the same or different IMS networks.

non-native identity: identity which is not the native identity. The non-native identity may be an alternative identity, external alternative identity or a virtual identity.

federated UEs: a group of UEs which are configured to use the same public user identity.

user A: user A is the originating user, in the present document user A calls user B, where A can be using the identity C.

user B: user B is the terminating user, in the present document user A calls user B, where B can be reached under the identity D.

identity C: identity C is an identity that can be used by user A and is not registered by user A. Identity C can be an external alternative identity or a virtual identity.

identity D: identity D is an identity that can be used by user B and is not registered by user B. Identity D can be an external alternative identity or a virtual identity.

* * * Next Change * * * *

### 4.5.2	Activation/deactivation

The MuD and MiD services are activated at provisioning and deactivated at withdrawal or at the user’s request.

The user of MiD service decides which of its identities are active and can be used for incoming and outgoing calls by changing the "Activated" attribute in the <Shared-identity> or <Delagated-user> elements in the service configuration data.

The user decides if it permits another user to use its native identity.

* * * End of Changes * * * *