3GPP TSG-CT WG1 Meeting #122-e	C1-200619-revised

- Electronic meeting; 20-28 February 2020

Source:	Huawei, HiSilicon

Title:	Structure and data semantics for application level location tracking procedure

Spec:	3GPP TS 24.486 v0.3.0

Agenda item:	16.2.12

Document for:	Agreement

1. Introduction

This p-CR defines the structure and data semantics for for application level location tracking procedure.

2. Reason for Change

The specification needs to define the stage 3 details of the structure and data semantics for for application level location tracking procedure.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.486 v0.3.0.

* * * First Change * * * *

## 8.3	Structure

The VAE document shall conform to the XML schema described in clause 8.4.

The <location-tracking-info> element shall include one of the following:

a)	a <identity> element shall include a <V2X-UE-id> element; or

b)	a <geographical-identifier> element shall include a <geo-id> element.

The <identity> element shall include a <V2X-UE-id> child element.

The <geographical-identifier> element shall include…?

The <location-tracking> element shall include a <result> element.

* * * Next Change * * * *

## 8.5	Data semantics

<identity> is a mandatory element used to include the identity of a VAL client. The <identity> element contains a <V2X-UE-id> attribute that contains the identity of the VAL client.

<geographical-identifier> is a…?

<location-tracking> is a mandatory element used to include the V2X message delivery response information. The <registration-info> element contains the <result> sub-element. The <result> is a mandatory element which indicates a value "success" or "fail".