3GPP TSG-CT WG1 Meeting #123-e	C1-202310

- Electronic meeting; 16-24 April 2020

Source:	Huawei, HiSilicon

Title:	MBMS suspension notification procedure

Spec:	3GPP TS 24.548 v1.0.0

Agenda item:	16.2.20

Document for:	Agreement

1. Introduction

This p-CR defines the MBMS suspension notification procedure for 3GPP TS 24.548.

2. Reason for Change

The specification needs to define the stage 3 details of the MBMS suspension notification procedure defined in 3GPP TS 23.434 clause 14.3.4.7.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.548 v1.0.0.

* * * First Change * * * *

- 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[3]	3GPP TS 24.547: "Identity management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification".

[4]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[5]	3GPP TS 24.486: "Vehicle-to-Everything (V2X) Application Enabler (VAE) layer; Protocol aspects; Stage 3".

[6]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point".

[7]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[8]	3GPP TS 29.549: "Service Enabler Architecture Layer (SEAL); Application Programming Interface (API) specification".

[9]	IETF RFC 3428: "Session Initiation Protocol (SIP) Extension for Instant Messaging".

[10]	IETF RFC 3841: "Caller Preferences for the Session Initiation Protocol (SIP)".

[11]	IETF RFC 4566: "Session Description Protocol".

[12]	IETF RFC 4825: "The Extensible Markup Language (XML) Configuration Access Protocol (XCAP)".

[13]	IETF RFC 5795: "The Robust Header Compression (ROHC) Framework".

[14]	OMA OMA-TS-XDM_Group-V1_1_1-20170124-A: "Group XDM Specification".

[r2616]	IETF RFC 2616: "Hypertext Transfer Protocol -- HTTP/1.1".

* * * Next Change * * * *

#### 6.2.3.6	MBMS suspension notification procedure

* * * Next Change * * * *

##### 6.2.3.6.1	Client procedure

Upon receiving an HTTP POST request message containing:

a)	a Content-Type header field set to "application/vnd.3gpp.seal-mbms-usage-info +xml"; and

b)	an application/vnd.3gpp.seal-mbms-usage-info+xml MIME body with an <mbms-suspension-reporting-instruction> elment in the <mbms-info> root element;

the SNRM-C shall send an HTTP 204 (No Content) response according to IETF RFC 2616 [r2616] towards the SNRM-S.

If the SNRM-C detects the MBMS suspension and has not received a <suspension-reporting> element set to "disable", the SNRM-C shall generate an HTTP POST request message according to IETF RFC 2616 [r2616]. In the HTTP POST request message, the SNRM-C:

a)	shall include a Request-URI set to the URI corresponding to the identity of the SNRM-S;

b)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-mbms-usage-info +xml";

c)	shall include an application/vnd.3gpp.seal-mbms-usage-info+xml MIME body with an <mbms-suspension-report> element in the <mbms-info> root element which:

1) shall include an <identity> element set to the identity of the VAL user or VAL UE that reports MBMS suspension;

2)	if at least one MBMS bearer is about to be suspended:

i)	shall include an <mbms-suspension-status> element set to "suspending";

ii)	shall set the <number-of-reported-bearers> element to the total number of the included <suspended-TMGI> elements and <other-TMGI> elements;

iii)	shall include <suspended-TMGI> element(s) set to the TMGI value for each of the MTCHs on the same MCH corresponding to the MBMS bearers about to be suspended; and

iv)	may include <other-TMGI> elements, if available, corresponding to the TMGI values for other MTCHs on the same MCH as the MBMS bearers to be suspended; and

3)	if the MBMS bearer is no longer about to be suspended, shall include:

i)	an <mbms-suspension-status> element set to "not-suspending";

ii)	a <number-of-reported-bearers> element set to the number of included <suspended-TMGI> elements; and

iii)	a <suspended-TMGI> element set to the corresponding TMGI value for each of the MTCHs of the MBMS bearers that are no longer about to be suspended.

d)	shall send the HTTP POST request message towards the SNRM-S according to IETF RFC 2616 [r2616].

* * * Next Change * * * *

##### 6.2.3.6.2	Server procedure

If the SNRM-S decide on a subset of all VAL UEs in the MBMS broadcast area that shall report on MBMS bearer suspension, the SNRM-S shall generate an HTTP POST request message according to IETF RFC 2616 [r2616]. In the HTTP POST request message, the SNRM-S:

a)	shall include a Request-URI set to the URI corresponding to the identity of the SNRM-C;

b)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-mbms-usage-info +xml";

c)	shall include an application/vnd.3gpp.seal-mbms-usage-info+xml MIME body with an <mbms-suspension-reporting-instruction> element in the <mbms-info> root element which:

1)	if a unicast bearer is used for MBMS suspension reporting, shall include:

i)	an <identity> element set to the identity of the VAL user or VAL UE that shall report MBMS suspension; and

ii)	a <suspension-reporting> element indicating to enable or disable the suspension reporting for the SNRM-C;

2) if a multicast bearer is used for MBMS suspension reporting, shall include:

i)	a <suspension-reporting-client-subset> element containing a uniquely defined subset of NRM clients that shall report MBMS suspension; and

d) shall send the HTTP POST request message towards the SNRM-C according to IETF RFC 2616 [r2616].

* * * End of Change * * * *