- 3GPP TSG CT WG1 Meeting 124-e	TDoc C1-203972

- Electronic meeting, 2-10 June 2020

Title:	Inclusion of pending S-NSSAI(s) in the requested NSSAI

Response to:

Release:	Rel-16

Work Item:	eNS

Source:	3GPP CT1

To:	3GPP SA2

Cc:

Contact person:	Lin Shu

shulin@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

CT1 has observed following requirement in TS 23.501 sub 5.15.5.2.1:

"The UE shall not include in the Requested NSSAI any of the S-NSSAIs from the Pending NSSAI the UE stores, regardless of the Access Type."

During the stage 3 NAS protocol implementation, CT1 has identified following questions if stage 3 is going to above stage 2 requirement based on current NAS protocol design principles.

Below example is taken for questions:

- UE has configured NSSAI = {A, B, C, D}

- UE sent requested NSSAI = {A, B} over 3GPP access

- UE received pending NSSAI = {A, B}

Question #0: Is the UE mandated to complete the ongoing NSSAA procedure for {A, B} before initiating any subsequent mobility registration update procedure in which the UE needs to request any of slices {A, B}?

Questions #1 - #3 apply if the answer to Question #0 is “no”.

Question #1: Before the completion of network slice-specific authentication and authorization (NSSAA) for {A, B}, if the UE needs to initiate a new mobility registration update procedure to register to different slices over 3GPP access, e.g. to register to {A, B, C} or {A, C}, then what should the UE do in terms of requested NSSAI and what should the AMF do regarding allowed NSSAI, rejected NSSAI, and/or pending NSSAI?

Question #2: Before the completion of NSSAA for {A, B}, if the UE has to initiate a new mobility registration update procedure to register to same slices {A, B} over 3GPP access (e.g. the UE moves out of the current TAI list), then what should the UE do in terms of requested NSSAI and what should the AMF do regarding allowed NSSAI, rejected NSSAI, and/or pending NSSAI?

Question #3: Before the completion of NSSAA for {A, B} over 3GPP access, if the UE has to register to non-3GPP access for slices including any of {A, B}, e.g. {A, B}, {A, B, C} or {A, C}, then what should the UE do in terms of requested NSSAI and what should the AMF do regarding allowed NSSAI, rejected NSSAI, and/or pending NSSAI?

# 2	Actions

To 3GPP SA2

ACTION: 	3GPP CT1 kindly asks SA2 to answer above questions.

# 3	Dates of next TSG CT WG1 meetings

TSG CT WG1 Meeting 125-e 	e-meeting	20-28 August 2020

TSG CT WG1 Meeting 126-e	TBD