3GPP TSG-CT WG1 Meeting #124-e	C1-203059

Electronic meeting, 2-10 June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.587 | CR | 0042 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of PC5 QoS mapping configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  | Date: |  |  | 2020-06-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SA2 agreed S2-2003421 (TS 23.287 CR 0113) replacing stage-2 requirements on PC5 QoS mapping configuration with: ------------------------- - The mapping of V2X service types (e.g. PSID or ITS-AID) to PC5 QoS parameters defined in clause 5.4.2 (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc.). ------------------------- |  |  |  |  |  |  |  |  |
| Summary of change: |  | PC5 QoS mapping configuration is replaced with V2X service identifier to PC5 QoS parameters mapping rules. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage-3 not aligned with stage-2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 23.287 CR 0113 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Changes in revision 1: - CR category changed to "F" |  |  |  |  |  |  |  |  |

***** change *****

### 5.2.3	Configuration parameters for V2X communication over PC5

The configuration parameters for V2X communication over PC5 consist of:

a)	a validity timer for the validity of the configuration parameters for V2X communication over PC5;

b)	a list of PLMNs and RATs in which the UE is authorized to use V2X communication over PC5 when the UE is served by E-UTRA or served by NR. Each entry of the list contains a PLMN ID and RATs in which the UE is authorized to use V2X communication over PC5;

c)	an indication of whether the UE is authorized to use V2X communication over PC5 when the UE is not served by E-UTRA and not served by NR;

d)	list of RATs in which the UE is authorized to use V2X communication over PC5 when the UE is not served by E-UTRA and not served by NR;

e)	per geographical area:

1)	radio parameters for V2X communication over PC5 applicable when the UE is not served by E-UTRA, not served by NR and is located in the geographical area, with an indication of whether these radio parameters are "operator managed" or "non-operator managed";

f)	optionally, a list of V2X service identifier to Tx profiles mapping rules. Each mapping rule contains one or more V2X service identifiers and a Tx profile;

g)	configuration parameters for privacy support, consisting of:

1)	a list of V2X services requiring privacy. Each entry of the list contains one or more V2X service identifiers and one or more geographical areas where the privacy is required; and

2)	a privacy timer value;

Editor’s note:	The encoding of the Privacy timer is FFS.

h)	configuration parameters for a V2X communication over PC5 in E-UTRA, consisting of:

1)	a list of V2X service identifier to destination layer-2 ID mapping rules. Each mapping rule contains one or more V2X service identifiers and the destination layer-2 ID;

2)	optionally, a default destination layer-2 ID;

3)	a list of PPPP to PDB mapping rules. Each mapping rule contains a ProSe Per-Packet Priority (PPPP) and a Packet Delay Budget (PDB);

4)	optionally, list of V2X service identifier to V2X E-UTRA frequency mapping rules. Each mapping rule contains one or more V2X service identifiers and the V2X E-UTRA frequencies with associated geographical areas; and

5)	optionally, a list of the V2X services authorized for ProSe Per-Packet Reliability (PPPR). Each entry of the list contains one or more V2X service identifiers and a ProSe Per-Packet Reliability (PPPR) value; and

i)	configuration parameters for a V2X communication over PC5 in NR, consisting of:

1)	optionally, a list of V2X service identifier to V2X NR frequency mapping rules. Each mapping rule contains one or more V2X service identifiers and the V2X NR frequencies with associated geographical areas;

2)	a list of V2X service identifier to destination layer-2 ID for broadcast mapping rules. Each mapping rule contains one or more V2X service identifiers and the destination layer-2 ID for broadcast;

3)	optionally, a default destination layer-2 ID for broadcast;

4)	a list of V2X service identifier to destination layer-2 ID for groupcast mapping rules. Each mapping rule contains one or more V2X service identifiers and the destination layer-2 ID for groupcast;

5)	a list of V2X service identifier to default destination layer-2 ID for unicast initial signaling mapping rules. Each mapping rule contains one or more V2X service identifiers and the default destination layer-2 ID for initial signalling to establish unicast connection;

6)	a list of V2X service identifier to PC5 QoS parameters mapping rules. The PC5 QoS parameters are specified in clause 5.4.2 of 3GPP TS 23.287 [3]; and

7)	an SLRB configurations which is a list of SLRB mapping rules applicable when the UE is not served by E-UTRA and is not served by NR. Each SLRB mapping rule contains a PC5 QoS profile and an SLRB. The PC5 QoS profile contains the following parameters:

i)	the PC5 QoS profile contains a PQI;

ii)	if the PQI of the PC5 QoS profile identifies a GBR QoS, the PC5 QoS profile contains a PC5 flow bit rates consisting of a guaranteed flow bit rate (GFBR) and a maximum flow bit rate (MFBR);

iii)	if the PQI of the PC5 QoS profile does not identify a GBR QoS, the PC5 QoS profile contains the PC5 link aggregated bit rate consisting of a per link aggregate maximum bit rate (PC5 LINK-AMBR);

iv)	the PC5 QoS profile contains a range; and

v)	the PC5 QoS profile can contain the priority level, the averaging window, and the maximum data burst volume. If one or more of the priority level, the averaging window or the maximum data burst volume are not contained in the PC5 QoS profile, their default values apply.