3GPP TSG-CT WG1 Meeting #124-e	C1-203953

- Electronic meeting; 2-10 June 2020	(was C1-203349)

Source:	Huawei, HiSilicon

Title:	Network monitoring procedure; V2X UE subscription for network monitoring information

Spec:	3GPP TS 24.486 v1.1.0

Agenda item:	16.2.12

Document for:	Agreement

1. Introduction

This p-CR defines the V2X UE subscription for the network monitoring procedure for 3GPP TS 24.486.

2. Reason for Change

The specification needs to define the stage 3 details of the V2X UE subscription for the network monitoring procedure defined in 3GPP TS 23.286 clause 9.7.2 and 9.7.3.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.486 v1.1.0.

* * * First Change * * * *

## 6.11	Network monitoring by the V2X UE procedure

6.11.1	V2X UE subscription for network monitoring information

#### 6.11.1.1	Client procedure

In order to subscribe for the network monitoring information from the VAE-S, the VAE-C shall send an HTTP POST request according to procedures specified in IETF RFC 2616 [19]. In the HTTP POST request, the VAE-C:

a)	shall set the Request-URI to the URI corresponding to the identity of the VAE-S;

b)	shall include a Content-Type header field set to "application/vnd.3gpp.vae-info+xml";

c)	shall include an application/vnd.3gpp.vae-info+xml MIME body with a <subscription-request> element in the <VAE-info> root element:

1)	shall include a <identity> element with a <V2X-UE-id> child element set to the identity of the UE which requests the registration;

2)	shall include a <subscription-events> element with one or more <event> child element set to the network monitoring events (e.g. uplink degradation, congestion, overload, coverage) to be subscribed; and

3)	shall include a <triggering-criteria> element set to the criteria to indicate when the VAE-S sends the monitoring reports to the VAE-C;

#### 6.11.1.2	Server procedure

Upon reception of an HTTP POST request message containing:

a)	a Content-Type header field set to "application/vnd.3gpp.vae-info+xml"; and

b)	an application/vnd.3gpp.vae-info+xml MIME body with a <subscription-request> element in the <VAE-info> root element;

the VAE-S:

a)	shall store the received geographical area information if the VAE-C is authorized and allowed to access the network monitoring information;

b)	shall include with a <V2X-UE-id> child element within the <identity> element of the <subscription-response> element, and set it to the identity of the UE which requests to subscribe for the network monitoring information from the VAE-S; and

c)	shall reply with a HTTP response with a <result> element of the <subscription-response> element set to a value "success" or "fail".

* * * End of Change * * * *