3GPP TSG-CT WG1 Meeting #125-e	C1-204976

Electronic meeting, 20-28 August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.548 | CR | 0002 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to identity element of MBMS bearers request |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL |  |  |  |  |  | Date: |  |  |  | 2020-08-08 |  |
| Category: | F |  |  |  |  |  |  | Release: |  |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |  |
| Reason for change: | The specification specifies the identity element of MBMS bearers request in clause 6.2.3.2.1: ii) an <identity> element set to the identity of the VAL group that the MBMS bearer is requested for; However, clause 7.3.3 states: The <request> element shall include: a) a <requester-indentity> element; b) a <VAL-group-id> element; and clause 7.5.3 states: <request> is an element used to include the multicast resource management requested information. The <request> element contains the following sub-elements: a) <requester-indentity>, an element contains the identity of the VAL server performing the request; b) <VAL-group-id>, an element contains the identity of the VAL group that the MBMS bearer is requested for; In Stage 2 of TS23.434, quote of clause 14.3.2.10: #### 14.3.2.10 MBMS bearers request Table 14.3.2.10-1 describes the information flow for the MBMS bearers request from VAL server to NRM server. Table 14.3.2.10-1: MBMS bearers request Information element; Status; Description. Requester Identity; M; The identity of the VAL server performing the request.. VAL group ID; M; The identity of the group that the MBMS bearer is requested for.. Service announcement mode; M; Indicates whether the request is sent by NRM server or by the VAL server. QoS; M; Indicates the requested QoS for the bearer. Broadcast area; O; Indicate the area where the MBMS bearer is requested for. Endpoint information; M; Information of the endpoint of the VAL server to which the user plane notifications have to be sent.. Therefore, the identity element should be corrected to be consistent. |
| Summary of change: | 1. Correct the identity element description of MBMS bearers request. |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: | 1. Inconsistent specification. |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: | 6.2.3.2.1 |  |  |  |  |  |  |  |  |  |  |  |
|  | Y |  | N |  |  |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.2.3.2.1	VAL server procedure

When a user originates a request for a VAL service group communication session for one of these areas, in order to use the pre-established MBMS bearers, the VAL server shall generate an HTTP POST request according to procedures specified in IETF RFC 2616 [15]. In the HTTP POST request message, the VAL server:

a)	shall include a Request-URI set to the URI corresponding to the identity of the SNRM-S;

b)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-mbms-usage-info +xml";

c)	shall include an application/vnd.3gpp.seal-mbms-usage-info+xml MIME body and in the <mbms-info> root element:

1)	shall include an <request> element which shall include:

i)	a <requester-identity> element set to the identity of the VAL server performing the request;

ii)	a <VAL-group-id> element set to the identity of the VAL group that the MBMS bearer is requested for;

iii)	a <service-anouncement-mode> indicating whether the request is sent by NRM server or by the VAL server;

iv)	a <QoS> element indicating the requested QoS for the bearer;

v)	an optional <broadcast-area> element indicating the area where the MBMS bearer is requested for; and

vi)	an <endpoint-info> element set to the information of the endpoint of the VAL server to which the user plane notifications have to be sent; and

d)	shall send the HTTP POST request message towards the SNRM-S according to IETF RFC 2616 [15].

* * * End of Change * * * *