3GPP TSG-CT1 Meeting #125-e 	C1-205213

# Online,  , 20th Aug 2020 - 28th Aug 2020 											(was C1-204878)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.519 | CR | 0010 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update PSFP stream identification parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The PSFP Stream filter instance specification in TS 24.519 clause 9.8 is incomplete as the parameter StreamHandleSpec only refers to a IEEE 802.1CB tsnStreamIdEntry object in Stream identity table. However, the IEEE 802.1CB Stream identity table is only known to the TSN AF and not available in the DS-TT and NW-TT. 8; 7; 6; 5; 4; 3; 2; 1. Length of Stream filter instance contents; octet 4. StreamHandleSpec value; octet 5 octet 8. PrioritySpec value; octet 9 octet 12. StreamGateInstanceID value; octet 13 octet 16. Figure 9.8.2: Stream filter instance Instead of sending StreamHandleSpec value it is proposed to send the information from the referred tsnStreamIdEntry of Stream identity table which is based on tsnStreamIdIdentificationType as specified in IEEE 802.1CB clause 9.1.1.6: 9.1.1.6 tsnStreamIdIdentificationType An enumerated value indicating the method used to identify packets belonging to the Stream. The enumeration includes an Organizationally Unique Identifier (OUI) or Company ID (CID) to identify the organization defining the enumerated type. The values defined by this standard are shown in Table 9-1. Please note this issue is also addressed in stage2 as part of CR#2403 to TS 23.501 (see S2-2005028). |
| Summary of change: |  | The Stream filter instance is enhanced with the tsnStreamIdIdentificationType and the input parameters needed for Stream identification function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DS-TT/NW-TT not able to perform PSFP stream identification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 9.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 23.501 CR 2403 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1 - Update missing change marks |  |  |  |  |  |  |  |  |

*** First change ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[5]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[5A]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane nodes".

[5B]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[6]	IEEE 802.1AB-2016: "IEEE Standard for Local and metropolitan area networks -- Station and Media Access Control Connectivity Discovery".

[7]	IEEE 802.1Q-2018: "Standard for Local and metropolitan area networks--Bridges and Bridged Networks".

[8]	Void

[9]	IEEE 802.1Qcc-2018: "Standard for Local and metropolitan area networks - Bridges and Bridged Networks - Amendment: Stream Reservation Protocol (SRP) Enhancements and Performance Improvements".

[10]	IEEE 802.1CB-2017: "IEEE Standard for Local and metropolitan area networks-Frame Replication and Elimination for Reliability".

*** Next change ***

## 9.8	Stream filter instance table

The purpose of the Stream filter instance table information element is to convey a Stream filter instance table as defined 3GPP TS 23.501 [2] table 5.28.3.1-1.

The Stream filter instance table information element is coded as shown in figure 9.8.1, figure 9.8.2, figure 9.8.3, figure 9.8.4, figure 9.8.5, figure 9.8.6, and table 9.8.1.

The Stream filter instance table is a type 6 information element with a minimum length of 3 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stream filter instance table IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Stream filter instance table contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| Stream filter instance 1 |  |  |  |  |  |  |  | octet 4* octet m* |
| … |  |  |  |  |  |  |  |  |
| Stream filter instance n |  |  |  |  |  |  |  | octet n* octet o* |

Figure 9.8.1: Stream filter instance table information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of Stream filter instance contents |  |  |  |  |  |  |  | octet 4 |
| PrioritySpec value |  |  |  |  |  |  |  | octet 5 octet 8 |
| StreamGateInstanceID value |  |  |  |  |  |  |  | octet 9 octet 12 |
| tsnStreamIdIdentificationType value |  |  |  |  |  |  |  | octet 13 octet 16 |
| tsnStreamIdParameters |  |  |  |  |  |  |  | octet 17 octet m |

Figure 9.8.2: Stream filter instance

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of tsnStreamIdParameters contents |  |  |  |  |  |  |  | octet 17 |
| tsnCpeNullDownDestMac value |  |  |  |  |  |  |  | octet 18 octet 23 |
| tsnCpeNullDownTagged value |  |  |  |  |  |  |  | octet 24 |
| tsnCpeNullDownVlan value |  |  |  |  |  |  |  | octet 25 octet 26 |

Figure 9.8.3: tsnStreamIdParameters for tsnStreamIdIdentificationType = 00-80-C2 01

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of tsnStreamIdParameters contents |  |  |  |  |  |  |  | octet 17 |
| tsnCpeSmacVlanDownSrcMac value |  |  |  |  |  |  |  | octet 18 octet 23 |
| tsnCpeSmacVlanDownTagged value |  |  |  |  |  |  |  | octet 24 |
| tsnCpeSmacVlanDownVlan value |  |  |  |  |  |  |  | octet 25 octet 26 |

Figure 9.8.4: tsnStreamIdParameters for tsnStreamIdIdentificationType = 00-80-C2 02

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of tsnStreamIdParameters contents |  |  |  |  |  |  |  | octet 17 |
| tsnCpeDmacVlanDownDestMac value |  |  |  |  |  |  |  | octet 18 octet 23 |
| tsnCpeDmacVlanDownTagged value |  |  |  |  |  |  |  | octet 24 |
| tsnCpeDmacVlanDownVlan value |  |  |  |  |  |  |  | octet 25 octet 26 |
| tsnCpeDmacVlanDownPriority value |  |  |  |  |  |  |  | octet 27 |
| tsnCpeDmacVlanUpDestMac value |  |  |  |  |  |  |  | octet 28 octet 33 |
| tsnCpeDmacVlanUpTagged value |  |  |  |  |  |  |  | octet 34 |
| tsnCpeDmacVlanUpVlan value |  |  |  |  |  |  |  | octet 35 octet 36 |
| tsnCpeDmacVlanUpPriority value |  |  |  |  |  |  |  | octet 37 |

Figure 9.8.5: tsnStreamIdParameters for tsnStreamIdIdentificationType = 00-80-C2 03

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of tsnStreamIdParameters contents |  |  |  |  |  |  |  | octet 17 |
| tsnCpeIpIdDestMac value |  |  |  |  |  |  |  | octet 18 octet 23 |
| tsnCpeIpIdTagged value |  |  |  |  |  |  |  | octet 24 |
| tsnCpeIpIdVlan value |  |  |  |  |  |  |  | octet 25 octet 26 |
| tsnCpeIpIdIpType value |  |  |  |  |  |  |  | octet 27 |
| tsnCpeIpIdIpSource value |  |  |  |  |  |  |  | octet 28 octet a |
| tsnCpeIpIdIpDestination value |  |  |  |  |  |  |  | octet a+1 octet b |
| tsnCpeIpIdDscp value |  |  |  |  |  |  |  | octet b+1 |
| tsnCpeIpIdNextProtocol value |  |  |  |  |  |  |  | octet b+2 |
| tsnCpeIpIdSourcePort value |  |  |  |  |  |  |  | octet b+3 octet b+4 |
| tsnCpeIpIdDestinationPort value |  |  |  |  |  |  |  | octet b+5 octet b+6 |

Figure 9.8.6: tsnStreamIdParameters for tsnStreamIdIdentificationType = 00-80-C2 04

Table 9.8.1: Stream filter instance table

Value part of the Stream filter instance table information element (octets 4 to o)

Stream filter instance table contents (octets 4 to o) This field consists of zero or more Stream filter instances.

Stream filter instance (octets 4 to m)

Length of Stream filter instance contents (octet 4) Length of Stream filter instance contents contains the length of the value part of Stream filter instance in octets.

PrioritySpec value (octets 5 to 8) PrioritySpec value contains the value of PrioritySpec as specified in IEEE 802.1Q [7] table 12-32.

StreamGateInstanceID value (octets 9 to 12) StreamGateInstanceID value contains the value of StreamGateInstanceID as specified in IEEE 802.1Q [7] table 12-32.

tsnStreamIdIdentificationType value (octets 13 to 16) tsnStreamIdIdentificationType value contains the value of tsnStreamIdIdentificationType in the form of four octets as specified in IEEE 802.1CB [10] clause 9.1.1.6. The first 3 octets contain the binary encoding of Organizationally Unique Identifier (OUI) or Company ID (CID). The 4th octet contains the binary encoded value of type number. IEEE 802.1CB [10] currently only specifies OUI/CID value 00-80-C2 with type number value 1, 2, 3 and 4. Other type numbers are reserved. Other OUI/CID values are outside the scope of the present document.

tsnStreamIdParameters (octets 17 to m)

Length of tsnStreamIdParameters (octet 17) Length of tsnStreamIdParameters contents contains the length of the value part of tsnStreamIdParameters in octets.

tsnCpeNullDownDestMac value (octets 18 to 23) tsnCpeNullDownDestMac value contains the value of tsnCpeNullDownDestMac as specified in IEEE 802.1CB [10] clause 9.1.2.1.

tsnCpeNullDownTagged value (octet 24) tsnCpeNullDownTagged value contains an enumerated value of tsnCpeNullDownTagged as specified in IEEE 802.1CB [10] clause 9.1.2.2 in the form of a binary encoded octet. Value “tagged” is encoded as binary 0, value “priority” is encoded as binary 1, and value “all” is encoded as binary 2. All other values are reserved.

tsnCpeNullDownVlan value (octets 25 to 26) tsnCpeNullDownVlan value contains the value of tsnCpeNullDownVlan as specified in IEEE 802.1CB [10] clause 9.1.2.3.

tsnCpeSmacVlanDownSrcMac value (octets 18 to 23) tsnCpeSmacVlanDownSrcMac value contains the value of tsnCpeSmacVlanDownSrctMac as specified in IEEE 802.1CB [10] clause 9.1.3.1.

tsnCpeSmacVlanDownTagged value (octet 24) tsnCpeSmacVlanDownTagged value contains an enumerated value of tsnCpeSmacVlanDownTagged as specified in IEEE 802.1CB [10] clause 9.1.3.2 in the form of a binary encoded octet. Value “tagged” is encoded as binary 0, value “priority” is encoded as binary 1, and value “all” is encoded as binary 2. All other values are reserved.

tsnCpeSmacVlanDownVlan value (octets 25 to 26) tsnCpeSmacVlanDownVlan value contains the value of tsnCpeSmacVlanDownVlan as specified in IEEE 802.1CB [10] clause 9.1.3.3.

tsnCpeDmacVlanDownDestMac value (octets 18 to 23) tsnCpeDmacVlanDownDestMac value contains the value of tsnCpeDmacVlanDownDestMac as specified in IEEE 802.1CB [10] clause 9.1.4.1.

tsnCpeDmacVlanDownTagged value (octet 24) tsnCpeDmacVlanDownTagged value contains an enumerated value of tsnCpeDmacVlanDownTagged as specified in IEEE 802.1CB [10] clause 9.1.4.2 in the form of a binary encoded octet. Value “tagged” is encoded as binary 0, value “priority” is encoded as binary 1, and value “all” is encoded as binary 2. All other values are reserved.

tsnCpeDmacVlanDownVlan value (octets 25 to 26) tsnCpeDmacVlanDownVlan value contains the value of tsnCpeDmacVlanDownVlan as specified in IEEE 802.1CB [10] clause 9.1.4.3.

tsnCpeDmacVlanDownPriority value (octet 27) tsnCpeDmacVlanDownPriority value contains the value of tsnCpeDmacVlanDownPriority as specified in IEEE 802.1CB [10] clause 9.1.4.4.

tsnCpeDmacVlanUpDestMac value (octets 28 to 33) tsnCpeDmacVlanUpDestMac value contains the value of tsnCpeDmacVlanUpDestMac as specified in IEEE 802.1CB [10] clause 9.1.4.5.

tsnCpeDmacVlanUpTagged value (octet 34) tsnCpeDmacVlanUpTagged value contains an enumerated value of tsnCpeDmacVlanUpTagged as specified in IEEE 802.1CB [10] clause 9.1.4.6 in the form of a binary encoded octet. Value “tagged” is encoded as binary 0, value “priority” is encoded as binary 1, and value “all” is encoded as binary 2. All other values are reserved.

tsnCpeDmacVlanUpVlan value (octets 35 to 36) tsnCpeDmacVlanUpVlan value contains the value of tsnCpeDmacVlanUpVlan as specified in IEEE 802.1CB [10] clause 9.1.4.7.

tsnCpeDmacVlanUpPriority value (octet 37) tsnCpeDmacVlanUpPriority value contains the value of tsnCpeDmacVlanUpPriority as specified in IEEE 802.1CB [10] clause 9.1.4.8.

tsnCpeIpIdDestMac value (octets 18 to 23) tsnCpeIpIdDestMac value contains the value of tsnCpeIpIdDestMac as specified in IEEE 802.1CB [10] clause 9.1.5.1.

tsnCpeIpIdTagged value (octet 24) tsnCpeIpIdTagged value contains an enumerated value of tsnCpeIpIdTagged as specified in IEEE 802.1CB [10] clause 9.1.5.2 in the form of a binary encoded octet. Value “tagged” is encoded as binary 0, value “priority” is encoded as binary 1, and value “all” is encoded as binary 2. All other values are reserved.

tsnCpeIpIdVlan value (octets 25 to 26) tsnCpeIpIdVlan value contains the value of tsnCpeIpIdVlan as specified in IEEE 802.1CB [10] clause 9.1.5.3.

tsnCpeIpIdIpType value (octet 27) tsnCpeIpIdIpType value contains the IP address type of tsnCpeIpIdIpSource and tsnCpeIpIdIpDestination as specified in IEEE 802.1CB [10] clause 9.1.5.4 and 9.1.5.5 in the form of a binary encoded octet. Value 1 indicates an IPv4 address type, and value 2 indicates an IPv6 address type. All other values are reserved.

tsnCpeIpIdIpSource value (octets 28 to a) tsnCpeIpIdIpSource value contains the value of tsnCpeIpIdIpSource as specified in IEEE 802.1CB [10] clause 9.1.5.4. In case of tsnCpeIpIdIpType indicating an IPv6 address type the value of tsnCpeIpIdIpSource is in the form of 16 binary encoded octets. In case of tsnCpeIpIdIpType indicating an IPv4 address type the value of tsnCpeIpIdIpSource is in the form of 4 binary encoded octets.

tsnCpeIpIdIpDestination value (octets a+1 to b) tsnCpeIpIdIpDestination value contains the value of tsnCpeIpIdIpDestination as specified in IEEE 802.1CB [10] clause 9.1.5.5. In case of tsnCpeIpIdIpType indicating an IPv6 address type the value of tsnCpeIpIdIpDestination is binary encoded in the form of 16 octets. In case of tsnCpeIpIdIpType indicating an IPv4 address type the value of tsnCpeIpIdIpDestination is binary encoded in the form of four octets.

tsnCpeIpIdDscp value (octet b+1) tsnCpeIpIdDscp value contains the value of tsnCpeIpIdDscp as specified in IEEE 802.1CB [10] clause 9.1.5.6.

tsnCpeIpIdNextProtocol value (octet b+2) tsnCpeIpIdNextProtocol value contains the value of tsnCpeIpIdNextProtocol as specified in IEEE 802.1CB [10] clause 9.1.5.7.

tsnCpeIpIdSourcePort value (octets b+3 to b+4) tsnCpeIpIdSourcePort value contains the value of tsnCpeIpIdSourcePort as specified in IEEE 802.1CB [10] clause 9.1.5.8.

tsnCpeIpIdDestinationPort value (octets b+5 to b+6) tsnCpeIpIdDestinationPort value contains the value of tsnCpeIpIdDestinationPort as specified in IEEE 802.1CB [10] clause 9.1.5.9.