- 3GPP TSG CT WG1 Meeting 125-e	TDoc C1-205xxx

- Electronic meeting, 20-28 August 2020

Title:	LS on clarifications for authorized user learning about the users whose floor requests are queued

Response to:

Release:	Release 17

Work Item:	MCProtoc17

Source:	3GPP TSG CT WG1

To:	3GPP TSG SA WG6

Cc:

Contact Person:

#### Name:	Kiran Kapale

Tel. Number:	+919886088631

E-mail Address:	kiran dot kapale at samsung dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

CT1 is working on MCPTT enhancements for Rel-17 for implementation of the stage 2 requirements that are not yet implemented in stage 3. We have found the following relevant requirements in TS 23.379:

3GPP TS 23.379:

Information flows:

10.9.1.2.5	Floor request cancel

10.9.1.2.6	Floor request cancel response

10.9.1.2.7	Floor request cancel notify

Procedures:

10.9.1.3.4.1               Floor request cancellation from the queue – MCPTT user initiated

10.9.1.3.4.2               Floor request cancellation from the queue - floor control server initiated

CT1 would like to get clarification from SA6 for the following issues:

- There is no capability for a user, authorised or not, to get a list of those users whose floor request is queued. How does SA6 envision the authorised user learning the contents of the list of users queued with floor requests in order to be able to specify the list of users to be cancelled from the floor queue?

  - CT1 notes that, if a list of queued users is provided to an authorised user, by the time the authorised user gets the list, the list has a very high probability of having changed.

- Would SA6 consider a procedure allowing an authorised user to request clearing the entire queue instead of specifying a list of users to be cleared?

- CT1 also do not understand the purpose of allowing the floor control server to choose to cancel users waiting in the floor request queue, since the floor control server has no knowledge of the purpose and value of the call other than the priority used by each group member. Was it intended that the ability of the floor control server to cancel users from the floor request queue be used to handle dropping lower priority users when the queue is full?

CT1 requests SA6 to provide clarifications for the above issues to help guide CT1 in MCPTT stage 3 development.

2. Actions:

To SA1 group.

ACTION: 	CT1 kindly asks SA6 to kindly provide clarifications on the issues listed above to help guide CT1 in MCPTT stage 3 development.

3. Date of Next SA6 Meetings:

CT1#126 15th – 23th October 2020	e-meeting