3GPP TSG-CT WG1 Meeting #126-e	C1-206001

Electronic meeting, 15-23 October 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.486 | CR | 0036 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to server procedure of V2X UE subscription for network monitoring information procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | V2XAPP |  |  |  |  |  | Date: |  |  | 2020-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The HTTP response message of the server procedure of V2X UE subscription for network monitoring information procedure is unclear, quote of Clause 6.6.2: #### 6.9.1.2 Server procedure Upon reception of an HTTP POST request message containing: a) a Content-Type header field set to "application/vnd.3gpp.vae-info+xml"; and b) an application/vnd.3gpp.vae-info+xml MIME body with a <subscription-info> element in the <VAE-info> root element; the VAE-S: a) shall store the received subscription information if the VAE-C is authorized and allowed to access the network monitoring information; b) shall include a <V2X-UE-id> child element within the <identity> element of the <subscription-info> element, and set it to the identity of the UE which requests to subscribe for the network monitoring information from the VAE-S; and c) shall reply with a HTTP response with a <result> element of the <subscription-info> element set to a value "success" or "fail". The HTTP response needs to be detailed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Specify the HTTP response message; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The HTTP response message of the server procedure of V2X UE subscription for network monitoring information procedure is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.9.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 6.9.1.2	Server procedure

Upon reception of an HTTP POST request message containing:

a)	a Content-Type header field set to "application/vnd.3gpp.vae-info+xml"; and

b)	an application/vnd.3gpp.vae-info+xml MIME body with a <subscription-info> element in the <VAE-info> root element;

the VAE-S:

a)	shall store the received subscription information if the VAE-C is authorized and allowed to access the network monitoring information;

b)	shall generate an HTTP 200 (OK) response according to IETF RFC 2616 [19]. In the HTTP 200 (OK) response message, the VAE-S:

1)	shall include a Content-Type header field set to "application/vnd.3gpp.vae-info+xml"; and

2)	shall include an application/vnd.3gpp.vae-info+xml MIME body with a <network-monitoring-subscription> element in the <VAE-info> root element:

i)	shall include a <V2X-UE-id> element set to the identity of the V2X UE subscribing the network monitoring information; and

ii)	shall include a <result> child element set to the value "success" or "failure" indicating success or failure of subscribing the network monitoring information; and

b)	shall send the HTTP 200 (OK) response towards the VAE-C.

* * * End of Change * * * *