3GPP TSG-CT WG1 Meeting #126-e	C1-206219

Electronic meeting, 15-23 October 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 2733 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Restriction in the usage of the 5GSM STATUS message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GProtoc17 |  |  |  |  |  | Date: |  |  | 2020-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | The following case has been identified: - The UE sends a PDU SESSION RELEASE COMPLETE message as a response to a PDU SESSION RELEASE COMMAND message from the SMF. - The AMF cannot forward the PDU SESSION RELEASE COMPLETE message and hence a DL NAS TRANSPORT message including 5GMM cause #90 "payload was not forwarded" sent to the UE. - The UE sends a 5GSM STATUS message with the 5GSM cause #96 "invalid mandatory information". - The AMF cannot forward the 5GSM STATUS message and hence a DL NAS TRANPSROT message including 5GMM cause #90 "payload was not forwarded" sent to the UE - The UE sends a 5GSM STATUS message with the 5GSM cause #43 "invalid PDU session identity". - Infinite repetition of Steps 4 and 5. The usage of the 5GSM STATUS message is very broad as can be seen from the TS: The purpose of the sending of the 5GSM STATUS message is to report at any time certain error conditions detected upon receipt of 5GSM protocol data. The 5GSM STATUS message can be sent by both the network and the UE (see example in figure 6.5.1.1). Thus, there is no means to avoid the case based on the current text. When a 5GSM message is received via a DL NAS TRANSPORT message including a 5GMM cause IE, detailed handling is specified in clause 5.4.5.3 and corresponding 5GSM procedure description. Therefore, there is no need for a 5GSM STATUS message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that 5GSM protocol data received via a DL NAS TRANSPORT message including a 5GMM cause IE should not be a trigger for the 5GSM status procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The problematic case described above cannot be avoided. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.5.1	General

The purpose of the sending of the 5GSM STATUS message is to report at any time certain error conditions detected upon receipt of 5GSM protocol data except when 5GSM protocol data was received via a DL NAS TRANSPORT message including a 5GMM cause IE. The 5GSM STATUS message can be sent by both the network and the UE (see example in figure 6.5.1.1).

Figure 6.5.1.1: 5GSM status procedure