3GPP TSG-CT WG1 Meeting #126-e	C1-205911

- Electronic meeting; 15-23 October 2020

Source:	Qualcomm Incorproated

Title:	Key Issue #1: Determination of the country of the UE location

Spec:	3GPP TR 24.821

Agenda item:	17.2.4

Document for:	Agreement

1. Introduction

This contribution proposed to add Key Issue #1: Determination of the country of the UE location” to TR 24.821

2. Reason for Change

In [1], SA3-LI has established the requirement to “enforce the use of a Core Network of PLMN in the country where the UE is physically located”. This means that the UE needs to be able to determine the country of its physical location. This goes beyond the requirements in the legacy PLMN selection procedure in TS 23.122, where the country is defined by the MCC of the serving PLMN.  MCC-based determination of the country would not work for satellite access because:

-	A 3GPP satellite access cell may span across multiple countries;

-	There are global/shared PLMN IDs with MCC that does not correspond to any country (MCC 9xx); and

-	In international areas, the UE cannot rely on the MCC.

Therefore, for PLMN selection for satellite access, the determination of the country of the UE’s physical location is a valid issue for which the solution needs to be provided.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.821.

4. References

[1]	S3i200056 Response LS on the “LS OUT on Location of UEs and associated key issues”

* * * First Change * * * *

## 5.1	Key issue #1: Determination of the country of the UE’s location

In [1], SA3-LI has established the requirement to "enforce the use of a Core Network of PLMN in the country where the UE is physically located". This means that, if the UE is able to determine the country of its physical location, it needs to select a PLMN belonging to the country. This goes beyond the requirements in the legacy PLMN selection procedure in 3GPP TS 23.122 [x]., where the country is defined by the MCC of the serving PLMN.  MCC-based determination of the country would not work for satellite access because:

-	a 3GPP satellite access cell may span across multiple countries;

-	there are global/shared PLMN IDs with MCC that does not correspond to any country (MCC 9xx); and

-	in international areas, the UE cannot rely on the MCC.

The following questions are expected to be studied within this key issue:

-	how the UE determines the country of its physical location and whether it  is in the scope of 3GPP;

-	what assumptions can/should be made about the methods for determining UE location (e.g. GNSS availability etc.) and the UE capabilities;

-	how the UE maps its physical geo-location (e.g. lat/long) into a country of its location; and

-	determination that the UE is located in international areas;

-	to what extent and under which conditions the UE could still rely on the MCC of the serving PLMN to determine the country of its physical location; and

-	the need to change or update the definition of the “country” in 3GPP TS 23.122 [x] and the impact of the new defiontion on the PLMN selection procedure.