3GPP TSG-CT WG1 Meeting #126-e	C1-205916

- Electronic meeting; 15-23 October 2020

Source:	Qualcomm Incorproated

Title:	Key issue #6: PLMN search intervals for satellite access

Spec:	3GPP TR 24.821

Agenda item:	17.2.4

Document for:	Agreement

1. Introduction

This contribution proposed to add Key Issue #6 PLMN search intervals for PLMN access.

2. Reason for Change

Search interval for periodic search for higher priority PLMNs is specified in 23.122. Currently, we have two different search intervals, for non-NB-IoT UE, and for NB-IoT UE, respectively. What interval will be used for PLMN search in satellite access needs to be studied.

In the legacy PLMN selection procedure, the UE searches for higher priority PLMN only among PLMNs with the same MCC as the RPLMN. It needs to be studied whether this rule should remain for satellite access.

The values for the intervals for the periodic search for higher priority PLMNs is a valid issue for which the solution needs to be provided.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.821.

* * * First Change * * * *

# Key issue #6: PLMN search for satellite access

The range for the search interval for periodic search for higher priority PLMNs is specified in 23.122. Currently, there are two different search interval ranges: for non-NB-IoT UE, and for NB-IoT UE, respectively. What interval will be used for PLMN search in satellite access needs to be studied.

In the legacy PLMN selection procedure, the UE searches for higher priority PLMN only among PLMNs with the same MCC as the RPLMN. It needs to be studied whether this rule should remain for satellite access.

The following questions are expected to be studied within this key issue:

-	Whether there are different priority levels for available PLMNs in satellite access?

-	Whether the periodic search for higher priority PLMNs in satellite access is needed?

-	Whether the need to have a separate range of values for the search interval for periodic search for higher priority PLMNs in satellite access;

-	Whether the search for higher priority PLMN should be limited to PLMNs where MCC matches the MCC of the RPLMN