3GPP TSG-CT WG1 Meeting #126-e	C1-206199

- Electronic meeting; 15-23 October 2020

Source:	SHARP

Title:	Proposal of solution for Key issue X found in C1-206198

Spec:	3GPP TR 23.700-10 v0.0.0

Agenda item:	17.3.3

Document for:	Agreement

1. Introduction

CT1 study for Enhanced IMS to 5GC Integration Phase 2 (FS_eIMS5G2) has started at CT1#125e.

2. Reason for Change

This solution addresses key issue#x to ensure IMS service uses a specific 5GC slice in the scenario of routing IMS traffic via multiple slices to a single or multiple IMS network. Specification of the key issue#x is found in C1-206198.

Given the fact that available S-NSSAI may be different per RA, the available S-NSSAI for IMS service may be different.

Considering above, this solution utilizes URSP and Allowed NSSAI for UE selecting a suitable S-NSSAI associated to area for IMS service.

3. Proposal

It is proposed to adopt the following solution in TR 23.700-10 v0.0.0 for key issue#x.

* * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.502: "Procedures for the 5G System".

* * * * Next change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

NSSAI	Network Slice Selection Assistance Information

S-NSSAI	Single NSSAI

URSP	UE Route Selection Policy

* * * * Next change * * * *

## 6.x	Solution #X: 5GC network slice selection for IMS traffic route via dedicated slice(s)

### 6.x.1	Description

### 6.x.1.1	General

This solution address key issue#x (Network Slicing and IMS) to ensure IMS service uses specific slice(s) in the 5GC.

The solution describes how the UE selects S-NSSAI(s) for IMS service including SIP signalling and media.

It is proposed that all IMS signalling related information is conveyed to the IMS network via a dedicated 5GC network slice and that media associated to a particular IMS session can be routed via different 5G network slice(s) based on the application's requirements.

When establishing a 5GS PDU session for IMS service, the UE selects an S-NSSAI from list of S-NSSAIs included in Allowed NSSAI based on URSP rules.

A URSP rule can e.g. contain an S-NSSAI for IMS service with associated registration area. If the current registration area associated to an S-NSSAI in Allowed NSSAI is available in the registration area associated to the S-NSSAI for IMS service in URSP rules, then UE selects the S-NSSAI for IMS service at current registration area. At IMS session setup, URSP rule(s) can also be used to establish dedicated 5GS PDU session(s) over the same or new S-NSSAI(s).

### 6.x.1.2	Procedure

The following flow describes the procedure for the UE selecting an S-NSSAI for IMS service.

Figure 6.x.1.2: Procedure to select an S-NSSAI for IMS service

The steps required are as follows:

1.	The UE performs registration procedure as described in clause 4.2.2 of 3GPP TS 23.502 [2]. Here, the registration accept message includes for example S-NSSAI #1.

2.	The AMF and PCF performs UE Policy Association Establishment as described in clause 4.16.11 of 3GPP TS 23.502 [2].

3.	Along with step 2, the PCF performs UE Configuration Update procedure as described in clause 4.2.4.3 of 3GPP TS 23.502 [2]. The MANAGE UE POLICY COMMAND message contains a URSP and the URSP contains for example S-NSSAI #1 for IMS service with available registration area and S-NSSAI #2 for other traffic.

4.	When UE requests to establish a 5GS PDU session for IMS service, if the UE determines S-NSSAI #1 in Allowed NSSAI is usable in the current registration area based on the URSP in which S-NSSAI #1 is associated to available registration area, then the UE selects S-NSSAI #1 for IMS service.

### 6.x.2	Impacts on existing nodes and functions

Editor's note:	This clause describes impacts to existing entities and interfaces.

### 6.x.3	Solution Evaluation

Editor's note:	This clause provides an evaluation of the solution.