3GPP TSG-CT WG1 Meeting #126-e	C1-20ccdd

Electronic meeting, 15-23 October 2020                                                   was C1-206439

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 2815 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on description of triggering UE to enter 5GMM-DEREGISTERED state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GProtoc17 |  |  |  |  |  | Date: |  |  | 2020-10-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | One additional scenario of triggering UE to enter 5GMM-DEREGISTERED state need to be clarified in sub-clause 5.2.2.1. The scenario is described below: The emergency attached UE enters in 5GMM-IDLE mode and then its periodic registration update timer expires; In TS 24.501, subclause 5.3.7 it indicates that in this scenario, emergency attached UE shall locally de-register from the network. A similar clarification is already present in TS24.301, section 5.2.2.1 - the UE attached for emergency bearer services is in EMM-IDLE mode and its periodic tracking area update timer expires (see subclause 5.3.5). which is required in TS24.501 also. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the below mentioned condition for triggering UE to enter 5GMM-DEREGISTERED state to sub-clause 5.2.2.1: - the UE attached for emergency services is in 5GMM-IDLE mode and its periodic registration update timer expires (see subclause 5.3.7). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing entry condition of triggering UE to enter 5GMM-DEREGISTERED state. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** FIRST CHANGE ****

#### 5.2.2.1	General

The state 5GMM-DEREGISTERED is entered in the UE, when:

a)	the de-registration is performed either by the UE or by the network (see subclause 5.5.2);

b)	the registration request is rejected by the AMF (see subclause 5.5.1.2.5 and 5.5.1.3.5);

c)	the service request is rejected by the AMF (see subclause 5.6.1);

d)	the UE is switched on; or

e)	the UE registered for emergency services is in 5GMM-IDLE mode and its periodic registration update timer expires (see subclause 5.3.7).

In state 5GMM-DEREGISTERED, the UE shall behave according to the substate as explained in subclause 5.2.2.3.

**** END OF CHANGES ****