3GPP TSG-CT WG1 Meeting #127bis-e	C1-210xxx

- Electronic meeting; 25-29 January 2021												(was C1-210119)

Source:	Qualcomm Incorporated

Title:	Solution to MINT Key Issue #6 (Notification that Disaster Condition is no longer applicable to the UEs)

Spec:	TR 24.811 v0.1.0

Agenda item:	17.2.9

Document for:	Agreement

1. Introduction

At CT1#127-e, Key Issue #6 on notification that Disaster Condition is no longer applicable to the UEs was agreed into MINT TR 24.811. The purpose of this contribution is to propose a solution for this Key Issue.

2. Reason for Change

It is proposed to specify the following:

-	upon being notified that a Disaster Condition no longer applies, the PLMNs without Disaster Condition may:

1)	turn off the notification to UEs (e.g. SIB flag) that a Disaster Condition applies;

2)	turn on a notification to UEs (e.g. SIB flag) that a Disaster Condition in another PLMN no longer applies;

3)	for the UEs in connected mode, perform a generic UE configuration update procedure with an indication that the Disaster Condition in another PLMN no longer applies, or trigger the UE to perform a mobility registratio update (using the generic UE configuration update procedure) followed by a registration reject with a new 5GMM cause value causing the UE to return to the PLMN previously with Disaster Condition; and

4)	 for the UEs in idle mode attempting to transition to connected mode, reject the UE’s request with a new 5GMM cause value causing the UE to return to the PLMN previously with Disaster Condition.

-	while camped on a PLMN without Disaster Condition, a UE shall periodically check the indication (e.g. SIB flag) from the serving PLMN that a Disaster Condition applies, if signalled, and check for an indication (e.g. SIB flag) from the serving PLMN that the Disaster Condition in another PLMN no longer applies.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 24.811 v0.1.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.011: "Service accessibility".

[3]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[x]	3GPP TS 23.122: "Non-Access-Stratum functions related to Mobile Station (MS) in idle mode".

* * * Next Change * * * *

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| E |  |  |  |  |  | X |  |  |

* * * Next Change * * * *

## 6.E	Solution #E: Solution for notification that Disaster Condition is no longer applicable to the UEs

### 6.E.1	Introduction

This is a solution for Key Issue #6 (Notification that Disaster Condition is no longer applicable to the UEs).

### 6.E.2	Detailed description

Upon being notified that a Disaster Condition in PLMN D no longer applies, a PLMN without Disaster Condition (PLMN A) currently serving Disaster Inbound Roamers may perform one or more of the following:

a)	if PLMN A was broadcasting an indication (e.g. in SIB) that a Disaster Condition in PLMN D applied, turn off the indication , and optionally start broadcasting an indication (e.g. in SIB) that the Disaster Condition in PLMN D no longer applies;

Editor's note: Extension of broadcast signalling is subject to agreement of RAN WGs.

Editor's note: Dynamic update of broadcast signalling is subject to agreement of RAN WGs.

Editor's note:	Input from SA3 is needed regarding potential security risks resulting from using broadcast signalling to convey information related to disaster roaming.

NOTE:	For how long the indication that the Disaster Condition in PLMN D no longer applies is broadcast by PLMN A depends on PLMN A’s operator policy.

b)	for the Disaster Inbound Roamers in 5GMM-CONNECTED mode:

1)	perform a generic UE configuration update procedure with an indication that the Disaster Condition in another PLMN no longer applies; or

2)	perform a generic UE configuration update procedure with "re-registration requested", then reject the UE’s registration request with 5GMM cause #ZZZ "disaster condition in other PLMN no longer applies"; or

c)	for the Disaster Inbound Roamers in 5GMM-IDLE mode which attempt to transition to 5GMM-CONNECTED mode by initiating a registration or service request procedure, reject the UE’s request with 5GMM cause #ZZZ "disaster condition in other PLMN no longer applies".

While camped on PLMN A, a Disaster Inbound Roamer shall periodically check the indication (e.g. SIB flag) broadcast by PLMN A that a Disaster Condition applies in PLMN D, if signalled, and check for an indication (e.g. SIB flag) broadcast by PLMN A that the Disaster Condition in PLMN D no longer applies.

Editor's note: How often the UE shall check for these broadcast indications is FFS.

Upon:

a)	detecting that the indication (e.g. SIB flag) broadcast by PLMN A that a Disaster Condition applies in PLMN D has been turned off;

b)	detecting that the indication (e.g. SIB flag) broadcast by PLMN A that the Disaster Condition in PLMN D no longer applies has been turned on;

c)	receiving a CONFIGURATION UPDATE COMMAND message with an indication that the Disaster Condition in another PLMN no longer applies; or

d)	receiving a REGISTRATION REJECT or SERVICE REJECT message with 5GMM cause #ZZZ "disaster condition in other PLMN no longer applies";

the UE shall remove the stored indication that a Disaster Condition applies to PLMN D, enter 5GMM-REGISTERED.PLMN-SEARCH and perform PLMN selection as specified in 3GPP TS 23.122 [x] subclause 4.4.3.1.1.

### 6.E.3	Impacts on existing nodes and functionality

UE

-	support for handling of the indication broadcast by PLMN A (e.g. SIB flag) that a Disaster Condition in D PLMN applies;

-	support for handling of the indication broadcast by PLMN A (e.g. SIB flag) that a Disaster Condition in D PLMN no longer applies;

-	support for CONFIGURATION UPDATE COMMAND message with an indication that the Disaster Condition in another PLMN no longer applies; and

-	support for 5GMM cause #ZZZ "disaster condition in other PLMN no longer applies ".

AMF of registered PLMN (without Disaster Condition)

- optionally, support for CONFIGURATION UPDATE COMMAND message with an indication that the Disaster Condition in another PLMN no longer applies; and

- optionally, support for 5GMM cause #ZZZ "disaster condition in other PLMN no longer applies ".

RAN of registered PLMN (without Disaster Condition)

- optionally, support for signalling the indication (e.g. SIB flag) that a Disaster Condition in PLMN D applies; and

- optionally, support for signalling the indication (e.g. SIB flag) that a Disaster Condition in PLMN D no longer applies.

* * * End of Changes * * * *