3GPP TSG-CT WG1 Meeting #127bis-e	C1-210179

Electronic meeting, 25-29 January 2021

Source:	vivo

Title:	MINT: Architectural Requirements

Spec:	3GPP TR 24.811

Agenda item:	17.2.9

Document for:	Agreement

2. Reason for Change

The clause of Architectural Requirements is still empty.

4. Proposal

It proposes to adopt the following considerations into the Architectural Requirements for the minimization of service interruption:

Solutions shall build on the 5G System architectural principles as in TS 23.501 [x], including flexibility and modularity for newly introduced functionalities.

In order to satisfy the normative stage-1 general requirements in TS 22.261 [3] subclause 6.31.2.2 and subclause 6.31.2.3, the system shall:

-	enable UEs to obtain information that a Disaster Condition applies to a particular PLMN or PLMNs;

-	enable a UE to access PLMNs in a forbidden PLMN list if a Disaster condition applies and no other PLMN is available except for PLMNs in the forbidden PLMN list;

-	support means for a PLMN operator to be aware of the area where Disaster Condition applies;

-	enable to support the provision of service to UEs only within the specific region where Disaster Condition applies;

-	enable a PLMN operator to be made aware of the failure or recovery of other PLMN(s) in the same country when the Disaster Condition is applied, or when the Disaster Condition is not applicable;

-	enable to provide efficient means for a network to inform UEs that a Disaster Condition is no longer applicable;

-	be able to provide a resource efficient means for a PLMN to indicate to potential Disaster Inbound Roamers whether they can access the PLMN or not;

-	enable UE to perform network reselection when a Disaster Condition has ended;

-	minimize congestion caused by Disaster Roaming; and

-	be able to collect charging information for a Disaster Inbound Roamer with information about the applied disaster condition.

* * * Change * * *

## 4.2	Architectural Requirements

The system shall satisfy the stage-1 requirements in TS 22.261 [3] subclause 6.31.2 and TS 22.011 [2] subclause 3.2.2.