3GPP TSG-CT WG1 Meeting #127-e	C1-207136

Electronic meeting, 13-20 November 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.587 | CR | 0155 | rev | - | Current version: | 16.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removing cause #6 "authentication failure" from the list of expected causes for PC5 unicast link security mode control procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17, eV2XARC |  |  |  |  |  | Date: |  |  | 2020-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | The cause #6 "authentication failure" is listed under the causes that can be received in The DIRECT LINK SECURITY MODE REJECT message for PC5 unicast link security mode control procedure. But there is no usage for that cause within the PC5 unicast link security mode control procedure, and that cause is used only within the authentication procedure as can be seen in the following subclause: ##### 6.1.2.6.5 PC5 unicast link authentication procedure not accepted by the target UE If the DIRECT LINK AUTHENTICATION REQUEST message cannot be accepted, the target UE shall create a DIRECT LINK AUTHENTICATION REJECT message. In this message, the target UE shall include a PC5 signaling protocol cause IE indicating one of the following cause values: #a: authentication failure. In fact, this cause can't be received within the PC5 unicast link security mode control procedure since this procedure is used to establish the security context and not to authenticate the peer UE. Authenticating the peer UE happens using the PC5 unicast link authentication procedure. Hence the cause #6 "authentication failure" shall be removed from the list of expected causes in DIRECT LINK SECURITY MODE REJECT message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing cause #6 "authentication failure" from the list of expected causes in DIRECT LINK SECURITY MODE REJECT message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The error will stay in the specs which can lead to ambiguity and wrong implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

##### 6.1.2.7.5	PC5 unicast link security mode control procedure not accepted by the target UE

If the DIRECT LINK SECURITY MODE COMMAND message cannot be accepted, the target UE shall send a DIRECT LINK SECURITY MODE REJECT message and abort the ongoing procedure that triggered the initiation of the PC5 unicast link security mode control procedure. The DIRECT LINK SECURITY MODE REJECT message contains a PC5 signalling protocol cause IE indicating one of the following cause values:

#b:	integrity failure;

#c:	UE security capabilities mismatch;

#d:	LSBs of KNRP-sess ID conflict;

#e:	UE PC5 unicast signalling security policy mismatch; or

#111:	protocol error, unspecified.

If the DIRECT LINK SECURITY MODE COMMAND message cannot be accepted because the PC5 unicast link security mode control procedure was triggered during a PC5 unicast link establishment procedure, that the selected security algorithms in the DIRECT LINK SECURITY MODE COMMAND message included the null integrity protection algorithm and the target UE’s PC5 unicast signalling integrity protection policy is set to "signalling integrity protection required", the target UE shall include PC5 signalling protocol cause #e "UE PC5 unicast signalling security policy mismatch" in the SECURITY MODE REJECT message.

If the DIRECT LINK SECURITY MODE COMMAND message cannot be accepted because the PC5 unicast link security mode control procedure was triggered during a PC5 unicast link re-keying procedure, the integrity protection algorithm currently in use for the PC5 unicast link is different from the null integrity protection algorithm and the selected security algorithms in the DIRECT LINK SECURITY MODE COMMAND message include the null integrity protection algorithm, the target UE, the target UE shall include PC5 signalling protocol cause #e "UE PC5 unicast signalling security policy mismatch" in the SECURITY MODE REJECT message.

Upon receipt of the DIRECT LINK SECURITY MODE REJECT message, the initiating UE shall stop timer T5007 and:

a)	if the PC5 signalling protocol cause IE in the DIRECT LINK SECURITY MODE REJECT message is set to #d, retransmit the DIRECT LINK SECURITY MODE COMMAND message with a different value for the 8 LSBs of KNRP-sess ID; and

***** End of changes *****