3GPP TSG-CT WG1 Meeting #128-e	C1-210728

- Electronic meeting; 25 February – 5 March 2021

Source:	Qualcomm Incorporated

Title:	Solution to Key Issue #9

Spec:	TR 24.811 v0.2.0

Agenda item:	17.2.9

Document for:	Agreement

1. Reason for Change

Solution #21 includes a solution for manual PLMN selection while a disaster condition applies, which was previously included in the scope of Key Issue #5 and was later moved to its own Key Issue (Key Issue #9). As a result, the corresponding text needs to be removed from Solution #21 (see C1-210724) and moved to a new Solution for Key Issue #9 by the present pCR.

Additionally, it is proposed to resolve the Editor’s note on handling of CAG cells and CAG supporting UEs in the PLMN without Disaster Condition for manual PLMN selection currently included in Solution #21 by specifying that when a disaster condition applies, the UE presents to the user the PLMN/access technology combination for CAG cells even if the CAG cells broadcast a CAG-ID which is not in the UE’s Allowed CAG list and do not broadcast that the PLMN allows a user to manually select the CAG-ID.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 24.811 v0.2.0.

* * * First Change * * * *

## 6.X	Solution #X: Solution for manual PLMN selection when a "Disaster Condition" applies

### 6.X.1	Introduction

This is a solution for Key Issue #9 (Handling of Disaster inbound roaming PLMNs in Manual PLMN selection).

### 6.X.2	Detailed description

The UE shall perform PLMN selection as specified in 3GPP TS 23.122 [7] subclause 4.4.3.1.2, with the following addition:

a)	if all of the available PLMNs are in the UE’s "forbidden PLMNs" list, the UE may indicate whether the available PLMNs can accommodate Disaster Inbound Roamers.

Editor's note:	Handling of CAG cells and CAG supporting UEs in the PLMN without Disaster Condition is FFS.

### 6.X.3	Impacts on existing nodes and functionality

UE

-	optionally, support for indicating that a PLMN available for manual selection can accommodate Disaster Inbound Roamers.

* * * End of Changes * * * *