3GPP TSG-CT WG1 Meeting #128-e	C1-210729

- Electronic meeting; 25 February – 5 March 2021

Source:	Qualcomm Incorporated

Title:	Evaluation of solutions for Key Issue #7

Spec:	3GPP TR 24.811 v0.2.0

Agenda item:	17.2.9

Document for:	Agreement

1. Reason for Change

This document proposes an evaluation of solutions for FS_MINT Key Issue # 7 (Prevention of signalling overload in PLMNs without Disaster Condition).

2. Proposal

It is proposed to agree the following changes to 3GPP TR 24.811 v0.2.0.

* * * First Change * * * *

# 7	Evaluations

Editor's note:	This clause will describe the evaluations on the solutions proposed in clause 6.

## 7.7	Key Issue #7: Prevention of signalling overload in PLMNs without Disaster Condition

Solution #1:

a)	only addresses the following question of Key Issue #7:

How to stagger the arrival of UEs in the PLMNs without Disaster Condition, so as to spread out registration attempts over time and keep the number of UEs attempting to register simultaneously within a manageable limit

b)	provides a solution for the question above ofKey Issue#7 if all of the following conditions are met:

1) the UE:

i) supports the non-3GPP access in addition to the 3GPP access;

ii) supports NAS over the non-3GPP access;

iii) supports connecting to N3WIF;

iv) was registered to the same PLMN over 3GPP and non-3GPP access before (and when) the disaster condition occurred;

v) is in 5GMM-CONNECTED mode over the non-3GPP access; and

vi) has not registered to another PLMN over the 3GPP access while the disaster condition is ongoing; and

2)	the PLMN with Disaster Condition:

i)	has N3IWF, and the AMF of the PLMN supports the N2 connection from the N3IWF; and

ii)	has a non-3GPP access network which is not affected by the Disaster Condition.

As such, Solution #1 cannot be the only solution to progress to normative phase and other solutions also need to be specified for fully address Key Issue #7;

c)	enables the network to provide the UE with an "expected duration of disaster" timer. This timer may not always be accurate with respect to the time when the disaster condition actually ends;

d)	enables the network to provide the UE with a list of PLMNs, optionally prioritized, for disaster roaming, which is similar to what is proposed in Solutions #39, #41 and #43; and

e)	relies on providing a "wait timer" to the UE to stagger the arrival of UEs in the PLMNs without Disaster Condition, which is similar to what is proposed in Solutions #39 and #43.

Solution #16:

a)	only addresses the following questions of Key Issue #7:

How to distribute the subscribers of the PLMN with Disaster Condition between the PLMNs without Disaster Condition available in the area where the Disaster Condition applies, so as to share the load as evenly as possible between the PLMNs without Disaster Condition; and

How to use new Access Identity 3 for the purpose of Disaster Inbound Roamer access control and signalling overload prevention in the PLMNs without Disaster Condition;

As such, Solution #16 is not sufficient to fully address Key Issue #7 and must be supplemented by other solutions addressing the remaining questions; and

b)	enables pre-configuration in the UE, or provisioning of the UE over NAS signalling, with the information required to distribute the UEs between the PLMNs which can accommodate disaster inbound roamers;

c)	makes use of existing SIB parameters;

d)	requires SA1 agreement to allocate new Access Identity values for disaster roaming; and

e)	requires RAN2 agreement to update the semantics of uac-BarringForAccessIdentity.

Solution #36:

a)	only addresses the following question of Key Issue #7:

How to enable a PLMN without Disaster Condition to efficiently prevent Disaster Inbound Roamers from attempting registration on the PLMN when the PLMN can no longer accept Disaster Inbound Roamers due to congestion;

As such, Solution #36 is not sufficient to fully address Key Issue #7 and must be supplemented by other solutions addressing the remaining questions; and

b)	relies on rejecting Disaster Inbound Roamers with a new 5GMM cause value indicating that the resources are not sufficient for the Disaster Inbound Roamers that triggers the UE to look for another PLMN, which is similar to what is proposed in Solution #39.

Solution #37:

a)	only addresses the following question of Key Issue #7:

How to enable a PLMN without Disaster Condition to efficiently prevent Disaster Inbound Roamers from attempting registration on the PLMN when the PLMN can no longer accept Disaster Inbound Roamers due to congestion;

As such, Solution #37 is not sufficient to fully address Key Issue #7 and must be supplemented by other solutions addressing the remaining questions;

b)	requires RAN2 agreement to create a new RRC establishment cause; and

c)	does not enable the UE to know that the rejection is due to congestion caused by the arrival of Disaster Inbound Roamers.

Solution #38:

a)	only addresses the following questions of Key Issue #7:

How to stagger the arrival of UEs in the PLMNs without Disaster Condition, so as to spread out registration attempts over time and keep the number of UEs attempting to register simultaneously within a manageable limit; and

How to enable a PLMN without Disaster Condition to efficiently prevent Disaster Inbound Roamers from attempting registration on the PLMN when the PLMN can no longer accept Disaster Inbound Roamers due to congestion.

As such, Solution #38 is not sufficient to fully address Key Issue #7 and must be supplemented by other solutions addressing the remaining questions;

b)	proposes to change the way that barring is applied for Access Identities, by associating a barring factor with Access Identity 3, whereas per current RAN2 specifications, barring factors are only associated with Access Categories. This requires RAN2 agreement. Additionally, it is not clear what the advantages of this approach are as compared to just creating a new Access Category for Disaster Inbound Roamers (as proposed in Solution #42).

Solution #39:

a)	only addresses the following questions of Key Issue #7:

How to distribute the subscribers of the PLMN with Disaster Condition between the PLMNs without Disaster Condition available in the area where the Disaster Condition applies, so as to share the load as evenly as possible between the PLMNs without Disaster Condition;

How to stagger the arrival of UEs in the PLMNs without Disaster Condition, so as to spread out registration attempts over time and keep the number of UEs attempting to register simultaneously within a manageable limit; and

How to enable a PLMN without Disaster Condition to efficiently prevent Disaster Inbound Roamers from attempting registration on the PLMN when the PLMN can no longer accept Disaster Inbound Roamers due to congestion.

As such, Solution #39 is not sufficient to fully address Key Issue #7 and must be supplemented by other solutions addressing the remaining questions;

b)	enables the network to provide the UE with a prioritized or weighted list of PLMNs for disaster roaming, which is similar to what is proposed in Solutions #1, #41 and #43;

c)	relies on providing a "disaster roaming wait range" to the UE to stagger the arrival of UEs in the PLMNs without Disaster Condition, which is similar to what is proposed in Solutions #1 and #43;

d)	proposes a new 5GMM cause value that can be used by the PLMNs without disaster when they encounter congestion due to the arrival of Disaster Inbound Roamers and that trigger the UE to look for another PLMN, which is similar to what is proposed in Solution #36; and

e)	proposes the use of a broadcast indicator to indicate that the PLMN can accommodate / no longer accommodate Disaster Inbound Roamers, which requires feedback from RAN2 and SA3.

Solution #40:

a)	only addresses the following questions of Key Issue #7:

How to stagger the arrival of UEs in the PLMNs without Disaster Condition, so as to spread out registration attempts over time and keep the number of UEs attempting to register simultaneously within a manageable limit; and

How to enable a PLMN without Disaster Condition to efficiently prevent Disaster Inbound Roamers from attempting registration on the PLMN when the PLMN can no longer accept Disaster Inbound Roamers due to congestion.

As such, Solution #40 is not sufficient to fully address Key Issue #7 and must be supplemented by other solutions addressing the remaining questions;

b)	proposes to change the way that barring is applied for Access Identities, by introducing a new offset value to the unified access control barring information. This requires RAN2 agreement. This solution enables the disaster roaming PLMN to setup stricter barring factor only affecting disaster inbound roamer UE for any access category it may use during in that PLMN..

Solution #41:

a)	only addresses the following question of Key Issue #7:

How to distribute the subscribers of the PLMN with Disaster Condition between the PLMNs without Disaster Condition available in the area where the Disaster Condition applies, so as to share the load as evenly as possible between the PLMNs without Disaster Condition;

As such, Solution #41 is not sufficient to fully address Key Issue #7 and must be supplemented by other solutions addressing the remaining questions;

b)	enables the network to provide the UE with a prioritized list of recommended PLMNs for disaster roaming, which is similar to what is proposed in Solutions #1, #39 and #43.

Solution #42:

a)	only addresses the following question of Key Issue #7:

How to stagger the arrival of UEs in the PLMNs without Disaster Condition, so as to spread out registration attempts over time and keep the number of UEs attempting to register simultaneously within a manageable limit;

How to use new Access Identity 3 for the purpose of Disaster Inbound Roamer access control and signalling overload prevention in the PLMNs without Disaster Condition; and

How to enable a PLMN without Disaster Condition to efficiently prevent Disaster Inbound Roamers from attempting registration on the PLMN when the PLMN can no longer accept Disaster Inbound Roamers due to congestion.

As such, Solution #42 is not sufficient to fully address Key Issue #7 and must be supplemented by other solutions addressing the remaining questions; and

b)	relies on the creation of a new Access Category, which is subject to SA1 agreement;

Solution #43:

a)	only addresses the following question of Key Issue #7:

How to distribute the subscribers of the PLMN with Disaster Condition between the PLMNs without Disaster Condition available in the area where the Disaster Condition applies, so as to share the load as evenly as possible between the PLMNs without Disaster Condition;

How to stagger the arrival of UEs in the PLMNs without Disaster Condition, so as to spread out registration attempts over time and keep the number of UEs attempting to register simultaneously within a manageable limit;

As such, Solution #43 is not sufficient to fully address Key Issue #7 and must be supplemented by other solutions addressing the remaining questions;

b)	enables the network to provide the UE with a prioritized list of PLMNs for disaster roaming, which is similar to what is proposed in Solutions #1, #39 and #41; and

c)	relies on providing a "minimum wait time" to the UE to stagger the arrival of UEs in the PLMNs without Disaster Condition, which is similar to what is proposed in Solutions #1 and #39; and

Summary:

The following key points can be observed from the evaluation above:

Observation 1: None of the solutions address all questions of Key Issue #7, which suggests that it will be necessary to combine pieces of different solutions for normative work.

Observation 2: One solution (Solution #1) relies on the use of non-3GPP access in specific conditions. It is not sufficient to address Key Issue #7 in all cases, but it could co-exist with solutions based on the use of 3GPP access.

Observation 3: Four solutions (Solutions #1, #39, #41 and #43) rely on the use of a prioritized list of PLMNs provided to the UE to distribute the subscribers of the PLMN with Disaster Condition between the PLMNs without Disaster Condition. When the disaster condition happens the congestion situation may be changing dynamically. A finer adjustment by the PLMNs offering disaster roaming is not possible when this information is pre-provisioned. The pre-configured distribution among various PLMNs assumes there is an equal probablity of finding all disaster roaming PLMNs in any location UE may find itself in during a disaster condition. If it is not true, all UEs may end up in the same PLMN (the only one available in the disaster area irrespective of the priority provisioned to the UEs). It requires the operator to create, maintain up-to-date and provision such information to all subscribers and roamers. The pre-configured information cannot apply to roamers who has not registered with the PLMN with disaster condition earlier.

Observation 4: Three solutions (Solutions #1, #39 and #43) rely on the use of a wait time of some kind at the UE to stagger the arrival of UEs in the PLMNs without Disaster Condition. As compared to only providing a "wait timer" (as in Solution #1) or a "minimum wait time" (as in Solution #43), providing a range and having the UE draw a random value within that range (as in Solution #39) has the following advantages:

a)	it provides an upper bound for the wait time, thereby limiting the service interruption; and

b)	it removes the need for the network to allocate different "wait timer" values to different UEs to achieve spreading out the registration attempts over time, since the randomization of the registration time is done at the UE.

Observation 5: Two solutions (Solutions #36 and #39) rely on the use of new 5GMM cause value indicating that the resources are not sufficient for the Disaster Inbound Roamers and which triggers the UE to look for another PLMN. This could result into a UE trying in sequence to register with all PLMNs offering Disaster Roaming to the UE and getting rejected in all those PLMNs.

Observation 6: Three solutions (Solutions #38, #40 and #42) build on top of Unified access control concept which was introduced in 5GS with an intention to avoid diverse Reject with back-off timer (RRC, NAS) mechanisms and provide a unified access control framework.

* * * Next Change * * * *