3GPP TSG-CT WG1 Meeting #128-e	C1-21xxxx

- Electronic meeting; 25 February – 5 March 2021

Source:	InterDigital

Title:	Solution for Key Issue #4

Spec:	3GPP TR 24.811

Agenda item:	17.2.9

Document for:	Agreement

1. Introduction

This contribution proposes a solution to one of the questions in Key Issue #4.

2. Reason for Change

In the TS 24.811 v0.2.0, the following question of Key Issue#4 was proposed:

-	How a Disaster Roaming PLMN can limit the area of service to Inbound Disaster Roamers to the region where Disaster Condition applies

4. Proposal

It is proposed to agree the following changes to 3GPP TR 24.811 v0.2.0.

* * * First Change * * * *

## 6.X	Providing information to the RAN of PLMN A

### 6.X.1	Introduction

This is a solution for the following question of KI#4:

-	How a Disaster Roaming PLMN can limit the area of service to Inbound Disaster Roamers to the region where Disaster Condition applies

In this solution, PLMN D is the PLMN with Disaster Condition and PLMN A is the PLMN that accepts inbound disaster roamers.

This solution assumes that

- the UE has been informed of the Dissater Condition in PLMN D

- the UE is about to register in PLMN A

### 6.X.2	Detailed description

The UE will provide the RAN of PLMN A with information to select proper AMF.

The above is achieved by the following steps below:

-	during the RRC Connection Establishment in PLMN A, the UE will send to the RAN of PLMN A, an indication that it is an “Inbound Roamer” due to “Disaster Condition”. This new information is sent, in addition to the GUAMI and 5G-S-TMSI of the UE

-	The RAN will then use these parameters in order to select a special AMF that the PLMN A has designated for Disaster Inbound Roamers

-	In order to spread the inbound (disaster) roamers, the RAN nodes should be configured by the operator to choose different AMF entities. This choice could be based on the geographic area and/or a priority order.

Editor’s Note: Whether the addition of the parameter to the RRC message is a feasible solution is left for the RAN      WGs to decide.

### 6.X.3	Impacts on existing nodes and functionality

UE:

-	Needs to include a new indicaiton that the UE is a Diaster Inbound Roamer during the RRC Connection establishment