- 3GPP TSG CT WG1 Meeting 130-e	TDoc C1-213395

- Electronic meeting, 20-28 May 2021

Title:	Reply LS on introducing extended DRX for RedCap UEs

Response to:	LS on introducing extended DRX for RedCap UEs (C1-212839/R2-2104374)

Release:	Rel-17

Work Item:	NR_redcap-Core

Source:	CT1

To:	RAN2

Cc:	SA2, RAN3

Contact person:	Lin Shu

shulin@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

CT1 thanks RAN2’s LS on introducing extended DRX for RedCap UEs.

CT1 has discussed RAN2 agreements indicated in the LS and would like to provide following feedback:

- It is feasible to support eDRX for 5GMM-IDLE(RRC_IDLE) with eDRX cycle up to 10485.76s for RedCap UEs in NR/5GCN if the AMF knows the UE is a RedCap UE and eDRX is used for it. However, CT1 would like to note that it does not make sense if the maximum supported eDRX value is larger or equal to the periodic update timer used for RedCap UEs and currently the default value of the periodic update timer is 54m.

- In order to support eDRX for RRC_INACTIVE with eDRX cycle up to 10485.76s for RedCap UEs in NR/5GCN, to avoid NAS message retransmission at the network, the NAS retransmission timer needs to be extended to about 3h and to avoid NAS procedure failure, the NAS retransmission timer needs to be extended to about 0.6h (= 36m).

- The NAS retransmission timer extended to about 3h can cause some problems related to NAS, e.g. bad user experience due to delayed DL NAS procedure, the AMF can be easily overloaded due to suspend all DL 5GSM messages, and NAS procedure collisions happen more often.

# 2	Actions

To 3GPP RAN2

ACTION: 	3GPP CT1 kindly asks RAN2 to take above feedback into account.

# 3	Dates of next TSG CT WG1 meetings

TBD