- 3GPP TSG CT WG1 Meeting 130-e	TDoc C1-213000

- Electronic meeting, 20-28 May 2021

Title:	Reply LS on Small data transmission

Response to:	LS R2-2104644/C1-212849 on Small data transmission from RAN2

Release:	Rel-17

Work Item:	5GProtoc17

Source:	ZTE

To:	RAN2

Cc:	SA2

Contact person:	Shuang Liang

liang.shuang3@zte.com.cn

+86-15850539616

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

CT1 thanks TSG RAN for their LS on Small data transmission.

CT1 would like to confirm that the actual transport mechanism (e.g., SDT or UP-EDT etc) used by lower layers to transmit the pending data/NAS procedure is transparent to NAS layer. The UE NAS layer will handle the pending uplink user data packet and signalling in the RRC_INACTIVE state as legacy.

Further for the question of RAN2:

RAN2 agreed that only radio bearers configured for SDT are resumed and additional UL and DL data can be exchanged between UE and network as part of a given SDT session while the UE is still in RRC_INACTIVE (i.e. without transition to RRC_CONNECTED). In this case, if new UL data or NAS message becomes available for non-SDT radio bearers (which are suspended), would it be possible that NAS triggers another request to transition into RRC_CONNECTED and provides access category, access identities and resume cause.

CT1 would like to provide the following feedback:

While the UE is in 5GMM-CONNECTED mode with RRC inactive indication and receives an indication from the lower layers that the RRC connection has been suspended, any pending NAS signalling  (except a REGISTRATION REQUEST message with the NG-RAN-RCU bit of the 5GS update type IE set to "UE radio capability update needed") or uplink data packet(which is not distinguished by the NAS layer), the NAS layer can trigger the lower layers to transition to RRC_CONNECTED state.

# 2	Actions

To RAN2

ACTION: 	3GPP TSG CT WG1 asks RAN2 to take the above feedback into account.

# 3	Dates of next TSG CT WG1 meetings

TSG CT WG1 Meeting 131-e (electronic)	19th -27th August