3GPP TSG-CT WG1 Meeting #130-e	C1-212911

- Electronic meeting; 20th – 28th May

Source:	Qualcomm Incorporated, Nokia

Title:	Conclusion for Solution 2

Spec:	3GPP TR 24.821

Agenda item:	17.2.4

Document for:	Agreement

1. Introduction

TR 24.821 contains two solutions for key issue #2: PLMN selection in international areas. No new solutions for this key issue is expected to be added to the TR.

2. Reason for Change

There is a need for the conclusion to key issue #2.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 24.821 v1.1.0.

* * * First Change * * * *

## 8.x	Conclusion for key issue 2

The following is the list of the issues studied in this key issue and the respective conclusions for the normative phase:

a)	how the UE determines which PLMN(s) can be selected in determined country of its physical location (if it is determined that the UE is not in an international area); and

b)	what information could be available to the UE to help in selecting, in a given country, a PLMN whose CN is physically located in the same country as the UE:

-	pre-configured in the UE; and

-	provided by PLMN.

For a), the following conclusion is adopted:

i)	The UE prioritizes the PLMN/RAT combinations that the UE determines as allowed to operate in the country of the UE location over the PLMN/RAT combinations that the UE has no information about whether they are allowed to operate in the country fo the UE location or not  The UE determines the PLMNs allowed to operate in the country of the UE location based on the mapping between one or more MCC(s) and one or more PLMN(s) allowed to operate int he country of the MCC. The mapping is provisioned in the UE;

ii)	If the UE has received the reject cause #78 and MCC IE from the serving PLMN, the UE uses that information to filter out the serving PLMN and to set the MCC of the country of the UE location, respectively, in the PLMN selection procedure; and

iii)	In other aspects, and if the UE is not provisioned with the mapping described in i),  the UE follows the legacy PLMN selection procedure (i.e. PLMN selection for terrestrial accesss).

NOTE 1: If only PLMNs that the UE determines as not allowed to provide service in the country of the UE location are available, the UE behavior will be specified in the normative phase. For example, the UE could perform PLMN selection by ignoring the mapping described in i).

For b), the following conclusion is adopted:

i)	The UE is optionally provisioned with the mapping between one or more MCC(s) and one or more PLMN(s) that are allowed to operate in the country of the MCC; and

ii)	The mapping can be stored in the ME or in the USIM,

Editor’s note:	For the mapping stored in the ME, whether the provisioning can be delivered via a mechanism specified by CT1 or not is FFS.

NOTE 2: 	How the USIM configuration is encoded is up to CT6 to decide.

* * * No more changes * * * *