3GPP TSG-CT WG1 Meeting #130-e	C1-212911

- Electronic meeting; 20th – 28th May

Source:	Qualcomm Incorporated, Nokia, Nokia Shanghia Bell

Title:	Conclusion for Solution 2

Spec:	3GPP TR 24.821

Agenda item:	17.2.4

Document for:	Agreement

1. Introduction

TR 24.821 contains two solutions for key issue #2: PLMN selection in international areas. No new solutions for this key issue is expected to be added to the TR.

2. Reason for Change

There is a need for the conclusion to key issue #2.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 24.821 v1.1.0.

* * * First Change * * * *

## 8.x	Conclusion for key issue 2

The following is the list of the issues studied in this key issue and the respective conclusions for the normative phase:

a)	how the UE determines which PLMN(s) can be selected in determined country of its physical location (if it is determined that the UE is not in an international area); and

b)	what information could be available to the UE to help in selecting, in a given country, a PLMN whose CN is physically located in the same country as the UE.

-	pre-configured in the UE; and

-	provided by PLMN.

For a), the following conclusion is adopted:

i)	The UE prioritizes the PLMN/RAT combinations that the UE determines as allowed to operate in the country of the UE location over:

-	the PLMN/RAT combinations that the UE has no information about whether they are allowed to operate in the country for the UE location or not; or

-	the PLMN/RAT combinations that the UE determines as not allowed to operate in the country of the UE location.

The UE determines the PLMNs allowed to operate in the country of the UE location based on information provided by the network and/or an  optional mapping between one or more countries and one or more PLMNs that are allowed to operate in the respective country(ies) (see bullet b) below);

NOTE 1: Whether the mapping can apply to PLMNs other than HPLMN/EHPLMN is FFS.

ii)	If the UE has received the reject cause #78 and optionally country of the UE location from the serving PLMN’s AMF during registration procedure or attempting access for services while being registered, the UE uses that information to filter out the serving PLMN and to determine the country of the UE location, respectively, in the PLMN selection procedure. The duration and the area for the validity of the information received from the serving PLMN’s AMF will be determined in the normative phase; and

iii)	When the UE is not provisioned with or does not support the optional mapping described in i) and the UE has not received the reject cause #78, the UE follows the legacy PLMN selection procedure (i.e. PLMN selection for terrestrial accesss).

For b), the following conclusion is adopted:

i)	The UE is optionally provisioned with the mapping between one or more countries and one or more PLMNs that are allowed to operate in the respective country(ies); and

ii)	The optional mapping can be stored in the ME or in the USIM,

NOTE 2:	For the mapping in the ME, whether the provisioning is through a 3GPP specific mechanism or is implementation specific is dealt with in normative phase.

NOTE 3: 	How the optional mapping stored in USIM is encoded is up to CT6 to decide.

* * * No more changes * * * *