- 3GPP TSG CT WG1 Meeting 130-e	TDoc C1-21xxxx

- Electronic meeting, 20-28 May 2021

Title:	Reply LS on introducing extended DRX for RedCap UEs

Response to:	LS R2-2104374 on introducing extended DRX for RedCap UEs from RAN2

Release:	Release 17

Work Item:	NR_redcap-Core

Source:	CT1

To:	RAN2, SA2

Cc:	RAN3

Contact person:	Lena Chaponniere

lguellec.chaponniere@gmail.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

CT1 thanks RAN2 for their LS on introducing extended DRX for RedCap UEs. CT1 would like to provide the following feedback:

- In RRC_IDLE, as at extended DRX DL NAS procedures are initiated with consideration of Paging Occasions, extended DRX cycles of up to 10485.76 seconds are already supported since Release 16 for NB-IoT connected to the 5GC. If the AMF knows the UE is a RedCap UE, it is feasible from a NAS protocol point of view to extend this support to RedCap UEs in RRC_IDLE in Release 17 in CT1 specifications. Furthermore, CT1 would like to note that extended DRX cycles have dependency on the periodic update timer value used for RedCap UEs.

- In RRC_INACTIVE, as DL NAS procedures are initiated without consideration of Paging Occasions, from a NAS protocol point of view, in order to support extended DRX cycles of up to 10485.76 seconds, to avoid NAS message retransmission at the network, the NAS retransmission timer needs to be extended to about 3 hours and to avoid NAS procedure failure, the NAS retransmission timer needs to be extended to about 0.6 hr (= 36 minutes). Extending the NAS retransmission timer to about 3 hours can cause some problems related to NAS, e.g. bad user experience due to delayed DL NAS procedure, the AMF can be easily overloaded due to suspending all DL 5GSM messages, and NAS procedure collisions will happen more often.

- There is no UE impact for NAS procedure supervision to support extended DRX cycles.

- CT1 updates can be done only once corresponding RAN2 and SA2 normative requirements are available.

2	Actions

To RAN2 and SA2

ACTION: 	3GPP TSG CT WG1 asks RAN2 and SA2 groups to take the above feedback into account.

# 3	Dates of next TSG CT WG1 meetings

TBD