3GPP TSG-CT WG1 Meeting #130-e	C1-213041r03

- Electronic meeting; 20 – 28  May 2021

Source:	Apple, Convida Wireless, Ericsson, InterDigital, Samsung, LG Electronics, Qualcomm Incorporated, Huawei, HiSilicon

Title:	Conclusion for KI#3

Spec:	3GPP TS 24.811 v1.1.0

Agenda item:	17.2.9

Document for:	Approval

1. Introduction

The pCR presents the text proposal for conclusion section of KI#3 ‘Indication of accessibility from other PLMNs without Disaster Condition to the UE’ in 24.811.

2. Reason for Change

The following table summarizes the topics to be concluded based on solutions for this Key Issue and the outcome from the moderated email discussion.

| Issue | Feedback from Email Discussion | Proposals |
| --- | --- | --- |
| Pre-configuration of PLMN list (Sol#11, Sol#15) | 7 companies are of the view that pre- configuration and/or provisioning of disaster roaming network selection information is needed. Out of this, 1 company wants this to be restricted to home country only. 1 company wanted pre- provisioning/configuration of Access Identity 3 to restrict some UEs from performing disaster roaming 5 companies think that pre- configuration/provisioning of disaster roaming information is not needed. | - Allow networks to pre- provision information. - Allow UEs to perform network selection if there is no PLMN available from pre- provisioned list. Pre- provisioning performed over non-3GPP AN is captured as an EN. |
| Indication of accessibility using access identities (Sol#16) | There was only 1 company in support of using access identities to indicate accessibility from PLMNs offering disaster roaming. | - Recommend not to proceed |
| SIB Broadcast of PLMN IDs (Sol#12,Sol#13) | 8 companies supported providing accesibility information through SIBs. 7 companies are of the opinion that information contains the PLMN IDs for which that PLMN is offering disaster roaming service. | - A PLMN indicates "list of PLMNs" for which it is offering disaster roaming. The design of SIB is upto RAN. |
| PWS broadcast of disaster roaming (Sol#14) | There was only 1 company supporting the method of providing accessibility indication through PWS messages. | - Recommend not to proceed |
| RAN sharing (Sol#10) | Only 1 company supported this method. 9 companies did not support this method and 3 companies did not support it as a mandatory method for providing disaster roaming. | - Recommend not to proceed |

4. Proposal

It is proposed to agree the following changes to 3GPP TR 24.811 v1.1.0.

* * * First Change * * * *

## 8.3	Conclusion of Key Issue #3

For Key Issue#3, the following aspects are concluded.

- The solution #10 is not progressed in the normative phase of FS_MINT-CT.

- The UE shall perform disaster roaming only if HPLMN has configured the UE with a ‘list of PLMN(s) for disaster roaming’ with at least one entry in it. The list is either pre-configured in the USIM or provided by the HPLMN following a successful registration procedure.

- The UE shall not perform disaster roaming if HPLMN has not configured the UE with a ‘list of PLMN(s) for disaster roaming’ or the number of elements in the list is zero.

- While roaming, the Registered PLMN may provide the ‘list of PLMN(s) for disaster roaming’ after a successful registration procedure. The UE shall ignore this information if ‘list of PLMN(s) for disaster roaming’ is empty .

- Provisioning ‘list of PLMN(s) for disaster roaming’ over non-3GPP access is not part of the solution in Rel-17.

- PLMN offering disaster roaming shall indicate accessibility for disaster roamers through SIB messages. The indication may contain the list of PLMN(s) with Disaster condition for which disaster roaming is offered.

- The solution #14 is not progressed in the normative phase of FS_MINT-CT.

- UE of PLMN D with Disaster Condition shall determine that PLMN A offers disaster roaming when PLMN A's NG-RAN cell broadcasts PLMN ID of PLMN D in the list of PLMN(s) with Disaster Condition for which disaster roaming is offered by PLMN A. For PLMN selection, the UE shall prioritize the determined PLMNs offering the disaster roaming which are also included in UE's ‘list of PLMN(s) for disaster roaming’, above the determined PLMNs offering the disaster roaming which are not included in UE's ‘list of PLMN(s) for disaster roaming’.

NOTE: the design of the SIB messages is defined by RAN WG2.

* * * End of Changes * * * *