3GPP TSG-CT WG1 Meeting #130-e	C1-213247

- Electronic meeting; 20-28 May 2021

Source:	InterDigital

Title:	EAS Discovery Notification Procedure

Spec:	3GPP TS 24.558

Agenda item:	17.2.16

Document for:	Agreement

1. Introduction

3GPP TS 24.558 contains a clause on Edge Enabler Server API Definitions, which needs to be completed based on the requirements and procedures spcified in TS 23.558, clause 8.5.

2. Reason for Change

The EAS discovery procedure enables the EEC to obtain information about available EASs of interest. The discovery of the EASs is based on matching EAS discovery filters provided in the request. 3GPP TS 23.558 clause 8.5 provides stage-2 level requirements and signalling flows for the EAS discovery procedure. Clause 8.5.4 of 3GPP TS 23.558 provides the detail requirements for defining the necessary API, which is already included in TS 24.558 and notes were added for FFS where applicable.

TS 23.558 clause 8.5.2.3.2 & 8.5.2.3.3 together illustrate the EAS discovery procedure based on Subscribe/Notify model, which needs to be specified in TS 24.558. Current pCR addresses the following Editors Notes in TS 24.558, clause 6.3.4:

6.3.4: Editor’s note: The definition of notifications is FFS.

6.3.4.1: Editor’s note: It is for FFS if a custom operation is used to allow an EEC to request for EAS discovery information as defined in 3GPP TS 23.558.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.558 v0.2.0.

* * * First Change * * * *

### 6.3.4	Notifications

#### 6.3.4.1	General

Table 6.3.4.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| EAS Discovery Notification | {notificationDestination} | POST | Notifies the subscriber EEC with EAS discovery information. |

#### 6.3.4.2	EAS Discovery Notification

##### 6.3.4.2.1	Description

EAS Discovery notification is used by the EES to notify an EEC with EAS discovery information. The EEC may subscribe to the EAS discovery information as a pre-condition for receiving notification.

##### 6.3.4.2.2	Notification definition

The POST method shall be used by the EES for the notification and the callback URI shall be the one provided by the EEC during the service provisioning subscription.

Callback URI: {notificationDestination}

This method shall support the URI query parameters specified in table 6.3.4.2.2-1.

Table 6.3.4.2.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.3.4.2.2-2 and the response data structures and response codes specified in table 6.3.4.2.2-3.

Table 6.3.4.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EasDiscoveryNotification | M | 1 | Notification of EAS discovery information. |

Table 6.3.4.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

* * * End Change * * * *