- 3GPP TSG CT WG1 Meeting 130-e	TDoc C1-213823

- Electronic meeting, 20-28 May 2021

Title:	Reply LS on Small data transmission

Response to:	LS R2-2104644/C1-212849 on Small data transmission from RAN2

Release:	Rel-17

Work Item:	5GProtoc17

Source:	ZTE

To:	RAN2

Cc:	SA2

Contact person:	Shuang Liang

liang.shuang3@zte.com.cn

+86-15850539616

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

CT1 thanks TSG RAN2 for their LS on Small data transmission.

Before providing our feedback and answers, CT1 would like to ask RAN2 to provide answers to the following questions:

-	Is the actual transport mechanism (i.e., SDT or non-SDT) to transmit the pending uplink user data determined by the lower layers without the NAS layer involvement?

-	Can the first pending uplink NAS message and subsequent multiple rounds of uplink and downlink NAS messages be transported via SDT?

CT1 would like to indicate that TS 24.501 has not specified but left to implementation whether the pending uplink NAS message is provided to the lower layers at the time  when the NAS layer requests the lower layers to transition to RRC_CONNECTED state. Hence, it cannot be gauranteed that the uplink NAS message is avaialble at the lower layers when the lower layers sends the RRC resume request to the network.

For the question of RAN2:

RAN2 agreed that only radio bearers configured for SDT are resumed and additional UL and DL data can be exchanged between UE and network as part of a given SDT session while the UE is still in RRC_INACTIVE (i.e. without transition to RRC_CONNECTED). In this case, if new UL data or NAS message becomes available for non-SDT radio bearers (which are suspended), would it be possible that NAS triggers another request to transition into RRC_CONNECTED and provides access category, access identities and resume cause.

CT1 would like to provide the following feedback on likely (non-exhuastive) impacts:

While the UE is in 5GMM-CONNECTED mode with RRC inactive indication, if there is pending uplink NAS message (except a REGISTRATION REQUEST message with the NG-RAN-RCU bit of the 5GS update type IE set to "UE radio capability update needed") or pending uplink data packets (which are subject to the suspended user plane resources), the impact to NAS will be that NAS layer need to request the lower layers to transition to RRC_CONNECTED state. At such time, NAS will also need to again provide access category, access identities and RRC establishment cause (i.e. resume cause used in RAN2) to the lower layers for purpose of UAC checking.

Furthermore, any newe pending uplink data packets associated with the PDU session with no suspended user plane resources, will require the Service Request procedure is initiated. This will be an additional impact.

In addition to above, CT1 would like RAN2 to take note that, when entering Idle mode, any ongoing SDT session will be interrupted:

The following text can be found in TS 24.501:-

“Upon a trigger to send a REGISTRATION REQUEST message with the NG-RAN-RCU bit of the 5GS update type IE set to "UE radio capability update needed", the UE in 5GMM-CONNECTED mode with RRC inactive indication shall move to 5GMM-IDLE mode.”

# 2	Actions

To RAN2

ACTION: 	3GPP TSG CT WG1 kindly asks RAN2 to provide answers to above questions and to take the provided feedback into account.

# 3	Dates of next TSG CT WG1 meetings

TSG CT WG1 Meeting 131-e (electronic)	19th -27th August