3GPP TSG-CT WG1 Meeting #131-e	C1-214231

# E-meeting, 19-27 August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.486 | CR | 0114 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | XML schema for UE initiated session-oriented service termination procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XAPP |  |  |  |  |  | Date: |  |  | 2021-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The specification needs to define the stage 3 details of the XML schema for UE initiated session-oriented service termination procedure defined in 3GPP TS 23.286 clause 9.19.3.4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add the XML schema for UE initiated session-oriented service termination procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The XML schema for UE initiated session-oriented service termination procedure is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.4.2	XML schema

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="urn:3gpp:ns:vaeInfo:1.0"

xmlns:vaeinfo="urn:3gpp:ns:vaeInfo:1.0"

elementFormDefault="qualified"

attributeFormDefault="unqualified"

xmlns:xenc="http://www.w3.org/2001/04/xmlenc#">

<!-- root XML element -->

<xs:element name="vae-info" type="vaeinfo:vaeinfo-Type" id="vae"/>

<xs:complexType name="vaeinfo-Type">

<xs:sequence>

<xs:element name="registration-info" type="vaeinfo:tRegistrationType" minOccurs="0"/>

<xs:element name="de-registration-info" type="vaeinfo:tDeregistrationType" minOccurs="0"/>

<xs:element name="location-tracking-info" type="vaeinfo:tLocationTrackingType"/>

<xs:element name="message-info" type="vaeinfo:tMessageType" minOccurs="0"/>

<xs:element name="service-discovery-info" type="vaeinfo:tServiceDiscoveryType" minOccurs="0"/>

<xs:element name="local-service-info" type="vaeinfo:tLocalServiceType" minOccurs="0"/>

<xs:element name="layer2-group-id-mapping" type="vaeinfo:tLayer2GroupIDMappingType" minOccurs="0"/>

<xs:element name="network-monitoring-subscription-info" type="vaeinfo:tNetworkMonitoringSubscriptionType" minOccurs="0"/>

<xs:element name="v2x-usd-announcement" type="vaeinfo:tUSDAnnouncementType"/>

<xs:element name="set-pc5-parameters-info" type="vaeinfo:tSetPC5ParametersInfoType"/>

<xs:element name="id-list-notification" type="vaeinfo:tIdListNotificationType" minOccurs="0"/>

<xs:element name="network-monitoring-info-notification" type="vaeinfo:tNetworkMonitoringInfoNotificationType" minOccurs="0"/>

<xs:element name="communication-status-info" type="vaeinfo:tCommunicationStatusInfoType" minOccurs="0"/>

<xs:element name="v2v-communication-assistance-info" type="vaeinfo:tV2vCommunicationAssistanceInfoType" minOccurs="0"/>

<xs:element name="dynamic-group-info-update" type="vaeinfo:tDynamicGroupInfoUpdateType" minOccurs="0"/>

<xs:element name="dynamic-group-info-update-indication" type="vaeinfo:tDynamicGroupInfoUpdateIndicationType" minOccurs="0"/>

<xs:element name="dynamic-group-info-update-consent" type="vaeinfo:tDynamicGroupInfoUpdateConsentType" minOccurs="0"/>

<xs:element name="session-oriented-termination-trigger-info" type="vaeinfo:tSessionOrientedServiceTerminationInfoType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax"/> minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tRegistrationType">

<xs:sequence>

<xs:element name="v2x-ue-id" type="vaeinfo:contentType" minOccurs="0" maxOccurs="1"/>

<xs:element name="v2x-service-id" type="xs:string" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="UE-supported-RATs-list" type="tRATType" minOccurs="1" maxOccurs="1"/>

<xs:element name="result" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tRATType">

<xs:sequence>

<xs:element name="RAT" type="vaeinfo:contentType" minOccurs="1" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="vaeinfo:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tDeregistrationType">

<xs:sequence>

<xs:element name="v2x-ue-id" type="vaeinfo:contentType" minOccurs="0" maxOccurs="1"/>

<xs:element name="reception-uri" type="xs:anyURI" minOccurs="0" maxOccurs="1"/>

<xs:element name="v2x-service-id" type="xs:string" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="result" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tLocationTrackingType">

<xs:sequence>

<xs:element name="v2x-ue-id" type="vaeinfo:contentType" minOccurs="0" maxOccurs="1"/>

<xs:element name="geo-id" type="vaeinfo:contentType" minOccurs="0" maxOccurs="1"/>

<xs:element name="result" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="operation" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="anyExt" type="vaeinfo:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tMessageType">

<xs:sequence>

<xs:element name="v2x-ue-id" type="vaeinfo:contentType" minOccurs="0" maxOccurs="1"/>

<xs:element name="v2x-group-id" type="vaeinfo:contentType" minOccurs="0" maxOccurs="1"/>

<xs:element name="payload" type="xs:string" minOccurs="1" maxOccurs="unbounded"/>

<xs:element name="v2x-service-id" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="geo-id" type="vaeinfo:contentType" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="message-reception-ind" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="message-reception-uri" type="xs:anyURI" minOccurs="0" maxOccurs="1"/>

<xs:element name="result" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tServiceDiscoveryType">

<xs:sequence>

<xs:element name="v2x-ue-id" type="vaeinfo:contentType" minOccurs="0" maxOccurs="1"/>

<xs:element name="result" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="service-discovery-data" type="vaeinfo:tServiceDiscoveryDataType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tLocalServiceType">

<xs:sequence>

<xs:element name="v2x-ue-id" type="vaeinfo:contentType" minOccurs="0" maxOccurs="1"/>

<xs:element name="geo-id" type="vaeinfo:contentType" minOccurs="0" maxOccurs="1"/>

<xs:element name="result" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="local-service-info-content" type="vaeinfo:tLocalServiceInfoContentType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tLayer2GroupIDMappingType">

<xs:sequence>

<xs:element name="dynamic-group-info" type="vaeinfo:tDynamicGroupInfoType" minOccurs="1" maxOccurs="1"/>

<xs:element name="prose-layer2-group-id" type="vaeinfo:contentType" minOccurs="1" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tNetworkMonitoringSubscriptionType">

<xs:sequence>

<xs:element name="v2x-ue-id" type="vaeinfo:contentType" minOccurs="0" maxOccurs="1"/>

<xs:element name="subscription-events" type="vaeinfo:tSubscriptionEventType" minOccurs="0" maxOccurs="1"/>

<xs:element name="triggering-criteria" type="vaeinfo:tTriggeringCriteriaType"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tUSDAnnouncementType">

<xs:sequence>

<xs:element name="v2x-ue-id" type="vaeinfo:contentType" minOccurs="0" maxOccurs="1"/>

<xs:element name="v2x-usd-configuration-data" type="vaeinfo:tUSDType" minOccurs="1" maxOccurs="1"/>

<xs:element name="anyExt" type="vaeinfo:anyExtType" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tSetPC5ParametersInfoType">

<xs:sequence>

<xs:element name="v2x-ue-id" type="vaeinfo:contentType" minOccurs="0" maxOccurs="1"/>

<xs:element name="pc5-parameters-configuration-data" type="vaeinfo:tPC5ParametersConfigurationDataType" minOccurs="0" maxOccurs="1"/>

<xs:element name="result" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="anyExt" type="vaeinfo:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tIdListNotificationType">

<xs:sequence>

<xs:element name="dynamic-group-id" type="vaeinfo:contentType" minOccurs="1" maxOccurs="1"/>

<xs:element name="group-member-id" type="vaeinfo:tGroupMemberIdType" minOccurs="1" maxOccurs="unbounded"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tNetworkMonitoringInfoNotificationType">

<xs:sequence>

<xs:element name="v2x-ue-id" type="vaeinfo:contentType" minOccurs="0" maxOccurs="1"/>

<xs:element name="network-monitoring-info" type="vaeinfo:tNetworkMonitoringInfoType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tCommunicationStatusInfoType">

<xs:sequence>

<xs:element name="v2x-ue-id" type="vaeinfo:contentType" minOccurs="1" maxOccurs="1"/>

<xs:element name="v2v-communication-mode" type="xs:string" minOccurs="1" maxOccurs="1"/>

<xs:element name="v2x-service-id" type="xs:string" minOccurs="0"/>

<xs:element name="cell-info" type="xs:string" minOccurs="0"/>

<xs:element name="communication-link-status-info" type="xs:string" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tV2vCommunicationAssistanceInfoType">

<xs:sequence>

<xs:element name="v2x-ue-id" type="vaeinfo:contentType" minOccurs="1" maxOccurs="1"/>

<xs:element name="v2x-service-id" type="xs:string" minOccurs="0"/>

<xs:element name="v2v-communication-assistance" type="xs:string" minOccurs="1" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tDynamicGroupInfoUpdateType">

<xs:sequence>

<xs:element name="result" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="endpoint-info" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="dynamic-group-info-to-update" type="vaeinfo:tDynamicGroupInfoType" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tDynamicGroupInfoUpdateIndicationType">

<xs:sequence>

<xs:element name="dynamic-group-info" type="vaeinfo:tDynamicGroupInfoType" minOccurs="1" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tDynamicGroupInfoUpdateConsentType">

<xs:sequence>

<xs:element name="result" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="dynamic-group-info" type="vaeinfo:tDynamicGroupInfoType" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tSessionOrientedTerminationTriggerInfoType">

<xs:choice>

<xs:element name="session-id" type="xs:string" minOccurs="1" maxOccurs="1"/>

<xs:element name="result" type="xs:string" minOccurs="1" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:choice>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="contentType">

<xs:choice>

<xs:element name="vaeURI" type="xs:anyURI"/>

<xs:element name="vaeString" type="xs:string"/>

<xs:element name="vaeBoolean" type="xs:boolean"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:choice>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tServiceDiscoveryDataType">

<xs:sequence>

<xs:element name="v2x-service-map" type="vaeinfo:tServiceMapType" minOccurs="0" maxOccurs="unbounded"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tServiceMapType">

<xs:sequence>

<xs:element name="v2x-service-id" type="xs:string" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="v2x-as-address" type="vaeinfo:contentType" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tLocalServiceInfoContentType">

<xs:sequence>

<xs:element name="v2x-server-usd" type="vaeinfo:tUSDType" minOccurs="0" maxOccurs="1"/>

<xs:element name="v2x-as-address" type="vaeinfo:contentType" minOccurs="0" maxOccurs="1"/>

<xs:element name="v2x-usd" type="vaeinfo:tUSDType" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tUSDType">

<xs:sequence>

<xs:element name="TMGI" type="xs:hexBinary" minOccurs="1"/>

<xs:element name="mbms-service-areas" type="vaeinfo:tMbmsServiceAreasType" minOccurs="0"/>

<xs:element name="frequency" type="xs:unsignedLong" minOccurs="0"/>

<xs:element name="v2x-mbms-sdp" type="xs:string"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tMbmsServiceAreasType">

<xs:sequence>

<xs:element name="MbmsServiceAreaId" type="xs:hexBinary" minOccurs="1" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute/>

</xs:complexType>

<xs:complexType name="tDynamicGroupInfoType">

<xs:sequence>

<xs:element name="dynamic-group-id" type="vaeinfo:contentType" minOccurs="1" maxOccurs="1"/>

<xs:element name="group-leader-id" type="vaeinfo:contentType" minOccurs="1" maxOccurs="1"/>

<xs:element name="group-definition" type="xs:string" minOccurs="1" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tSubscriptionEventType">

<xs:sequence>

<xs:element name="Event" type="xs:string" minOccurs="0" maxOccurs="unbounded"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tTriggeringCriteriaType">

<xs:sequence>

<xs:element name="cell-change" type="vaeinfo:tCellChange" minOccurs="0"/>

<xs:element name="tracking-area-change" type="vaeinfo:tTrackingAreaChangeType" minOccurs="0"/>

<xs:element name="plmn-change" type="vaeinfo:tPlmnChangeType" minOccurs="0"/>

<xs:element name="mbms-sa-change" type="vaeinfo:tMbmsSaChangeType" minOccurs="0"/>

<xs:element name="mbsfn-area-change" type="vaeinfo:tMbsfnAreaChangeType" minOccurs="0"/>

<xs:element name="periodic-report" type="vaeinfo:tIntegerAttributeType" minOccurs="0"/>

<xs:element name="travelled-distance" type="vaeinfo:tIntegerAttributeType" minOccurs="0"/>

<xs:element name="vertical-application-event" type="vaeinfo:tVerticalAppEventType" minOccurs="0"/>

<xs:element name="geographical-area-change" type="vaeinfo:tGeographicalAreaChange"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tCellChange">

<xs:sequence>

<xs:element name="any-cell-change" type="vaeinfo:tEmptyTypeAttribute" minOccurs="0"/>

<xs:element name="enter-specific-cell" type="vaeinfo:tSpecificCellType" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="exit-specific-cell" type="vaeinfo:tSpecificCellType" minOccurs="0" maxOccurs="unbounded"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="vaeinfo:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tEmptyTypeAttribute">

<xs:complexContent>

<xs:extension base="vaeinfo:tEmptyType">

<xs:attribute name="trigger-id" type="xs:string" use="required"/>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="tSpecificCellType">

<xs:simpleContent>

<xs:extension base="vaeinfo: tNcgi">

<xs:attribute name="trigger-id" type="xs:string" use="required"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:complexType name="tTrackingAreaChangeType">

<xs:sequence>

<xs:element name="any-tracking-area-change" type="vaeinfo:tEmptyTypeAttribute" minOccurs="0"/>

<xs:element name="enter-specific-tracking-area" type="vaeinfo:tTrackingAreaIdentity" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="exit-specific-tracking-area" type="vaeinfo:tTrackingAreaIdentity" minOccurs="0" maxOccurs="unbounded"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="vaeinfo:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tTrackingAreaIdentityFormat">

<xs:restriction base="xs:string">

<xs:pattern value="\d{3}\d{3}[0-1]{16}"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tTrackingAreaIdentity">

<xs:simpleContent>

<xs:extension base="vaeinfo:tTrackingAreaIdentityFormat">

<xs:attribute name="trigger-id" type="xs:string" use="required"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:complexType name="tPlmnChangeType">

<xs:sequence>

<xs:element name="any-plmn-change" type="vaeinfo:tEmptyTypeAttribute" minOccurs="0"/>

<xs:element name="enter-specific-plmn" type="vaeinfo:tPlmnIdentity" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="exit-specific-plmn" type="vaeinfo:tPlmnIdentity" minOccurs="0" maxOccurs="unbounded"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="vaeinfo:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tPlmnIdentityFormat">

<xs:restriction base="xs:string">

<xs:pattern value="\d{3}\d{3}"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tPlmnIdentity">

<xs:simpleContent>

<xs:extension base="vaeinfo:tPlmnIdentityFormat">

<xs:attribute name="trigger-id" type="xs:string" use="required"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:complexType name="tMbmsSaChangeType">

<xs:sequence>

<xs:element name="any-mbms-sa-change" type="vaeinfo:tEmptyTypeAttribute" minOccurs="0"/>

<xs:element name="enter-specific-mbms-sa" type="vaeinfo:tMbmsSaIdentity" minOccurs="0"/>

<xs:element name="exit-specific-mbms-sa" type="vaeinfo:tMbmsSaIdentity" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="vaeinfo:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tMbmsSaIdentityFormat">

<xs:restriction base="xs:integer">

<xs:minInclusive value="0"/>

<xs:maxInclusive value="65535"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tMbmsSaIdentity">

<xs:simpleContent>

<xs:extension base="vaeinfo:tMbmsSaIdentityFormat">

<xs:attribute name="trigger-id" type="xs:string" use="required"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:complexType name="tMbsfnAreaChangeType">

<xs:sequence>

<xs:element name="any-mbsfn-area-change" type="vaeinfo:tMbsfnAreaIdentity" minOccurs="0"/>

<xs:element name="enter-specific-mbsfn-area" type="vaeinfo:tMbsfnAreaIdentity" minOccurs="0"/>

<xs:element name="exit-specific-mbsfn-area" type="vaeinfo:tMbsfnAreaIdentity" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="vaeinfo:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tMbsfnAreaIdentityFormat">

<xs:restriction base="xs:integer">

<xs:minInclusive value="0"/>

<xs:maxInclusive value="255"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tMbsfnAreaIdentity">

<xs:simpleContent>

<xs:extension base="vaeinfo:tMbsfnAreaIdentityFormat">

<xs:attribute name="trigger-id" type="xs:string" use="required"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:complexType name="tIntegerAttributeType">

<xs:simpleContent>

<xs:extension base="xs:integer">

<xs:attribute name="trigger-id" type="xs:string" use="required"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:complexType name="tVerticalAppEventType">

<xs:sequence>

<xs:element name="initial-log-on" type="vaeinfo:tEmptyTypeAttribute" minOccurs="0"/>

<xs:element name="location-configuration-received" type="vaeinfo:tEmptyTypeAttribute" minOccurs="0"/>

<xs:element name="any-other-event" type="vaeinfo:tEmptyTypeAttribute" minOccurs="0"/>

minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="vaeinfo:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tGeographicalAreaChange">

<xs:sequence>

<xs:element name="any-area-change" type="vaeinfo:tEmptyTypeAttribute" minOccurs="0"/>

<xs:element name="enter-specific-area" type="vaeinfo:tSpecificAreaType" minOccurs="0"/>

<xs:element name="exit-specific-area-type" type="vaeinfo:tSpecificAreaType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="vaeinfo:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tSpecificAreaType">

<xs:sequence>

<xs:element name="geographical-area" type="vaeinfo:tGeographicalAreaDef"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="vaeinfo:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:attribute name="trigger-id" type="xs:string" use="required"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tPC5ParametersConfigurationDataType">

<xs:sequence>

<xs:element name="expiration-time" type="xs:dateTime" minOccurs="1" maxOccurs="1"/>

<xs:element name="plmn-list" type="vaeinfo:tPlmnType" minOccurs="1" maxOccurs="1"/>

<xs:element name="authorized-when-not-served-by-E-UTRAN" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="radio-parameters" type="vaeinfo:tRadioParametersType" minOccurs="1" maxOccurs="1"/>

<xs:element name="v2x-service-ids-list" type="vaeinfo:tIDListType" minOccurs="1" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="vaeinfo:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tPlmnType">

<xs:sequence>

<xs:element name="plmn-id" type="vaeinfo:contentType" minOccurs="1" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="vaeinfo:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tRadioParametersType">

<xs:sequence>

<xs:element name="radio-parameters-content" type="xs:string" minOccurs="1" maxOccurs="unbounded"/>

<xs:element name="geographical-area" type="vaeinfo:tGeographicalAreaDef"/>

<xs:element name="anyExt" type="vaeinfo:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tIDListType">

<xs:sequence>

<xs:element name="v2x-service-id" type="vaeinfo:contentType" minOccurs="1" maxOccurs="unbounded"/>

<xs:element name="layer2-id" type="vaeinfo:contentType" minOccurs="1" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="vaeinfo:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tGeographicalAreaDef">

<xs:sequence>

<xs:element name="polygon-area" type="vaeinfo:tPolygonAreaType" minOccurs="0"/>

<xs:element name="ellipsoid-arc-area" type="vaeinfo:tEllipsoidArcType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="vaeinfo:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tPolygonAreaType">

<xs:sequence>

<xs:element name="corner" type="vaeinfo:tPointCoordinate" minOccurs="3" maxOccurs="15"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="vaeinfo:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tEllipsoidArcType">

<xs:sequence>

<xs:element name="center" type="vaeinfo:tPointCoordinate"/>

<xs:element name="radius" type="xs:nonNegativeInteger"/>

<xs:element name="offset-angle" type="xs:unsignedByte"/>

<xs:element name="included-angle" type="xs:unsignedByte"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="vaeinfo:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tPointCoordinate">

<xs:sequence>

<xs:element name="longitude" type="vaeinfo:tCoordinateType"/>

<xs:element name="latitude" type="vaeinfo:tCoordinateType"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="vaeinfo:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tGroupMemberIdType">

<xs:sequence>

<xs:element name="v2x-ue-id" type="vaeinfo:contentType" minOccurs="1" maxOccurs="1"/>

<xs:element name="group-scope" type="xs:string" minOccurs="1" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tNetworkMonitoringInfoType">

<xs:sequence>

<xs:element name="triggering-criteria" type="vaeinfo:tTriggeringCriteriaType" minOccurs="1" maxOccurs="1"/>

<xs:element name="uplink-quality-level" type="vaeinfo:tIntegerAttributeType" minOccurs="0"/>

<xs:element name="congestion-info" type="vaeinfo:tIntegerAttributeType" minOccurs="0"/>

<xs:element name="geographical-area" type="vaeinfo:tGeographicalAreaDef" minOccurs="0"/>

<xs:element name="time-validity" type="vaeinfo:tIntegerAttributeType" minOccurs="0"/>

<xs:element name="MBMS-level" type="vaeinfo:tMbmsLevelType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tMbmsLevelType">

<xs:sequence>

<xs:element name="MBMS-coverage-level" type="vaeinfo:tIntegerAttributeType" minOccurs="0"/>

<xs:element name="MBMS-bearer-level-event" type="xs:string" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="vaeinfo:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

</xs:schema>

* * * End of Change * * * *