3GPP TSG-CT WG1 Meeting #131-e	C1-214150

- Electronic meeting; 19th – 27th August 2021

Source:	Qualcomm Incorporated, Nokia, Nokia Shanghia Bell

Title:	Conclusion for Key issue 2

Spec:	3GPP TR 24.821

Agenda item:	17.2.4

Document for:	Agreement

1. Introduction

TR 24.821 contains three solutions for key issue #2. There is a need for the conclusion to key issue #2.

2. Discussion

In CT1#130-e. there was a discussion on the conclusion for key issue #2. The comments against adopting the conclusion in C1-213931, which was a combo of solutions 2,3 and 19, focused on the argument that there is no authority “legimitated” to provision the UE with the the global mapping between countries and PLMNs that are allowed to operate in the respective countries. Taking these comments into account, this contribution modifies the proposed way forward to limit the scope of the UE provisioning to the serving or the home PLMN, as follows:

-	HPLMN (e.g. HPLMN using MCC 9xx) may provision in USIM information about the countries in which the HPLMN/EHPLMN is allowed to operate.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 24.821 v1.2.0.

* * * First Change * * * *

## 8.x	Conclusion for key issue 2

The following is the list of the issues studied in this key issue and the respective conclusions for the normative phase:

a)	how the UE determines which PLMN(s) can be selected in determined country of its physical location (if it is determined that the UE is not in an international area); and

b)	what information could be available to the UE to help in selecting, in a given country, a PLMN whose CN is physically located in the same country as the UE.

-	pre-configured in the UE; and

-	provided by PLMN.

For a) and b), the following conclusion is adopted:

-	The UE is optionally provisioned by the HPLMN with information about one or more countries in which the HPLMN is allowed to operate over satellite access. This information is provisioned in USIM; and

-	The AMF of the serving PLMN may signal (e.g. in registration procedure) information about one or more countries in which the serving PLMN is allowed to operate over satellite access.

The UE uses this information to filter out the PLMNs that are not allowed to operate in the country of the UE location.

NOTE 1: 	How the information is encoded in USIM is up to CT6 to decide.

* * * No more changes * * * *