3GPP TSG-CT WG1 Meeting #131-e	C1-214210

- E-meeting, 19-27 August 2021

Source:	Huawei, HiSilicon

Title:	Client procedure of C2 communication mode selection by UAE Client procedure

Spec:	3GPP TS 24.257 V0.1.0

Agenda item:	17.2.21

Document for:	Agreement

1. Introduction

This p-CR defines the client procedure of C2 communication mode selection by UAE Client procedure for 3GPP TS 24.257.

2. Reason for Change

The specification needs to define the stage 3 details of the client procedure of C2 communication mode selection by UAE Client procedure defined in 3GPP TS 23.255 clause 7.4.2.3.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.257 v0.1.0.

* * * First Change * * * *

#### 6.4.1.2	C2 communication mode selection by UAE Client procedure

UAE Clients (UAV and UAV-C) select a primary and secondary C2 communication mode based on C2 communication mode configuration, then the UAE-C shall generate an HTTP POST request according to IETF RFC 7231 [x2]. In the HTTP POST request message, the UAE-C:

a)	shall set the Request-URI to the URI received in the UAE client UE configuration document via the SCM-S;

b)	shall include a Content-Type header field set to "application/vnd.3gpp.uae-info+xml";

c)	shall include an application/vnd.3gpp.uae-info+xml MIME body with a <C2-communication-mode-notification-info> element in the <UAE-info> root element which:

1)	shall include a <UAV-id> element set to the identifier of the UAV / UAV-C;

2)	shall include a <peer-UAV-id> element set to the identifier of the peer UAV / UAV-C;

3)	shall include a <selected-primary-C2-communication-mode> element indicating the selected primary C2 communication mode; and

4)	may include a <selected-secondary-C2-communication-mode> element indicating the selected secondary C2 communication mode; and

d)	shall send the HTTP POST request towards the UAE-S.

* * * End of Change * * * *