3GPP TSG-CT WG1 Meeting #131-e	C1-21xxxx

- E-meeting, 19-27 August 2021

Source:	Huawei, HiSilicon

Title:	Client procedure of UAE-layer assisted dynamic C2 mode switching procedure

Spec:	3GPP TS 24.257 V0.1.0

Agenda item:	17.2.21

Document for:	Agreement

1. Introduction

This p-CR defines the client procedure of UAE-layer assisted dynamic C2 mode switching procedure for 3GPP TS 24.257.

2. Reason for Change

The specification needs to define the stage 3 details of the client procedure of UAE-layer assisted dynamic C2 mode switching procedure defined in 3GPP TS 23.255 clause 7.4.2.4.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.257 v0.1.0.

* * * First Change * * * *

#### 6.4.1.3	UAE-layer assisted dynamic C2 mode switching procedure

Upon detecting a condition for switching C2 communication mode based on local conditions (e.g. using the C2 communication mode switching policy) or based on a command from the UAS application specific server, the UAE-C shall generate an HTTP POST request according to IETF RFC 7231 [x2]. In the HTTP POST request message, the UAE-C:

a)	shall set the Request-URI to the URI corresponding to the identity of the UAE-S;

b)	shall include a Content-Type header field set to "application/vnd.3gpp.uae-info+xml";

c)	shall include an application/vnd.3gpp.uae-info+xml MIME body with a <C2-related-trigger-event-report> element in the <UAE-info> root element which shall include:

1)	a <UAE-client-id> element set to the identifier of the UAE client which indicates the QoS downgrade; and

2)	an <application-QoS-related-event> element including the expected or actual application QoS/QoE parameters which were changed (i.e. latency, throughput, reliability, jitter); and

d)	shall send the HTTP POST request towards the UAE-S.

Editor’s note: The C2 operation mode switching performed procedure is FFS.

* * * End of Change * * * *