3GPP TSG-CT WG1 Meeting #131-e	C1-214213

- E-meeting, 19-27 August 2021

Source:	Huawei, HiSilicon

Title:	Server procedure of UAE-layer assisted dynamic C2 mode switching procedure

Spec:	3GPP TS 24.257 V0.1.0

Agenda item:	17.2.21

Document for:	Agreement

1. Introduction

This p-CR defines the server procedure of UAE-layer assisted dynamic C2 mode switching procedure for 3GPP TS 24.257.

2. Reason for Change

The specification needs to define the stage 3 details of the server procedure of UAE-layer assisted dynamic C2 mode switching procedure defined in 3GPP TS 23.255 clause 7.4.2.4.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.257 v0.1.0.

* * * First Change * * * *

6.4.2.3	UAE-layer assisted dynamic C2 mode switching

Upon receiving an HTTP POST request containing:

a)	a Content-Type header field set to "application/vnd.3gpp.uae-info+xml"; and

b)	an application/vnd.3gpp.uae-info+xml MIME body with a <C2-related-trigger-event-report> element,

the UAE-S:

a)	shall obtain a location report for the UAE-C by the SLM-S;

b)	shall determine the switching of the C2 mode from direct to network assisted or vice versa or to USS/UTM navigated;

c)	if the switching of the C2 mode is from direct to network assisted or vice versa, may send a C2 mode switching confirmation request to the UAS application specific server;

d)	if the switching of the C2 mode is from direct to USS/UTM navigated, shall send a C2 mode switching confirmation request to the UAS application specific server; and

e)	the UAE-S shall generate an HTTP 200 (OK) response according to IETF RFC 7231 [x2]. In the HTTP 200 (OK) response message, the UAE-S:

NOTE: If the UAE-S has sent a C2 mode switching confirmation request to the UAS application specific server, the UAE-S shall wait and receive from the UAS application specific server a C2 mode switching confirmation response and then generate the HTTP 200 (OK) response message.

1)	shall include a Request-URI set to the URI corresponding to the identity of the UAE-C;

2)	shall include a Content-Type header field set to "application/vnd.3gpp.uae-info+xml";

3)	shall include an application/vnd.3gpp.uae-info+xml MIME body with a <C2-operation-mode-switching> element in the <UAE-info> root element which:

i)	shall include a <UAE-server-id > elementset to the identifier of the UAE server which instructs the UAS to apply the C2 mode switching;

ii)	shall include a <C2-operation-mode-switching-requirement> element indicating the type of the C2 mode switching to be applied;

iii)	may include a <time-validity> element set to the time validity for the C2 switching requirement; and

iv)	may include a <geographical-area> element indicating the area for which the C2 switching applies; and

4)	shall send the HTTP 200 (OK) message towards the UAE-C.

* * * End of Change * * * *