3GPP TSG-CT WG1 Meeting #131-e	C1-214256

- E-meeting, 19-27 August 2021

Source:	InterDigital

Title:	IP address allocation for L3 UE-to-Network Relay

Spec:	3GPP TS 24.554 v 0.3.0

Agenda item:	17.2.18

Document for:	Agreement

1. Introduction

There are the configuration parameters for direct discovery.

2. Reason for Change

In the TS 23.304 v 1.0.0, mechanism for IP address/prefix allocation for communication with a 5G ProSe Layer-3 ProSe UE-to-Network Relay is provided for which TS 24.554 should be updated.

3. Conclusions

The mechanism for IP address/prefix allocation in TS 24.554 is provided for L3 U2N relay.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.554 v 0.3.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS); Stage 2".

[3]	IETF RFC 7230: "Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing".

[4]	IETF RFC 7231: "Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content".

[5]	3GPP TS 24.526: "UE policies for 5G System (5GS); Stage 3".

[6]	OMA-WAP-TS-PushOTA-V2_1-20110405-A: "Push Over the Air".

[7]	OMA-AD-Push-V2_2-20110809-A: "Push Architecture".

[8]	WAP-168-ServiceLoad-20010731-a: "Service Loading".

[9]	3GPP TS 29.555: "Inter-5G Direct Discovery Name Management Function (DDNMF) signalling aspects; Stage 3".

[10]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[11]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[12]	3GPP TS 23.003: "Numbering, addressing and identification".

[13]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[14]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[15]	3GPP TS 38.304: "User Equipment (UE) procedures in Idle mode and RRC Inactive state".

[16]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[17]	3GPP TS 24.555: "Proximity-services (ProSe) in 5G System (5GS); User Equipment (UE) policies; Stage 3".

[18]	3GPP TS 24.587: "Vehicle-to-Everything (V2X) services in 5G System (5GS); Protocol aspects; Stage 3".

[19]	3GPP TS 29.343: "Proximity-services (ProSe) function to ProSe application server aspects (PC2); Stage 3".

[20]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[21]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[ts23501]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[rfc2131]	IETF RFC 2131: "Dynamic Host Configuration Protocol".

[rfc4039]	IETF RFC 4039: "Rapid Commit Option for the Dynamic Host Configuration Protocol version 4 (DHCPv4)".

[rfc4862]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

* * * Next Change * * * *

#### 8.2.x.y	IP address allocation for remote UE in 5G ProSe layer-3 UE-to-network relay procedure

When one of the two UEs on the direct link acts as a 5G ProSe Layer-3 UE-to-network relay, the PDU session type is IPv4, IPv6 or IPv4v6 and is used for relaying IP traffic over PC5 reference point, the two UEs shall select the IP version (IPv4 or IPv6) to be used based on the following rules:

-	if the 5G ProSe Layer-3 UE-to-network relay UE has indicated "DHCPv4 Server" in the IP address config IE, the remote UE shall initiate the IPv4 address configuration with DHCPv4 procedure acting as a DHCP client, according to IETF RFC 2131 [rfc2131] and IETF RFC 4039 [rfc4039];

-	if the 5G ProSe Layer-3 UE-to-network relay UE has indicated "IPv6 Router" in the IP address config IE, the remote UE shall initiate the IPv6 address configuration with IPv6 stateless address auto-configuration acting as an IPv6 host based on IETF RFC 4862 [rfc4862];

-	if the 5G ProSe Layer-3 UE-to-network relay UE has indicated "IPv6 Router" in the IP address config IE, the remote UE may use stateless DHCPv6 for additional parameter configuration, as defined in TS 23.501 [ts23501]; and

-	if the 5G ProSe Layer-3 UE-to-network relay UE has indicated "DHCPv4 Server & IPv6 Router" in the IP address config IE, the remote UE shall choose the IP version and initiate the corresponding IP address configuration procedure as a client or host.

* * * End of Change * * * *