3GPP TSG-CT WG1 Meeting #131-e	C1-21xxxx

- E-meeting, 19-27 August 2021

Source:	Qualcomm Incorporated

Title:	5G ProSe: Correction on precedence of configuration

Spec:	3GPP TS 24.554

Agenda item:	17.2.18

Document for:	Agreement

1. Introduction

The principle of the precedence of the ProSe configuration has been changed.

2. Reason for Change

In the last SA2 meeting (SA2#145e), on the configuration parameter precendence, it was agreed that the general sequence of PCF > App Server via PC1 > UICC > ME will be always used (S2-2103927). If the App server wants to take precedence for configuration of ProSe group member discovery and ProSe groupcast mode direct communication, it needs to access to PCF(via NEF) for configuration parameter provisioning.

3. Proposal

It is proposed to agree the following changes to 3GPP TS <TS 24.554>.

* * * First Change * * * *

### 5.2.2	Precedence of 5G ProSe configuration information

The 5G ProSe configuration information for ProSe direct discovery, ProSe direct communication, ProSe UE-to-network relay and ProSe usage reporting configuration and rules can be:

a)	pre-configured in the ME;

b)	configured in the UICC;

c)	provided as a ProSeP by PCF;

d)	provided by a ProSe Application Server via PC1 reference point except:

1)	authorization policy for 5G ProSe direct discovery and restricted ProSe discovery UE ID for restricted direct discovery in parameters for 5G ProSe direct discovery;

2)	authorization policy in parameters for 5G ProSe direct communication;

3)	the following parameters for the role of a 5G ProSe UE-to-Network Relay:

i)	authorization policy for acting as a 5G ProSe Layer 3 and/or Layer 2 UE-to-Network Relay when "served by NG-RAN";

ii)	QoS mapping rules for 5G ProSe Layer 3 UE-to-Network Relay; and

iii)	 a mapping of ProSe Service(s) to ProSe Application Server address information for 5G ProSe Layer 3 UE-to-Network Relay to relay Ethernet or Unstructured traffic from Remote UE by using IP type PDU session; or

4)	the following parameters for the role of a Remote UE:

i)	authorization policy for using a 5G ProSe Layer 3 and/or Layer 2 UE-to-network Relay;

5)	the following parameters for the role of a 5G ProSe UE-to-Network Relay as well as for the role of a 5G ProSe Remote UE:

i)	radio parameters for 5G ProSe relay discovery when the UE is not "served by NG-RAN "; and

ii)	radio parameters for 5G ProSe relay communication when the UE is not "served by NG-RAN"; or

e)	a combination of case a), b), c) or d) above.

The UE should not request or accept any 5G ProSe configuration information from PCF when the UE is working as a Remote UE using a 5G ProSe Layer 3 UE-to-Network Relay without involving N3IWF.

The UE shall use the 5G ProSe configuration information in the following order of decreasing precedence:

a)	the 5G ProSe configuration information provided as a ProSeP by PCF;

b)	the 5G ProSe configuration information provided by a ProSe Application Server via PC1 reference point;

c)	the 5G ProSe configuration information configured in the UICC; and

d)	the 5G ProSe configuration information pre-configured in the ME.

* * * End of Change * * * *